| ANTIFOULING CODE |  |
| --- | --- |
| GR-PM-AFS | ANTIFOULING SYSTEMS |

# SECTION 1

## Background

  - Anti-fouling paints are used to coat the bottoms of ships to prevent sea life such as algae and mollusks attaching themselves to the hull – thereby slowing down the ship and increasing fuel consumption.

  - The International Convention on the control of harmful anti-fouling systems on ships defines “anti- fouling systems” as “a coating, paint, surface treatment, surface or device that is used on a ship to control or prevent attachment of unwanted organisms”.

  - In the early days of sailing ships, lime and later arsenic were used to coat ships' hulls, until the modern chemicals industry developed effective anti-fouling paints using metallic compounds.

  - These compounds slowly "leach" into the sea water, killing barnacles and other marine life that have attached to the ship. But the studies have shown that these compounds persist in the water, killing sea life, harming the environment and possibly entering the food chain. One of the most effective anti- fouling paints, developed in the 1960s, contains the organotin  tributylin (TBT), which has been proven to cause deformations in oysters and sex changes in whelks.

# SECTION 2

## Application

  - The International Convention on the control of harmful anti-fouling systems on ships will prohibit the use of harmful organotin in anti-fouling paints used on ships and will establish a mechanism to prevent the potential future use of other harmful substances in anti-fouling systems.

  - Ships of above 400 gross tonnage and above engaged in international voyages (excluding fixed or floating platforms, FSUs and FPSOs) will be required to undergo an initial survey before the ship is put into service or before the International Anti-fouling System E-certificate is issued for the first time; and a survey when the anti-fouling systems are changed or replaced

  - Ships of 24 meters or more in length but less than 400 gross tonnages engaged in international voyages(excluding fixed or floating platforms, FSUs and FPSOs) will have to carry a Declaration on Anti-fouling Systems signed by the owner or authorized agent. The Declaration will have to be accompanied by appropriate documentation such as a paint receipt or contractor invoice.

  - Anti-fouling systems to be prohibited or controlled will be listed in an annex (Annex 1) to the Convention, which will be updated as and when necessary.

  - Annex I attached to the Convention and adopted by the Conference states that by an effective date of 1January 2003, all ships shall not apply or re-apply organotin compounds which act as biocides in anti- fouling systems.

By 1 January 2008 (effective date), ships either:

    - shall not bear such compounds on their hulls or external parts or surfaces; or

    - shall bear a coating that forms a barrier to such compounds leaching from the underlying on-compliant anti-fouling systems.

This applies to all ships (including fixed and floating platforms, floating storage units (FSUs), and Floating Production Storage and Offtake units (FPSOs).

# SECTION 3

## Surveys

  - Article 10 of the International Convention on the Control of Harmful Anti-fouling Systems on Ships, 2001, hereinafter referred to as. the Convention, prescribes that ships shall be surveyed and certified in accordance with the regulations of Annex 4 of the Convention.

  - The sole purpose of the survey activities described in these Guidelines is to verify compliance with the provisions of the Convention. Consequently, such surveys do not relate to any aspect not regulated by the Convention even if such aspects relate to the performance of an anti-fouling system on the hull of a ship, including the quality of workmanship during the application process.

  - An initial survey should be carried out:

- for a newbuilding; or

- for an existing ship, before the International Anti-fouling System E-certificate required under Regulations 2 or 3 of Annex 4 to the Convention is issued for the first time.

  - In order to ease the burden on the companies and other parties, the initial survey of the anti- fouling system on existing ships may best be carried out in connection with a drydock survey.

  - A survey should be carried out whenever an anti-fouling system is changed or replaced. Such surveys should cover the same scope as in Section 5.2.

  - A major conversion affecting the anti-fouling system of a ship may be considered as a newbuilding as determined by the Administration.

  - Repairs generally do not require a survey. However, repairs affecting approximately twenty- five

(25) percent or more of the anti-fouling system, should be considered as a change or replacement of the anti-fouling system.

  - A non-compliant anti-fouling system controlled under Annex I of the Convention, that under goes repair must be repaired, or replaced with a compliant anti-fouling system.

# SECTION 4

## Request for survey

  - Prior to any survey, a job request and the request for service forms should be submitted by the Company to GR Class-Classified For Standard (GR CLASS).

  - A request for survey will be approved (or not) according to GR Class according to internal procedures.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa. In case of Barbados, to be found in https://barbadosmaritime.org/documents/bulletins/. In case of conflict, the surveyor should follow the instruction issued by the flag statethat GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

# SECTION 5

## Conduct of surveys

  - Surveys for new buildings

- As part of the survey, it should be verified that the anti-fouling system specified by the documentation submitted with the request for survey complies with the Convention. The survey should include verification that the anti-fouling system applied is identical to the system specified in the request for survey.

.1.2. A declaration and supporting information from the anti-fouling system manufacturer, confirming that the anti-fouling system applied, or intended to be applied to the ship is in compliance with the requirements of the Convention (with an identification of the version of the Convention referred to). Such declaration should provide the following information contained in the Record of Anti-Fouling System, as can be found in Appendix 1 to Annex 4 of the Convention:

  - Type of anti-fouling system (Examples of suitable wording could be: Organotin-free self polishing type, Organotin-free ablative type, Organotin-free conventional, Biocide-free silicon type paint, others. In the case of an anti-fouling system containing no active ingredients, the words .biocide-free should be used.)

  - Name of anti-fouling system manufacturer

  - Name and colour of anti-fouling system

  - Active ingredient(s) and their Chemical Abstract Service Registry Number (CAS number(s))

  - Information required by the surveyor regarding compliance of product with the Convention should be found in a declaration from the anti-fouling system manufacturer which may be provided on the anti-fouling system container and/or on supportive documentation (such as Material Safety Data Sheets (MSDSs), or similar). A link between the supportive documentation and the relevant container should exist.

- Taking into account experience gained and the prevailing circumstances, the verification required by paragraph 5.1.1 should include one or more of the following tasks, as deemed necessary to verify compliance:

- Checking that the product identification on anti-fouling system containers used during the application process is identical to the system specified in the request for survey.

- Sampling of the anti-fouling system.

- Testing of the anti-fouling system.

- Other checks conducted on site.

- The verification tasks set out in paragraph 5.1.2 should be conducted at any time, either before, during, or after the anti-fouling system has been applied to the ship, as deemed necessary to verify compliance. No checks or tests must affect the integrity, structure or operation of the anti-fouling system.

  - Surveys of existing ships intending to apply a new anti-fouling system

- If the existing anti-fouling system is confirmed by an International Anti-fouling System E-certificate not to be controlled under Annex 1 of the Convention, the provisions described in paragraph 5.1 apply.

- If the existing anti-fouling system is declared not to be controlled under Annex 1 of the Convention, without being documented by an International Anti-Fouling System E-certificate, a verification should be carried out to confirm that the anti-fouling system complies with the requirements of the Convention. This verification may be based on sampling and/or testing and/or reliable documentation, as deemed necessary based on experience gained and the existing circumstances.

Documentation for verification could e.g. be MSDSs, or similar, a declaration of compliance from the anti- fouling system manufacturer, invoices from the shipyard and/or the anti-fouling system manufacturer.

To verify the new anti-fouling system, the provisions described in paragraph 5.1 apply.

- If the existing anti-fouling system has been removed, the removal should be verified in addition to the provisions described in paragraph 5.1.

- If a sealer coat has been applied, a verification should be carried out to confirm that the name, type and colour of the sealer coat applied to the ship match those specified in the request for survey, and that the existing anti-fouling system has been covered with that sealer coat. Additionally, the provisions described in paragraph 5.1 apply.

- If the existing anti-fouling system is controlled under Annex 1 of the Convention, it should be removed according to sub-paragraph 5.2.3 or covered by a sealer coat according to sub-paragraph 5.2.4 not later than 1 January 2008. Prior to this date, the existing anti-fouling system may be over-coated with an anti- fouling system not controlled under Annex 1 without removing or sealing the existing anti-fouling system. This option should be stated on the International Anti-Fouling System E-certificate by ticking off the appropriate box. To verify the new anti-fouling system, the provisions described in paragraph 5.1 apply.

  - Surveys of existing ships requesting only an International Anti-fouling System E-certificate

- If the existing anti-fouling system is declared to be controlled under Annex 1 of the Convention (i.e. containing prohibited/regulated compounds), an International Anti-fouling System E-certificate may be issued on request stating that the anti-fouling system will be removed, or covered with a sealer coat when directed by the Convention.

- If the existing anti-fouling system is declared not to be controlled under Annex 1 of the Convention, a verification should be carried out to confirm that the anti-fouling system complies with the requirements of the Convention. This verification may be based on sampling and/or testing and/or reliable

documentation, as deemed necessary based on experience gained and the existing circumstances. Such documentation could be MSDSs or similar, a declaration of compliance from the anti-fouling system manufacturer, invoices from the shipyard and/or the anti-fouling system manufacturer. If this information raises no reasonable doubt that the system applied is compliant with Annex 1 of the Convention, the International Anti-fouling System E-certificate may be issued on this basis.

  - Surveys of ships prior to entry into force of the Convention

- Prior to the Convention having entered into force, an Administration/Recognized Organization may conduct surveys of ships in accordance with these Guidelines, and may then issue a Statement of Compliance to this effect.

- Ships capable of documenting full compliance with the Convention through such a statement of compliance may be issued, upon entry into force of the Convention, an International Anti- fouling System E-certificate on that basis, subject to any additional requirements from the Administration.

## New AFS regulations as of 1 January 2023.

    - IMO adopted amendments to the AFS Convention requiring ships bearing an anti-fouling system that contains Cyberdyne in the external coating layer of their hull or external parts or surfaces shall either (1) remove the antifouling system; or (2) apply a coating that forms a barrier to this substance leaching from the underlying non-compliance anti- fouling system. This is to be done at the next scheduled renewal of the anti-fouling system after 1 January 2023, but no later than 60 months following the last application to the ship of an anti-fouling system containing cybutryne.

# SECTION 6

## Issuing or endorsing the International Anti-fouling System E-certificate

  - The International Anti-fouling System E-certificate along with the Record (Form No. AFS-02) of Anti-fouling Systems should be:

- issued upon satisfactory completion of the initial survey;

- endorsed upon satisfactory completion of a survey for change or replacement of an anti-fouling system.

# SECTION 7

## Guidance for compliant anti-fouling systems

  - For the purpose of compliance with Annex I of the Convention, small quantities of organotin compounds acting as a chemical catalyst (such as mono- and di- substituted organotin compounds) are allowed, provided that they are present at a level which does not provide a biocidal effect to the coating. On

a practical level, when used as a catalyst, an organotin compound should not be present above 2,500 mg total tin per kg of dry paint.

# SECTION 8

## Reporting

  - The relevant survey checklist Form No. AFS-01 should be used when carrying out surveys and a copy submitted to GR Head Office. The Form No. AFS-02 is to be filled up during the survey and a copy

submitted to GR Head Office. The Form SURVEY STATEMENT (previous P24) should be filled and uploaded to the data base.

  - At first surveys by, copies of the previous recognized organization e-certificates and record of Anti- Fouling Systems should also be forwarded.

  - On completion of the First Survey by GR Class-Classified For Standard (GR CLASS) , an INTERIM e- certificate should be issued, valid for five (5) months along with record of Anti-Fouling Systems Form No. AFS-02.

  - On completion of renewal surveys by GR Class-Classified For Standard (GR CLASS) , an endorsement of the Record of Anti-fouling Systems should be issued (Form No. AFS-03) and a copy submitted to GR Head Office.

  - All necessary supporting documentation should be submitted to GR Head Office along with survey reports such as copy of supporting information from the anti-fouling system manufacturer, confirming that the anti-fouling system applied, or intended to be applied to the ship is in compliance with the requirements of the Convention (with an identification of the version of the Convention referred to)

# PROCEDURAL REFERENCES:

  - International Convention on the Control of Harmful Anti-fouling Systems on Ships, 2001 (AFS 2001)

  - Guidelines for survey and certification of anti-fouling system on ships (Resolution MEPC. 102 (48))

  - Guidelines for brief sampling on anti-fouling systems on ships (Resolution MEPC. 104 (49))

  - Guidelines for inspection of anti-fouling systems on ships (Resolution MEPC. 105 (49))

  - Amendments to the International Convention on the Control of Harmful Anti-fouling Systems on Ships, 2001 - RESOLUTION MEPC.331(76)

| ANTIFOULING CODE |  |
| --- | --- |
| GR-PM-AFS | ANTIFOULING SYSTEMS |

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 157: Control of Anti- Fouling System on Ships. | 03/2008 | https://panamashipregistry.com/circular- category/circulares/ |
| Barbados | NA | NA | https://barbadosmaritime.org/bulletins/ |
| Belize | MSN-0034: Entry into force of the Afs-Convention | 05/06/2008 | https://www.immarbe.com/circularslist/ |
| Sierra Leone | NA | NA | https://slmarad.com/maritime-circulars |
| Palau | MN-135.1: Control of harmful Anti-Fouling systems on ships | 10/2018 | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentation/instru ments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disposiciones |
| Nigeria | NA | NA | https://nimasa.gov.ng/press-center/marine-notice- adverts/ |
| Togo | Circular No. 0015C/TG/02/17: AFS Convention | 02/2017 | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |