GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

REPORT OF BALLAST WATER MANAGEMENT SURVEYS

| SHIP NAME | IMO No. | CALL SIGN | FLAG | CLASS No. |
| --- | --- | --- | --- | --- |

For Annual, Intermediate or renewal survey mark “X” to indicate “Yes” and “N” to indicate an outstanding action in appropriate block for each item. If an item is not

applicable mark “NA” in appropriate block.

Ships of 400 gross tonnage and above to which this Convention applies, excluding floating platforms, FSUs and FPSOs, shall be subject to surveys specified below:

- An initial survey before the ship is put in service or before the Certificate required under regulation E-2 or E-3 is issued for the first time. This survey shall verify that the Ballast Water Management plan required by regulation B-1 and any associated structure, equipment, systems, fitting, arrangements and material or processes comply fully with the requirements of this Convention. This survey shall confirm that a commissioning test has been conducted to validate the installation of any ballast water management system by demonstrating that its mechanical, physical, chemical and biological processes are working properly, taking into account the guidelines developed by the Organization.

- A renewal survey at intervals specified by the Administration, but not exceeding five years, except where regulation E-5.2, E-5.5, E-5.6, or E-5.7 is applicable. This survey shall verify that the Ballast Water Management plan required by regulation B-1 and any associated structure, equipment, systems, fitting, arrangements and material or processes comply fully with the applicable requirements of this Convention.

- An intermediate survey within three months before or after the second Anniversary date or within three months before or after the third Anniversary date of the Certificate, which shall take the place of one of the annual surveys specified in paragraph 1.4. The intermediate surveys shall ensure that the equipment, associated systems and processes for Ballast Water Management fully comply with the applicable requirements of this Annex and are in good working order. Such intermediate surveys shall be endorsed on the Certificate issued under regulation E-2 or E-3.

- An annual survey within three months before or after each Anniversary date, including a general inspection of the structure, any equipment, systems, fittings, arrangements and material or processes associated with the Ballast Water Management plan required by regulation B-1 to ensure that they have been maintained in accordance with paragraph 9 and remain satisfactory for the service for which the ship is intended. Such annual surveys shall be endorsed on the Certificate issued under regulation E-2 or E-3.

- An additional survey either general or partial, according to the circumstances, shall be made after a change, replacement, or significant repair of the structure, equipment, systems, fittings, arrangements, and material necessary to achieve full compliance with this Convention. The survey shall be such as to ensure that any such change, replacement, or significant repair has been effectively made, so that the ship complies with the requirements of this Convention. This survey shall confirm that a commissioning test has been conducted to validate the installation of any ballast water management system by demonstrating that its mechanical, physical, chemical and biological processes are working properly, taking into account the guidelines developed by the Organization.

ALL CARGO SHIPS

1. INITIAL SURVEY

A. GENERAL

- On the navigation bridge the design and construction was examined on the plans	☐

- confirming that, if applicable, dosage instruction for active substances or preparations are available on board	☐

- confirming that the Ballast Water Management Plan has been provided (BWM 2004 Convention / ANNEX / Regulation B1)	☐

- confirming that the Ballast Water Record Book has been provided (BWM 2004 Convention / ANNEX / Regulation B2)	☐

- verifying that the BWMS is in conformity with the Type Approval Certificate of BWMS issued by the Administration or its representative (BWM	☐

2004 Convention / ANNEX / Regulation D2)

- confirming that certificate(s) for type approval of ballast water management system(s) are available (BWM 2004 Convention / ANNEX /	☐

Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that a statement has been provided by the Administration, or from a laboratory authorized by the Administration, confirming that	☐ the  electrical  and  electronic  components  of  the  ballast  water  management system(s)  have  been  type  tested  in  accordance  with  the specifications for environmental testing contained in Part 3 of the Annex of the Guidelines for Approval of Ballast Water Management Systems

(G8) (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that equipment manuals for major components of the ballast water management system(s) have been provided (BWM 2004	☐

Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that an operations and technical manual for the ballast water management system(s) specific to the ship and approved by the	☐ Administration, containing a technical description of the ballast water management system(s), operational and maintenance procedures, and  backup procedures in case of equipment malfunction has been provided (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that installation specifications for the ballast water management system(s) have been provided	☐

- (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- examining the ballast water management plan (BWM 2004 Convention / ANNEX / Regulation B1)	☐

- examination of plans for the installation of ballast water management systems (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004	☐

Convention / ANNEX / Regulation D2)

- if applicable, examination of plans for the installation of prototype ballast water treatment technologies (BWM 2004 Convention / ANNEX /	☐

Regulation D4)

- confirming that, if applicable, sufficient active substances are provided on board (BWM 2004 Convention / ANNEX / Regulation D3)	☐

- (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that if applicable the ballast water management recording device(s) are operable and that there is a sufficient supply of consumables	☐

for the recording device(s) on board (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- verifying that the BWMS installation has been carried out in accordance with the technical installation specification (BWM 2004 Convention /	☐

ANNEX / Regulation D2)

- verifying that the installation of the complete BWMS has been carried out in accordance with the manufacturer’s equipment specification	☐

(BWM 2004 Convention / ANNEX / Regulation D2)

- verifying that any operational inlets and outlets are located in the positions indicated on the drawings of the pumping and piping arrangements	☐

(BWM 2004 Convention / ANNEX / Regulation D2)

- confirming the satisfactory installation and operation of the ballast water management system, including any audible or visual alarms (BWM	☐

2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- verifying that the Control and Monitoring Equipment operates correctly; (BWM 2004 Convention / ANNEX / Regulation D2)	☐

- confirming  that  installation  commissioning  procedures  for  the  ballast  water  management  system(s)  have  been  provided  (BWM  2004	☐

Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that initial calibration procedures of the ballast water management system(s) have been provided (BWM 2004 Convention / ANNEX	☐

/ Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that, if applicable, a Statement of Compliance for a Prototype Ballast Water Treatment Technology has been provided (BWM 2004	☐

Convention / ANNEX / Regulation D4)

- Confirming that BWMS is in good working condition in accordance with Reg. D2	☐

- Confirming that a commissioning test has been conducted to validate the installation of any ballast water management system by demonstrating that its mechanical, physical, chemical and biological processes are working properly, taking into account the guidelines developed by the Organization.

- Confirming that Ballast piping system and pumping system are in good condition in accordance with Reg. D1	☐

- verifying that any operational inlets and outlets are located in the positions indicated on the drawings of the pumping and piping arrangements	☐

(BWM 2004 Convention / ANNEX / Regulation D2)

- verifying that, if applicable, the prototype ballast water treatment technology installation has been carried out in accordance with the approved	☐

Program and that the workmanship of the installation is satisfactory (BWM 2004 Convention / ANNEX / Regulation D4)

- Confirming that Prototype Ballast Water Treatment technologies and its system is in a good working condition in accordance with Reg. D4	☐

- confirming that sampling facilities are provided and so arranged in order to collect representative samples of the ships ballast water from the	☐ ballast water management system(s) intake(s) before the ballast discharge points and any other points necessary for sampling (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- after satisfactory survey, the International Ballast Water Management Certificate should be issued.	☐

2. ANNUAL SURVEY

- sighting the records of the recording device, if fitted (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX /   ☐

Regulation D2)

- confirming that the Ballast Water Management Plan is on board (BWM 2004 Convention / ANNEX / Regulation B1)	☐

- checking whether the appropriate entries have been made in the Ballast Record Book (BWM 2004 Convention / ANNEX / Regulation B2)	☐

- sighting the type test certificate(s) for the ballast water management system(s) (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004   ☐

Convention / ANNEX / Regulation D2)

- sighting, if applicable, the Statement of Compliance for a Prototype Ballast Water Treatment Technology (BWM 2004 Convention / ANNEX /   ☐

Regulation D4)

- examining externally the ballast water treatment system and confirming, as far as practicable its satisfactory operation (BWM 2004 Convention   ☐

/ ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that, if applicable, active substances in accordance with the manufacturers recommendations are provided on board (BWM 2004   ☐

Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that, if applicable, dosage instruction for active substance or preparations are available on board (BWM 2004 Convention / ANNEX /   ☐

Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- verifying, if applicable, that there is evidence that the Prototype Ballast Water Treatment Technology is continuing to be operated in accordance   ☐

with the approved program (BWM 2004 Convention / ANNEX / Regulation D4)

- Confirming that Ballast piping system and pumping system are in good condition in accordance with Reg. D1	☐

- Confirming that BWMS is in good working condition in accordance with Reg. D2	☐

- Confirming that Prototype Ballast Water Treatment technologies and its system is in a good working condition in accordance with Reg. D4	☐

- checking whether any new equipment has been fitted and, if so, confirm that it has been approved before installation and that any	☐

changes are reflected in the certificate

- where applicable examining externally the prototype ballast water treatment technology and confirming, as far as practicable its	☐

satisfactory operation (BWM 2004 Convention / ANNEX / Regulation D4)

- if a survey shows that the condition of a ship or its equipment is unsatisfactory; see part General section 4.8*

- *4.8 Completion of surveys

- 4.8.1 If a survey shows that the condition of the ship or its equipment is unsatisfactory, the officer of the Administration, nominated surveyor or   ☐

recognized organization should be guided by the requirements of SOLAS 74/88 regulation I/6(c), MARPOL 73/78/90 Annex I regulation 4(3)(d),

MARPOL 73/78/90 Annex II regulation 10(2)(c), the IBC Code 83/90 regulation 1.5.1.3, the IGC Code 83/90 regulation 1.5.1.3 and the BCH Code 85/90 regulation 1.6.1.3. These instruments require that corrective action be taken immediately and the Administration notified in due course. In cases where the corrective action has not been undertaken the relevant certificate should be withdrawn and the Administration notified immediately. If the ship is in the port of another Party, the appropriate authorities of the port State also notified immediately.

4.8.2 Although LLC 66/88 does not contain specific requirements, if a load line survey shows the condition of the ship or its equipment is unsatisfactory, the officer of the Administration, nominated surveyor or recognized organization should, nevertheless, be guided by 4.8.1.

- after satisfactory survey, the International Ballast Water Management Certificate should be endorsed	☐

3. INTERMEDIATE SURVEY

- sighting the records of the recording device, if fitted (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX /   ☐

Regulation D2)

- confirming that the Ballast Water Management Plan is on board (BWM 2004 Convention / ANNEX / Regulation B1)	☐

- checking whether the appropriate entries have been made in the Ballast Record Book (BWM 2004 Convention / ANNEX / Regulation B2)	☐

- sighting the type test certificate(s) for the ballast water management system(s) (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004   ☐

Convention / ANNEX / Regulation D2)

- confirming that, if applicable, dosage instruction for active substance or preparations are available on board (BWM 2004 Convention / ANNEX /   ☐

Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- checking whether any new equipment has been fitted and, if so, confirm that it has been approved before installation and that any changes are   ☐

reflected in the certificate

- Confirming that Ballast piping system and pumping system are in good condition in accordance with Reg. D1	☐

- Confirming that BWMS is in good working condition in accordance with Reg. D2	☐

- Confirming that Prototype Ballast Water Treatment technologies and its system is in a good working condition in accordance with Reg. D4	☐

- sighting, if applicable, the Statement of Compliance for a Prototype Ballast Water Treatment Technology (BWM 2004 Convention / ANNEX /   ☐

Regulation D4)

- examining externally the ballast water treatment system and confirming, as far as practicable its satisfactory operation (BWM 2004 Convention   ☐

/ ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that, if applicable, active substances in accordance with the manufacturer’s recommendations are provided on board (BWM 2004   ☐

Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- examining the ballast water management system for obvious defects, deterioration or damage including examining associated pumps, piping   ☐

and fittings for wear and corrosion (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D4)

- verifying, if applicable, that there is evidence that the Prototype Ballast Water Treatment Technology is continuing to be operated in accordance   ☐

with the approved programme (BWM 2004 Convention / ANNEX / Regulation D4)

- where applicable examining externally the prototype ballast water treatment technology and confirming, as far as practicable its satisfactory   ☐

operation (BWM 2004 Convention / ANNEX / Regulation D4)

- if a survey shows that the condition of a ship or its equipment is unsatisfactory, see part General section 4.8*

- *4.8 Completion of surveys

    - If a survey shows that the condition of the ship or its equipment is unsatisfactory, the officer of the Administration, nominated surveyor or recognized organization should be guided by the requirements of SOLAS 74/88 regulation I/6(c), MARPOL 73/78/90 Annex I regulation 4(3)(d), MARPOL 73/78/90 Annex II regulation 10(2)(c), the IBC Code 83/90 regulation 1.5.1.3, the IGC Code 83/90 regulation 1.5.1.3 and the BCH Code 85/90 regulation 1.6.1.3. These instruments require that corrective action be taken immediately, and the Administration notified in due course.   ☐ In cases where the corrective action has not been undertaken the relevant certificate should be withdrawn and the Administration notified

immediately. If the ship is in the port of another Party, the appropriate authorities of the port State also notified immediately.

    - Although LLC 66/88 does not contain specific requirements, if a load line survey shows the condition of the ship or its equipment is

unsatisfactory, the officer of the Administration, nominated surveyor or recognized organization should, nevertheless, be guided by 4.8.1.

- after satisfactory survey, the International Ballast Water Management Certificate should be endorsed	☐

4. RENEWAL SURVEY

- confirming that the Ballast Water Management Plan is on board (BWM 2004 Convention / ANNEX / Regulation B1)	☐

- checking whether the appropriate entries have been made in the Ballast Record Book (BWM 2004 Convention / ANNEX / Regulation B2)	☐

- sighting the type test certificate(s) for the ballast water management system(s) (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004	☐

Convention / ANNEX / Regulation D2)

- sighting, if applicable, the Statement of Compliance for a Prototype Ballast Water Treatment Technology (BWM 2004 Convention / ANNEX /	☐

Regulation D4)

- confirming that, if applicable, dosage instruction for active substance or preparations are available on board (BWM 2004 Convention / ANNEX	☐

/ Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- Confirming that Ballast piping system and pumping system are in good condition in accordance with Reg. D1	☐

- Confirming that BWMS is in good working condition in accordance with Reg. D2	☐

- Confirming that Prototype Ballast Water Treatment technologies and its system is in a good working condition in accordance with Reg. D4	☐

- sighting the records of the recording device, if fitted (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX /	☐

Regulation D2)

- if applicable, confirming, if necessary, by simulated test or equivalent, the satisfactory operation of the ballast water management systems	☐

(BWM 2004 Convention / ANNEX / Regulation D4)

- examining externally the ballast water treatment system and confirming, as far as practicable its satisfactory operation (BWM 2004 Convention	☐

/ ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- confirming that, if applicable, active substances in accordance with the manufacturer’s recommendations are provided on board (BWM 2004	☐

Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D2)

- examining the ballast water management system for obvious defects, deterioration or damage including examining associated pumps, piping	☐

and fittings for wear and corrosion (BWM 2004 Convention / ANNEX / Regulation D3) (BWM 2004 Convention / ANNEX / Regulation D4)

- checking whether any new equipment has been fitted and, if so, confirm that it has been approved before installation and that any changes	☐

are reflected in the certificate

- verifying, if applicable, that there is evidence that the Prototype Ballast Water Treatment Technology is continuing to be operated in accordance	☐

with the approved program (BWM 2004 Convention / ANNEX / Regulation D4)

- where applicable examining externally the prototype ballast water treatment technology and confirming, as far as practicable its satisfactory	☐

operation (BWM 2004 Convention / ANNEX / Regulation D4)

- after satisfactory survey, the International Ballast Water Management Certificate should be issued	☐

5. ADDITIONAL SURVEY (AFTER A CHANGE, REPLACEMENT, OR SIGNIFICANT REPAIR OF THE STRUCTURE, EQUIPMENT, SYSTEMS, FITTINGS, ARRANGEMENTS, AND MATERIAL)

1.- Confirming that a commissioning test has been conducted to validate the installation of any ballast water management system by demonstrating that its mechanical, physical, chemical, and biological processes are working properly, taking into account the guidelines developed by the Organization

Place and Date of Survey

GR CLASS REPRESENTATIVE

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)

E-mail: info@grclass.com Web: www.grclass.com

Form BWMC-01	                                                                                 Approved by: GM			Page 4 of 4