# PURPOSE

  - The purpose of the present procedure is to define and document the standard GR Class  procedure for carrying out water ballast inspections and certification of ships for vessels certified with GR Class  and in light of the Water Ballast Management Requirements.

# RESPONSIBILITY

  - GR Class  Technical Department is required to bear the responsibility of the application of this survey procedure.

# SCOPE OF APPLICATION

  - The present procedure applies whenever a survey and certification activities are to be undertaken by GR Class according to the Water Ballast Management Convention (BWMC).

# PROCEDURE DESCRIPTION

  - APPLICATION:

    - BWMC is applicable to:

- Ships entitled to fly the flag of a Party;

- ships not entitled to fly the flag of a Party but which operate under the authority of a Party

    - This Convention shall not apply to:

- ships not designed or constructed to carry Ballast Water;

- ships of a Party which only operate in waters under the jurisdiction of that Party, unless the Party determines that the discharge of Ballast Water from such ships would impair or damage their environment, human health, property or resources, or those of adjacent or other States;

- ships of a Party which only operate in waters under the jurisdiction of another Party, subject to the authorization of the latter Party for such exclusion. No Party shall grant such authorization if doing so would impair or damage their environment, human health, property, or resources, or those of adjacent or other States. Any Party not granting such authorization shall notify the Administration of the ship concerned that this Convention applies to such ship;

- ships which only operate in waters under the jurisdiction of one Party and on the high seas, except for ships did not grant an authorization pursuant to sub-paragraph (c), unless such Party determines that the discharge of Ballast Water from such ships would impair or damage their environment, human health, property, or resources, or those of adjacent of other States.

- any warship, naval auxiliary or other ship owned or operated by a State and used, for the time being, only on government non-commercial service. However, each Party shall ensure, by the adoption of appropriate measures not impairing operations or operational capabilities of such ships owned or operated by it, that such ships act in a manner consistent, so far as is reasonable and practicable, with this Convention; and

- permanent Ballast Water in sealed tanks on ships, that is not subject to discharge.

# SURVEY AND CERTIFICATION:

    - Regulation E-1:

      - Ships of 400 gross tonnages and above to which BWMC applies; shall be subject to surveys specify below:

- An initial survey before the ship is put in service or before the Certificate required under regulation E-2 or E-3 is issued for the first time. This survey shall verify that the Ballast Water Management plan required by regulation B-1 and any associated structure, equipment, systems, fitting, arrangements and material or processes comply fully with the requirements of this Convention.

- A renewal survey at intervals specified by the Administration, but not exceeding five years, except where regulation E-5.2, E-5.5, E-5.6, or E-5.7 is applicable. This survey shall verify that the Ballast Water Management plan required by regulation B-1 and any associated structure, equipment, systems, fitting, arrangements and material or processes comply fully with the applicable requirements of this Convention.

- An intermediate survey within three months before or after the second Anniversary date or within three months before or after the third Anniversary date of the Certificate, which shall take the place of one of the annual surveys specified in paragraph 1.4. The intermediate surveys shall ensure that the equipment, associated systems and processes for Ballast Water Management fully comply with the applicable requirements of this

Annex and are in good working order. Such intermediate surveys shall be endorsed on the Certificate issued under regulation E-2.

- An annual survey within three months before or after each Anniversary date, including a general inspection of the structure, any equipment, systems, fittings, arrangements and material or processes associated with the Ballast Water Management plan required by regulation B-1 to ensure that they have been maintained in accordance with paragraph 9 and remain satisfactory for the service for which the ship is intended. Such annual surveys shall be endorsed on the Certificate issued under regulation E-2.

- According to the circumstances, an additional survey, either general or partial, shall be made after a change, replacement, or significant repair of the structure, equipment, systems, fittings, arrangements and material necessary to achieve full compliance with this Convention. The survey shall be such as to ensure that any such change, replacement, or significant repair has been effectively made, so that the ship complies with the requirements of this Convention. Such surveys shall be endorsed on the Certificate issued under regulation E-2.

    - Regulation E-2:

      - GR Class  shall ensure that a ship to which regulation E-1 applies is issued a Certificate after successful completion of an initial or Renewal surveys conducted in accordance with regulation E-1, in accordance with the instructions and provisions provided by Administrations. This certificate is called short term International/Statement of Complaince Water Ballast Management Certificate and is drawn up according to Form BWMC-ST and is issued by the attending surveyor upon satisfactory completion of the related survey.

      - Certificates shall be endorsed by the attending surveyor upon satisfactory completion of an Annual or Intermediate surveys.

      - Survey report Form (BWMC-01) is to be completed, as appropriate, to leave evidence that the corresponding survey has been completed.

      - Full Term certificate/statement of compliance will be issued by HO or appropriate regional offices following the appropriate process according to the Form (BWMC- FT).

    - Regulation E-4:

      - The Short term (Interim) and Full-term certificates are drawn up in the form set forth in Appendix I of the BWMC.

    - Regulation E-5:

      - For renewal surveys:

- When the renewal survey is completed within three months before the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of expiry of the existing Certificate.

- When the renewal survey is completed after the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of expiry of the existing Certificate.

- When the renewal survey is completed more than three months before the expiry date of the existing Certificate, the new Certificate shall be valid from the date of completion of the renewal survey to a date not exceeding five years from the date of completion of the renewal survey

# BALLAST MANAGEMENT PLAN (B-1):

    - Each ship shall have on board and implement a Ballast Water Management plan. Such a plan shall be approved by the Administration considering Guidelines developed by the IMO as per IMO Resolution MEPC.127 (53) - Guidelines for ballast water management and development of ballast water management plans (G4) adopted by the IMO on 22 July 2005.

    - The Ballast Water Management plan shall be specific to each ship and shall at least:

- Detail safety procedures for the ship and the crew associated with Ballast Water Management as required by the Convention;

- Provide a detailed description of the actions to be taken to implement the Ballast Water Management requirements and supplemental Ballast Water Management practices as set forth in the Convention;

- Detail the procedures for the disposal of Sediments:

  - at sea; and

  - to shore;

- Include the procedures for coordinating shipboard Ballast Water Management that involves discharge to the sea with the authorities of the State into whose waters such discharge will take place;

- Designate the officer on board in charge of ensuring that the plan is properly implemented;

- Contain the reporting requirements for ships provided for under the Convention; and

- Be written in the working language of the ship. If the language used is not English, French, or Spanish, a translation into one of these languages shall be included.

# BALLAST WATER RECORD BOOK:

    - Each ship shall have on board a Ballast Water record book that may be an electronic record system, or that may be integrated into another record book or system and, which shall at least contain the information specified in Appendix II.

    - Ballast Water record book entries shall be maintained on board the ship for a minimum period of two years after the last entry has been made and thereafter in the Company's control for a minimum period of three years.

    - In the event of the discharge of Ballast Water pursuant to regulations A-3, A-4 or B-

3.6 or in the event of other accidental or exceptional discharge of Ballast Water not otherwise exempted by the Convention, an entry shall be made in the Ballast Water record book describing the circumstances of, and the reason for, the discharge.

    - The Ballast Water record book shall be kept readily available for inspection at all reasonable times and, in the case of an unmanned ship under tow, may be kept on the towing ship.

    - Each operation concerning Ballast Water shall be fully recorded without delay in the Ballast Water record book. Each entry shall be signed by the officer in charge of the operation concerned and each completed page shall be signed by the master. The entries in the Ballast Water record book shall be in a working language of the ship. If that language is not English, French, or Spanish the entries shall contain a translation into one of those languages.

    - Officers duly authorized the flag state may inspect the Ballast Water record book on board any ship to which this regulation applies while the ship is in its port or offshore terminal, and may make a copy of any entry, and require the master to certify that the copy is a true copy. Any copy so certified shall be admissible in any judicial proceeding as evidence of the facts stated in the entry. The inspection of a Ballast Water record book and the taking of a certified copy shall be performed as expeditiously as possible without causing the ship to be unduly delayed.

# BALLAST WATER MANAGEMENT OF SHIP:

    - A ship constructed before 2009:

- With a Ballast Water Capacity of between 1,500 and 5,000 cubic meters, inclusive, shall conduct Ballast Water Management that at least meets the standard described in regulation D-1 or regulation D-2 until 2014, after which time it shall at least meet the standard described in regulation D-2;

- With a Ballast Water Capacity of less than 1,500 or greater than 5,000 cubic meters shall conduct Ballast Water Management that at least meets the standard described in regulation D-1 or regulation D-2 until 2016, after which time it shall at least meet the standard described in regulation D-2.

- A ship to which paragraph 1 above applies shall comply with paragraph 1 not later than the first intermediate or renewal survey, whichever occurs first, after the anniversary date of delivery of the ship in the year of compliance with the standard applicable to the ship.

    - A ship constructed in or after 2009:

a) With a Ballast Water Capacity of less than 5,000 cubic meters shall conduct Ballast Water Management that at least meets the standard described in regulation D-2.

    - A ship constructed in or after 2009, but before 2012:

a) With a Ballast Water Capacity of 5,000 cubic meters or more shall conduct Ballast Water Management in accordance with paragraph 1.2.

    - A ship constructed in or after 2012:

a) with a Ballast Water Capacity of 5000 cubic meters or more shall conduct Ballast Water Management that at least meets the standard described in regulation D-2.

    - The requirements of this regulation do not apply to ships that discharge Ballast Water to a reception facility designed taking into account the Guidelines developed by the IMO for such facilities.

    - Other methods of Ballast Water Management may also be accepted as alternatives to the requirements described in paragraphs 1 to 5, provided that such methods ensure at least the same level of protection to the environment, human health, property or resources, and are approved in principle by the Committee.

# REGULATIONS:

    - Standards for Ballast Water Management D-1:

- Ships performing Ballast Water exchange in accordance with this regulation shall do so with an efficiency of at least 95 percent volumetric exchange of Ballast Water.

- For ships exchanging Ballast Water by the pumping-through method, pumping through three times the volume of each Ballast Water tank shall be considered to meet the standard described in paragraph 1. Pumping through less than three times the volume may be accepted provided the ship can demonstrate that at least 95 percent volumetric exchange is met.

    - Ballast Water Performance Standards D-2:

- Ships conducting Ballast Water Management in accordance with this regulation shall discharge less than 10 viable organisms per cubic meters greater than or equal to 50 micrometers in a minimum dimension and less than 10 viable organisms per milliliter less than 50 micrometers in minimum dimension and greater than or equal to 10 micrometers in minimum dimension; and discharge of the indicator microbes shall not exceed the specified concentrations described in paragraph 2.

- Indicator microbes, as a human health standard, shall include:

  - Toxicogenic Vibrio cholerae (O1 and O139) with less than 1 colony forming unit (cfu) per 100 milliliters or less than 1 cfu per 1 gram (wet weight) zooplankton samples

  - Escherichia coli less than 250 cfu per 100 milliliters.

  - Intestinal Enterococci less than 100 cfu per 100 milliliters.

  - NEW SCHEDULE FOR IMPLEMENTATION DATES (MEPC 71)

    - New ships must meet the D-2 standard.

    - All ships must have: a ballast water management plan, a ballast water record book and an International Ballast Water Management Certificate.

    - Existing ships must meet at least the D-1 (ballast water exchange) standard; they may also choose to install a ballast water management system or otherwise meet the D-2 (discharge) standard but this is not mandatory until the corresponding compliance date

    - IOPPC Renewal survey after 8 September 2019 - A ship undergoing a renewal survey linked to the ship's International Oil Pollution Prevention Certificate after 8 September 2019 will need to meet the D-2 standard by the date of this renewal survey.

    - IOPPC Renewal survey between 8 September 2017 and 8 September 2019 - If the previous IOPPC renewal survey was between 8 September 2014 and 8 September 2017, then the ship must comply with D-2 standard by this renewal survey. If the previous IOPPC renewal survey was before 8 September 2014, then the ship can wait until the next renewal survey (which will be after 8 September 2019).

# COMMISIONING TEST

    - Amendments to regulation E-1.1.1 and E-1.1.5 of BWM Convention – Entry into Force 1 June 2022 - Mandatory commissioning testing of BWMS installed on or after 1 June 2022.

    - IMO MEPC 75 adopted the amendments to the BWMC (resolution of MEPC.325(75)) which introduce mandatory commissioning testing of BWMS at the initial survey for new ships and at an additional survey for new BWMS to be

installed on existing ships.

    - The purpose of commissioning testing is to validate the installation of a ballast water management system (BWMS) by demonstrating that its mechanical, physical, chemical and biological processes are working properly. Commissioning testing is not intended to validate the design of type-approved BWMS that are approved by the Administration.

    - Surveyor to be guided by GR check list.

  - The following Guidance for the commissioning testing of BWMS has been developed for use by persons fitting and verifying the installation of BWMS in accordance with:

    - regulation E-1 of the Convention;

    - paragraph 8.2.5 of the BWMS Code, which requires that the Administration issuing

the international ballast water management certificate verify that installation commissioning procedures are on board the ship in a suitable format;

- paragraph 8.3.6 of the BWMS Code, which requires that the installation commissioning procedures have been completed prior to the issuance of the IBWMC following the installation of a BWMS; and

- paragraph 1.18 of resolution MEPC.174(58), which provides that, when a type-approved ballast water management system is installed on board, an installation survey according to section 8 should be carried out

# PROCEDURAL REFERENCES

  - Guidelines for ballast water management and development of ballast water management plans (G4) adopted by the IMO on 22 July 2005 - IMO Resolution MEPC.127 (53)

  - PMA MMC-362 - List of accepted Companies with type approval Certificate of Ballast Water Management System.

  - PMA MMC-348 - Recognized Organizations authorized for the issuance of the Provisional Certificate under the International Ballast Water Management Convention, 2004.

  - PMA MMC-345 – Ballast Water Management Convention 2004, Panama Policy.

  - SKAN MC 74/17 - Ballast Water Management Implementation

  - SKAN MC 115/22 - Sampling and Analysis of Ballast Water at Commissioning Tests Ballast Water Management Systems.

  - Barbados Bulletin 272 – Ballast Water Management Convention.

  - Belize MMN-17-003r1 – Survey and Certification for the BWM Convention.

  - SLMARAD MC-36 – Ballast Water Management Policy.

# RECORD REFERENCES

  - Ballast Water Management Certificate Short Term Form (BWMC-ST)

  - Ballast Water Management Certificate Full Term Form (BWMC-FT)

  - Ballast Water Management Survey Report (BWMC-01)

# DEFINITIONS

  - BWMC: Ballast Water Management Certificate.

  - Ballast Water Tank: means any tank, hold, or space used for the carriage of ballast water.