| SOLAS CH. 1-REG.10 |  |
| --- | --- |
| GR-PM-BS | BOTTOM SURVEYS AND INSPECTIONS |

General

  - The Owner or attending surveyor is to notify GR whenever the outside of the ship's bottom and related items can be examined in drydock or on a slipway and ask for the corresponding Job Request (JR).

  - There is to be a minimum of two examinations of the outside of the ship’s bottom and related items during each five-year special survey period. One such examination is to be carried out in conjunction with the special survey. In all cases the interval between any two such examinations is not to exceed 36 months. An extension of examination of the ship’s bottom of 3 months beyond the due date can be granted in exceptional circumstances (unavailability of dry-docking facilities; unavailability of repair facilities; unavailability of essential materials, equipment, or spare parts; or delays incurred by action taken to avoid severe weather conditions). Under these circumstances, it is to be contacted the head office and request an extension under flag state. GR head office or corresponding branch will provide requirements for this.

  - Examinations of the outside of the ship’s bottom and related items of ships is normally to be carried out with the ship in drydock. However, consideration may be given to alternate examination while the ship is afloat as an In-water Survey, subject to provisions of concerned flag state.

  - The interval between examinations of the outside of the ship's bottom and related items for ships operating in fresh water and for certain harbour or non-self-propelled craft may be greater than that given in 1.1

  - Compliance with this instructive does not absolve the Owner from compliance with the requirements of SOLAS as amended, especially when shorter intervals between examination of the ship’s bottom for certain types of ship are required.

# SECTION 2

Scope of Surveys

  - When a ship is in drydock or on a slipway, it is to be placed on blocks of sufficient height and with the necessary staging to permit the examination of elements such as shell plating including bottom and bow plating, stern frame and rudder, sea chests and valves, propeller, etc.

  - The shell plating is to be examined for excessive corrosion, or deterioration due to chafing or contact with the ground and for any undue unfairness or buckling. Special attention is to be paid to the connection between the bilge strakes and the bilge keels. Important plate unfairness or other deterioration which do not necessitate immediate repairs are to be recorded.

  - Sea chests and their gratings, sea connections and overboard discharge valves and cocks and their fastenings to the hull or sea chests are to be examined. Valves and cocks need not be opened up more than once in a special survey period unless considered necessary by the Surveyor.

  - Visible parts of rudder, rudder pintles, rudder shafts and couplings and stern frame are to be examined. If considered necessary by the Surveyor, the rudder is to be lifted or the inspection plates removed for the examination of pintles. The clearance in the rudder bearings is to be ascertained and recorded. Where applicable, pressure test of the rudder may be required as deemed necessary by the surveyor.

  - Visible parts of propeller and stern bush are to be examined. The clearance in the stern bush and the efficiency of the oil gland, if fitted, are to be ascertained and recorded. For controllable pitch propellers, the Surveyor is to be satisfied with the fastenings and tightness of hub and blade sealing. Dismantling need not to be carried out unless considered necessary by the Surveyor.

  - Visible parts of side thrusters are to be examined. Other propulsion systems which also have maneuvering characteristics (such as directional propellers, vertical axis propellers, water jet units) are to be examined externally with focus on the condition of gear housing, propeller blades, bolt locking  and other fastening arrangements. Sealing arrangement of propeller blades, propeller shaft and steering column shall be verified.

| SOLAS CH. 1-REG.10 |  |
| --- | --- |
| GR-PM-BS | BOTTOM SURVEYS AND INSPECTIONS |

In-water surveys

  - The In-water Survey is to provide the information normally obtained from a docking survey. Special consideration shall be given to ascertaining rudder bearing clearances and stern bush clearances of oil stern bearings based on a review of the operating history, on board testing and stern oil sample reports. These considerations are to be included in the proposals for in-water survey which are to be submitted in advance of the survey so that satisfactory arrangements can be agreed with GR.

  - Every in-water survey permit is to be requested for proceeding to either GR head office or corresponding branch.

  - The In-water Survey is to be carried out with the ship in sheltered water and preferably with weak tidal streams and currents. The in-water visibility and the cleanliness of the hull below the waterline is to be clear enough to permit a meaningful examination which allows the surveyor and the in-water survey firm to determine the condition of the plating, appendages and the welding. GR is to be satisfied with the methods of orientation of the divers, which should make use where necessary of permanent markings on the plating at selected points.

  - The equipment, procedure for observing and reporting the survey are to be discussed with the parties involved prior to the In-water Survey, and suitable time is to be allowed to permit the in-water survey firm to test all equipment beforehand.

  - The In-water Survey is to be carried out under the surveillance of an GR surveyor.

  - The Surveyor is to be satisfied with the method of pictorial representation, and a good two-way communication between the Surveyor and divers is to be provided.

  - If the In-water Survey reveals damage or deterioration that requires early attention, the Surveyor may require that the ship be drydocked in order that a detailed survey can be undertaken, and the necessary repairs carried out.

# SECTION 4

Reporting

  - The following are GR checklist and reports applicable for the bottom survey assignment:

    - Bottom survey inspection report	BS-01

    - Supplement to the bottom survey inspection report	BS-02

    - Docking survey check list	BS-03

    - GR Survey Statement Report	P24 REFERENCE

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC No. 204: Outside Ship’s Bottom Inspection and Dry docking Interface Periods for Panamanian Flagged Vessels. | 02/2022 | https://panamashipregistry.com/circula r-category/circulares/ |
|  | MMC No. 289: Bottom Surveys of Mobile Offshore Units (Drilling and non-Drilling) certificate under MODU Code adoption. | 04/2014 |  |
| Barbados | NA |  | https://barbadosmaritime.org/bulletins / |

| SOLAS CH. 1-REG.10 |  |
| --- | --- |
| GR-PM-BS | BOTTOM SURVEYS AND INSPECTIONS |

| Belize | MMN-18-001: Outside Ship ́s Bottom Inspection Requirements for Belize Registered Vessels. | 15/01/20 18 | https://www.immarbe.com/circularslist / |
| --- | --- | --- | --- |
| Sierra Leone | MC No. 13: Extension of Bottom Survey, Special Survey or Statutory Certificates Policy. | 10/01/20 17 | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information -centre/online-library/marine- documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/document ation/instruments-and-laws/ |
| Honduras | NA | NA | https://merchanGMarine.gob.hn/aspect os-legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disp osiciones |
| Nigeria | NA | NA | https://nimasa.gov.ng/press- center/marine-notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circular s |
| Colombia | NA | NA | https://www.dimar.mil.co/normativida d |
| Guinea Bissau | NA | NA | NA |