# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

REPORT OF CARGO SHIP SAFETY CONSTRUCTION SURVEYS                                        ANNEX II – BULK CARRIERS

| SHIP NAME: | IMO No.: | CALL SIGN: | FLAG: | CLASS No. |
| --- | --- | --- | --- | --- |

II. BULK CARRIERS

1. ANNUAL SURVEYS

- Please, complete section 1(A, B, C) of form CCC-01 for general cargo

- Additionally please, verify the following:

- confirming, that the loading /unloading booklet required in SOLAS regulation VI/7 is on board (SOLAS 74/97 Reg.XII/8.1);

- Confirming, with restrictions imposed with respect to the carriage of cargoes with a density of 1,780 kg/m3 and above, that a triangle is permanently marked at midship (SOLAS 74/97 reg.XII/8.3);

- Confirming, that the loading instruments is on board functioning (SOLAS 74/97 reg.XII/11);

- Confirming, for bulk carriers of 150 m in length and upwards of single skin construction designed to carry solid bulk cargoes having a density of 1,780 kg/m3 and above, constructed before 1 July 1999,have, after the implementation date given in SOLAS 74/97 reg.XII/3, sufficient stability and strength to withstand flooding of the foremost cargo hold (SOLAS 74/97 reg.XII/3,4,5 and 6)1;

- Confirming approved Cargo Securing Manual for ships carrying cargo units including containers is on board (SOLAS 74/94 reg.VI/5.6);

- Confirming that the loading booklet for carriage of cargoes in bulk is on board (SOLAS 74/00 regVI/7);

- Confirming when appropriate, that the Ship Structure Access Manual is on board (SOLAS 74/00 Reg.II-1/3-7);

- Confirming that structural alterations performed, if any, have been approved by GR or other classification society and reported on the as-built drawings kept on board (SOLAS 74/04 reg.II-1/3-7).

.11. Confirming that the corrosion prevention system fitted to dedicated ballast water tanks is maintained (SOLAS 74/00 reg.II-1/3-3);

- Confirming, when appropriate, that a complete file of the enhanced survey reports and the Condition evaluation Report are on board;

- Confirming, when appropriate and as far as is practicable when examining internal spaces that the means of access to cargo and other spaces remain in good condition. (SOLAS 74/00/02 Reg.II-1/3-6);

- Examining the hold, ballast and dry space water level detectors and their audible and visual alarms (SOLAS 74/02 reg.XII/12);

- Checking the arrangements for availability of draining and pumping systems forward of the collision bulkhead (SOLAS 74/02 reg.XII/13);

- Examination of the hull and its closing appliances as far as can be seen (A. 744(18) as amended);

- Examination of watertight penetrations as far as practicable (A. 744(18) as amended);

1 If the vessel is in compliance with this Regulation, please complete section 25.1 of this report, if not; goes to section 25.2.

- please examine the following on Hatch covers and coamings,:

  - Confirm that no unapproved modification has been made to hatch covers, hatch coamings and their securing and sealing devices since last survey (A. 744(18) as amended);

  - Please verify proper opening and closing operation of at least 25% of cargo hatch covers and coaming within the forward length and at least one additional set at every annual (A. 744(18) as amended)

Cargo hatch cover and coaming

Stowage & securing in open

condition

Proper fit and efficiency of sealing  Operational testing of hydraulic and

in closed condition

power components, wires, chains,

and link drives

No.

No.

No.

No.

Additional Sets No.

No.

No.

  - Where the cargo hatch securing system does not function properly, repairs were carried out to surveyor’s satisfaction2;

- For each cargo hatch cover set, the following items should be surveyed:

- Cover panels, including side plates and stiffeners attachments that may be accessible in the open position by close-up survey (for corrosion, cracks & deformation);

- Sealing arrangement of perimeter of cross joints (gaskets for condition and permanent deformation, flexible

seals on combination carriers, gasket lips, compression bars, drainage channels and non return valves;

- Clamping devices, retaining bars, cleating(for wastage, adjustment and condition of rubber components)

- Closed cover locating devices (for distortion and attachment);

- Chain or rope pulleys;

- Guides;

- Guide rails and track wheels;

- Stoppers;

- Wires, chains tensioners and gypsies;

- Hydraulic system, electrical safety devices and interlocks;

- end and interpanel hinges, pins, and stools where fitted

- At each hatchway the comings with plating, stiffeners an brackets should be checked for corrosion, cracks and deformation, specially of the coaming tops;

- When considered necessary, the effectiveness of sealing arrangements may be proved by hose or chalk testing supplemented by dimensional measurements of seal compressing components;

- Where portable covers, wooden or steel pontoons are fitted, please check the satisfactory condition where applicable of:

- Please enter Either Good, Fair or Poor. For bad entries please complete RPT-02. All Cargo Hatch covers are to be assessed at least once every 5-year period. If there are indications of difficulty in operating and securing hatch covers, additional sets required in paragraph 18.2 here above, at the discretion of the surveyor, should be tested in operation.

- Please enter Either Good, Fair or Poor. For bad entries please complete RPT-02. All Cargo Hatch covers are to be assessed at least once every 5-year period. The closing of covers should include the fastening of all peripherals, and cross joint cleats or other securing devices. If there are indications of difficulty in operating and securing hatch covers, additional sets required in paragraph 18.2 here above, at the discretion of the surveyor, should be tested in operation.

- Please enter Either Good, Fair or Poor. For bad entries please complete RPT-02. All Cargo Hatch covers are to be assessed at least once every 5-year period. If there are indications of difficulty in operating and securing hatch covers, additional sets required in paragraph 18.2 here above, at the discretion of the surveyor, should be tested in operation.

2 Where hatch coves or coaming undergo substantial repairs, the strength of securing devices should be upgraded to comply with A. 744 (18) annex 13 as amended.

- wooden covers and portable beams, carriers or sockets for portable beam, and their securing devices

- steel pontoons including close-up survey of hatch cover plating;

- tarpaulins;

- cleats, battens and wedges;

- hatch securing bars and their securing devices;

- loading pads/bars and the side plate edge;

- guide plates and chocks;

- compression bars, drainage channels and drain pipes (if any).

- Examination of cargo holds

  - For ships over 10 years of age, the following should be carried out:

- Overall survey of ALL cargo holds3 and twin deck spaces

- Close-up examination of sufficient extent, minimum 25% of frames, to establish the condition of the lower region of the shell frames including approx. lower one third length of side frame at side shell and side frame end attachment and the adjacent shell plating in a forward cargo hold.4

.c Was considered necessary by the surveyor, or where extensive corrosion existed, that thickness measurement were taken5;

  - for ships over 15years of age, the following should be carried out:

- Overall Surveys of ALL cargo holds² and twin deck spaces.

- Close-up examination of sufficient extent, minimum 25% of frames, to establish the condition of the lower region of the shell frames including approx. lower one third length of side frame at side shell and side frame

end attachment and the adjacent shell plating in a forward cargo hold and one other selected cargo hold³.

- Was considered necessary by the surveyor, or where extensive corrosion existed, that thickness measurement were taken4;

- Examine all piping and penetrations in cargo holds, including overboard piping

- Examination of ballast tanks

  - If water ballast tanks are not coated or if class survey status requires internal examination please confirm that such spaces were inspected. Use SS-001/SS-002 to give details;

- Additional annual survey requirements for the foremost cargo hold of ships:

  - If the ship complies with SOLAS Chap. XII, Reg. 4.3 (as it is written below) an annual should consist of:

Bulk carriers of 150 m in length and upwards of single-side skin construction, carrying solid bulk cargoes having a density of 1,780 kg/m3 and above, constructed before 1 July 1999 shall, when loaded to the summer load line, be able to withstand flooding of the foremost cargo hold in all loading conditions and remain afloat in a satisfactory condition of equilibrium, as specified in paragraph 4. This requirement shall be complied with in accordance with the implementation schedule specified in regulation 3.

- for ships of 5 – 15 years of age, an overall survey of the foremost cargo hold, including close-up survey6 of sufficient extend, minimum 25% of frames is to be carried out to establish the condition7 of;

3  Where a protective coating is provided in cargo holds and is found in GOOD condition, the extent of close up survey and / thickness measurements may be especially considered. Use RPT-02 to give detail if this is the case.

4 Where this level of survey reveals the need for remedial measures, the survey should be extended to include a Close-up Survey of all of the shell frames and adjacent shell plating of that cargo hold as

well as a close-up survey of sufficient extent of all remaining cargo holds.

5 If the results of these thickness measurements indicate that substantial corrosion is found, the extent of thickness measurements should be increased in accordance with annex 10 of A.744 (18).

6  Thicknessmeasurementshouldbecarriedouttoanextentsufficienttodeterminebothgeneralandlocalcorrosionlevelsatareassubjecttoclose-upsurvey, asdescribedin25.1aand25.1b. Theminimumrequirementforthicknessmeasurementsare areas foundtobesuspect areas at thepreviousrenewal survey. Wheresubstantial corrosion is found, theextent of thickness measurements shouldbeincreasedwiththerequirements ofannex 10 of A. 744(18). Thethickness measurement may be dispensedwithprovidedthesurveyorissatisfiedbytheclose-upsurvey,thereisnostructuraldiminutionandtheprotectivecoating,whereapplied,remainseffective.

Where the protective coating, as referred to in the explanatory note below, in the foremost cargo hold is found to be in GOOD condition, the extent of close-up surveys and thickness measurements may be specially considered

7   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin25.1aabove,thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargohold.

  - Shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads;

  - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSRR

- for ships exceeding 15 years of age, an overall survey of the foremost cargo hold, including close-

up survey6 of sufficient extend, minimum 25% of frames is to be carried out to establish the condition8 of;

  - ALL shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads;

  - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSRR

  - If the ship does not complies with SOLAS Chap. XII, Reg. 4.3 an annual should consist of:

- for ships of 5 – 10 years of age, an overall survey of the foremost cargo hold, including close-up survey9 of sufficient extend, minimum 25% of frames is to be carried out to establish the condition10of;

  - Shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads in the forward cargo hold;

  - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSRR

- for ships of 10 – 15 years of age, an overall survey of the foremost cargo hold, including close-up survey11 of sufficient extend, minimum 25% of frames is to be carried out to establish the condition12of;

  - Shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads in the forward cargo hold;

  - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSRR

- for ships exceeding 15 years of age, an overall survey of the foremost cargo hold should include;

  - Examination of the item 18,19,20,21 & 22 above as applicable;

  - Checking the satisfactory operation of all mechanically hatch covers including stowage and securing in Open condition, proper and efficiency of sealing in closed condition, operational testing of hydraulic and power components, wires, chains and link drives;

  - The effectiveness of sealing arrangements of the foremost cargo hatch cover is to be checked by hose Testing or equivalent

  - Thickness measurement of the foremost cargo hatch cover, coaming plating and stiffeners should be carried out as given in annex 2, A. 744 (18) (requirements for thickness measurement at renewal surveys);

- A survey planning questionnaire is to be completed by the owner according to the form SPQ-BC for the foremost cargo hold

- A survey program is to be developed according to the form CCC-02 for the Foremost cargo hold

- for the hull, machinery and equipment of cargo ships the completion of the annual survey should consist of:

8   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin25.1aabove,thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargohold.

9  Thicknessmeasurementshouldbecarriedouttoanextentsufficienttodeterminebothgeneralandlocalcorrosionlevelsatareassubjecttoclose-upsurvey, asdescribedin25.2aand25.2b. Theminimumrequirementforthicknessmeasurementsare areas foundtobesuspect areas at thepreviousrenewal survey. Wheresubstantial corrosion is found, theextent of thickness measurements shouldbeincreasedwiththerequirements ofannex 10 of A. 744(18). Thethickness measurement may be dispensedwithprovidedthesurveyorissatisfiedbytheclose-upsurvey,thereisnostructuraldiminutionandtheprotectivecoating,whereapplied,remainseffective.

Where the protective coating, as referred to in the explanatory note below, in the foremost cargo hold is found to be in GOOD condition, the extent of close-up surveys and thickness measurements may be specially considered

10   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin25.2aabove,thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargohold.

11   Thicknessmeasurementshouldbecarriedouttoanextentsufficienttodeterminebothgeneralandlocalcorrosionlevelsatareassubjecttoclose-upsurvey, asdescribedin25.2aand25.2b. Theminimumrequirementforthicknessmeasurementsare areas foundtobesuspect areas at thepreviousrenewal survey. Wheresubstantial corrosion is found, theextent of thickness measurements shouldbeincreasedwiththerequirements ofannex 10 of A. 744(18). Thethickness measurement may be dispensedwithprovidedthesurveyorissatisfiedbytheclose-upsurvey,thereisnostructuraldiminutionandtheprotectivecoating,whereapplied,remainseffective.

Where the protective coating, as referred to in the explanatory note below, in the foremost cargo hold is found to be in GOOD condition, the extent of close-up surveys and thickness measurements may be specially considered

12   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin25.2aabove,thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargohold.

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
E-mail: info@grclass.com Web: www.grclass.com

Form CCC-02	                                                                                 Approved by: GM	 	Page 4 of 7

  - after a satisfactory survey, the Cargo Ship Safety Construction Certificate was endorsed;

  - if a survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

2. (INTERMEDIATE SURVEYS ON BULK CARRIERS)

- Please, complete section 2 of form SC-001 for general cargo

- Additionally please, complete section II-1 of this report:

- Additionally please, verify the following:

  - For ships 5 - 10 years of age, the following should be carried out:

    - Ballast tanks:

      - An overall survey of representative spaces selected by the surveyor should be carried out13. Please use SS-001/SS-002 to give details of the spaces examined14;

      - Areas found to be suspected areas at the previous renewal survey should also be overall and close-up surveyed. Use SS-01/SS-02 to give details where examined.

    - Cargo Holds:

An overall survey of ALL cargo hold, including close-up survey of sufficient extend, minimum 25% of frames is to be carried out to establish the condition15of;

      - Shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads in the forward cargo hold and one other selected cargo hold;

      - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSR

    - Extent of thickness measurement:

      - Areas subject to close up survey as required in item 3.1.b.i

      - Areas found to be suspect areas16 at the previous Renewal survey. See MTO section of CSSR

  - For ships 10 - 15 years of age, the following should be carried out:

    - Ballast tanks:

      - ALL salt water ballast tanks should be examined17. Please use SS-001/SS-002 to give details of the spaces examined18;

      - Areas found to be suspect areas at the previous Renewal survey should be overall and close-up Surveyed. See MTO section of CSSR

13 If such inspections reveal no visual structural defects, the examination may be limited to a verification of the protective coating remains efficient. Where POOR coatings condition, corrosion or other defects are found in salt water ballast spaces or where protective coating was not applied from the time of construction, the examination should be extended to other ballast spaces of the same type. See MTO section of CSS.

14 In salt water ballast spaces other than double bottom tanks, where protective coating is found in POOR condition and it is not renewed, or where a soft coating has been applied, or where a protective

coating was not applied from the time of construction, the tanks in questions should be examined and thickness measurement carried out as considered necessary at annual intervals. When such breakdown is found in salt water double bottom tanks, where a soft coating has been applied, or where a coating has not been applied, the tanks in question should be examined at annual intervals. When considered necessary by the surveyor, or where extensive corrosion exists, thickness measurement should be carried out.

15   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin3.1abaabove, thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargoholdas

wellasaclose-upsurveyofsufficientextentofallremainingcargoholds.

16 Wheresubstantialcorrosionisfound, theextentofthicknessmeasurementsshouldbeincreasedwiththerequirementsofannex 10 of A. 744(18). Thethicknessmeasurementmaybedispensedwithprovidedthesurveyorissatisfiedbytheclose-up survey,thereisnostructuraldiminutionandtheprotectivecoating,whereapplied,remainseffective.

Where the protective coating, as referred to in the explanatory note below, in the foremost cargo hold is found to be in GOOD condition, the extent of close-up surveys and thickness measurements may be specially considered.

17 If such inspections reveal no visual structural defects, the examination may be limited to a verification of the protective coating remains efficient.

18 Where POOR coatings condition, corrosion or other defects are found in salt water ballast spaces or where protective coating was not applied from the time of construction, the examination should be extended to other ballast spaces of the same type.

In salt water ballast spaces other than double bottom tanks, where protective coating is found in POOR condition and it is not renewed, or where a soft coating has been applied, or where a protective coating was not applied from the time of construction, the tanks in questions should be examined and thickness measurement carried out as considered necessary at annual intervals. When such breakdown is found in salt water double bottom tanks, where a soft coating has been applied, or where a coating has not been applied, the tanks in question should be examined at annual intervals. When considered necessary by the surveyor, or where extensive corrosion exists, thickness measurement should be carried out.

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
                                                                                                                             E-mail: info@grclass.com Web: www.grclass.com

Form CCC-02	                                                                  Approved by: GM	 	                                                                Page 5 of 7

    - Cargo Holds:

An overall survey of ALL cargo hold, including close-up survey of sufficient extend, minimum 25% of frames is to be carried out to establish the condition19of;

      - Shell frames including their upper and lower end attachment, adjacent shell plating and transverse bulkheads of ALL cargo hold;

      - Areas found to be suspect areas at the previous Renewal survey. See MTO section of CSSR

    - Extent of thickness measurement:

      - Areas subject to close up survey as required in item 3.1.b.

      - Areas found to be suspect areas20 at the previous Renewal survey. See MTO section of CSSR

    - For ore carriers:

      - All web frame rings – in one ballast wing tank;

      - On deck transverse – in each of the remaining ballast wing tanks;

      - Both transverse bulkheads – in one ballast wing tank;

      - On transverse bulkhead – in each of the remaining ballast wing tanks;

  - for ships exceeding 15 years of age;

    - Examination of the item 18,19,20,21 & 22 above as applicable;

    - Checking the satisfactory operation of all mechanically hatch covers including stowage and securing in open condition, proper and efficiency of sealing in closed condition, operational testing of hydraulic and power components, wires, chains and link drives;

    - The effectiveness of sealing arrangements of ALL hatch cover is to be checked by hose Testing or equivalent;

    - Thickness measurement of the hatch cover and coating plating and stiffeners should be carried out as given in annex 2, A. 744 (18) (requirements for thickness measurement at renewal surveys);

    - A survey planning questionnaire is to be completed by the owner according to the form SPQ-BC for ALL cargo holds;

    - A survey program is to be developed according to the form SP-BC for the ALL cargo holds

- after a satisfactory survey, the Cargo Ship Safety Construction Certificate is endorsed;

- if the survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

- If the inspection of the outside of the ship’s bottom is carried out in conjunction with Intermediate survey please, additionally complete section 4 of form CCC-001 for general cargo and section 4 of this report

19   Whereconsiderednecessarybythesurveyorasaresultoftheoverallandclose-upsurveyasdescribedin3.1abaabove, thesurveyshouldbeextendedtoincludeaclose-upsurveyofalloftheshellframesandadjacentshellplatingofthecargoholdas wellasaclose-upsurveyofsufficientextentofallremainingcargoholds.

20  Wheresubstantialcorrosionisfound, theextentofthicknessmeasurementsshouldbeincreasedwiththerequirementsofannex 10 of A. 744(18). Thethicknessmeasurementmaybedispensedwithprovidedthesurveyorissatisfiedbytheclose-up

survey,thereisnostructuraldiminutionandtheprotectivecoating,whereapplied,remainseffective.

Where the protective coating, as referred to in the explanatory note below, in the foremost cargo hold is found to be in GOOD condition, the extent of close-up surveys and thickness measurements may be specially considered.

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
E-mail: info@grclass.com Web: www.grclass.com

Form CCC-02	                                                                        Approved by: GM	 	                                                    Page 6 of 7

3. RENEWAL21 SURVEYS

- Please, complete section 3 of form CCC-001 for general cargo

- Please, additionally complete section 1 of this form and follow the guidelines (GUI-ESP-BC)

- Additionally please, verify the following:

  - All piping system within ballast tanks, pipe tunnels, cofferdams and void spaces should be examined and operationally tested under working conditions to ensure that their condition remains satisfactory

  - Verify that close-up survey and thickness measurement has been carried out according to Annex 1 and Annex 2 of A. 744 (18) as amended.

- After a satisfactory survey, the Cargo Ship Safety Construction Certificate should be issued.

4. DRY-DOCK SURVEYS22

.1 Please, complete section 4 of form CCC-01 for general cargo

Place and Date of Survey

GR CLASS Surveyor Name – ID No.

21 The renewal survey may be commenced at the fourth annual survey and be progressed during the succeeding year with a view to completion by the fifth anniversary date. As part of the preparation of the renewal survey, the survey program should be dealt with in advance of the renewal survey. The thickness measurement should not be held before the fourth annual survey.

22   Asurveyindry-dockshouldbeapartoftherenewalsurvey.Thereshouldbeaminimumoftwoinspectionsoftheoutsideoftheship'sbottomduringthe5-yearperiodofthecertificate.Inallcases,themaximumintervalbetweenbottominspectionsshouldnotexceed36months.Forships

of15yearsofageandover,inspectionoftheoutsideoftheship'sbottomshouldbecarriedoutwiththeshipindrydock.Forshipsoflessthan15yearsofage,alternateinspectionsoftheship'sbottomnotconductedinconjunctionwiththerenewalsurveyduringtheperiodicalsurveymaybe carriedoutwiththeshipafloat.Inspectionoftheshipafloatshouldonlybecarriedoutwhentheconditionsaresatisfactoryandtheproperequipmentandsuitablyqualifiedstaffsareavailable.

Ifasurveyindry-dockisnotcompletedinconjunctionwiththerenewalsurveyduringperiodicalsurveyorifthe36monthmaximumintervalreferredtoinisnotcompliedwith,theCargoShipSafetyConstructionCertificateshouldceasetobevaliduntilasurveyindry-dockiscompleted.

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
E-mail: info@grclass.com Web: www.grclass.com

Form CCC-02	                                                                                 Approved by: GM	 	Page 7 of 7