# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

REPORT OF CARGO SHIP SAFETY COSNTRUCTION SURVEYS ANNEX IV - CHEMICAL TANKERS

| SHIP NAME: | IMO No.: | CALL SIGN: | FLAG: | CLASS No. |
| --- | --- | --- | --- | --- |

III. CHEMICAL TANKERS

1. ANNUAL SURVEYS

- Please, complete section 1(A, B, C) of form CCC-01 for general cargo

- Confirming, when appropriate, that the requisite arrangements to regain steering capability in the event of the prescribed single failure are being maintained (SOLAS 74/88 reg.II-1/29);

- Examining, as far as practicable, the cargo, bilge ballast and stripping pump for undue gland seal leakage, verification of proper operation of electrical and mechanical remote operating and shutdown devices and operation of cargo pump room bilge system, and checking that pump foundations are intact;

- Confirming that the pump room ventilation system is operational, ducting intact, dampers are operational and screens clean;

- Verifying that installed pressure gauges on cargo discharge lines and level indicator systems are Operational;

- Examining access to bow arrangement (SOLAS 74/00 reg.II-1/3-3)

- Emergency towing arrangement (where applicable) has been examined and found to be approved by ship’s flag and are in accordance to MSC. 35(63) (SOLAS 74/06, Reg. II-1/3.4)

- Emergency towing arrangement procedures (Cargo ships constructed before 2010-01-01, not later than 2012-01-01, Cargo ships constructed on or after 2010-01-01) have been found on board and they include:

  - ; Drawings of fore and aft deck showing possible emergency towing arrangement (MSC.1/Circ.1255)

  - ; Inventory of the equipment on board that can be used for emergency situation (MSC.1/Circ.1255)

  - ; Means and method of communication (MSC.1/Circ.1255)

  - ; Sample procedures to facilitate the preparation for and conducting of emergency towing operation

- After a satisfactory survey, the Cargo Ship Safety Construction Certificate should be endorsed;

- If a survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

2. (INTERMEDIATE SURVEYS ON CHEMICAL TANKERS)

.1 Please, complete section 1(A, B, C & D) for general cargo

- Please, complete section 2 for general cargo

- Additionally please, verify the following:

- Testing the insulation resistance of electrical circuits in dangerous zones such as cargo pump rooms and  areas adjacent to cargo tanks, but in cases where a proper record of testing is maintained, consideration should be given to accepting recent readings.

- For ships over 10 years of age, an internal examination of selected cargo spaces; Use SS-001/SS-002 to inform HO which spaces were seen.

- After a satisfactory survey, the Cargo Ship Safety Construction Certificate is endorsed;

- If the survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

- If the inspection of the outside of the ship’s bottom is carried out in conjunction with Intermediate survey please, additionally complete section 4 of form CCC-01 for general cargo and Section 4 of this report

3. (RENEWAL SURVEYS ON CHEMICAL TANKERS)

- Please, complete section 1(A, B, C & D) for general cargo

- Please, complete section 3 for general cargo

# Place and Date of Survey

GR CLASS Surveyor Name – ID No.