# SECTION 1

## General

  - Surveyors carrying out surveys of ship structure and shipboard engineering systems with a view to issueCargo Ship Safety Construction (CCC) E-certificates are to be familiar with the applicable provisions of the International Convention for the Safety of Life at Sea (SOLAS), as amended and applicable for the vesselunder survey.

An Initial survey should be held before the ship is put in service, and should include a complete inspection, with tests, when necessary, of the structure, machinery, and equipment to ensure that the requirements relevant to the Safety Construction E-certificate are complied with and are fit for the service for which the ship is intended. Reference should be taken from HSSC requirements as applicable and the checklist form ICCC should be used.

  - CCC E-certificates are generally to be issued to classed vessels. If the vessel is not classed at all or it is classed by another society, additional survey requirements may apply and GR CLASS Head Office instructions should be sought in such cases. It should also be noted that some flag states, such as Panama, require some vessels under their Administration to be classed by a Classification Society duly recognized by them. If a vessel is classed by another organization, it should be ensured that the class e-certificate is valid and properly endorsed and that there are no outstanding surveys or recommendations. Safety Construction Surveys should be carried out in tandem with classification and load line surveys as far as practicable.

Note: Vessels under Panama flag that are required to be classed by a Panama Recognized Organization or Class Society but, for any reason, do not comply with this requirement have to be notified to GR CLASS in order to inform the flag state.

  - When requested to carry out CCC surveys, the existing e-certificate on board should be sighted to establish the category of survey due, the Transfer of Class and Certification procedure (PM-TD12) should be followed. Renewal surveys are required every five years and before the e-certificate in force has expired. Four annual endorsements are required, three of them are annual surveys and one of them an intermediate one, as per the Harmonized System of Survey and Certification (HSSC). Annual surveys are to be held within three months before and three months after the anniversary date. The intermediate survey should be held within three months before or after the second OR the third anniversary date of the CCC E-certificate. Under the HSSC, at least two (2) inspections of the outside of the ship´s bottom during any five-year period (validity of CCC E-certificate) are required. One of those inspections should be carried out on or after the fourth annual survey in conjunction with the renewal of the CCC E-certificate. In all cases the interval between any of two such inspections should not exceed thirty-six (36) months.

  - When carrying out surveys, the surveyors should ensure that the vessel complies fully with the applicable regulations including recent amendments such as Cargo securing manuals, Emergency

towing arrangements for tankers, enhanced survey procedures for tankers/ bulk carriers etc., safe access to tanker bows, tank venting arrangements, arrangements for oil piping systems etc. GR HeadOffice should be contacted for advice in case of doubt and before the issue/endorsement of necessary e- certificates.

# SECTION 2

# Initial Surveys (First Survey by GR Class Classified For Standard  (GR CLASS)

  - Previous certificates issued by other Classification Society or Recognized Organization (RO) duly authorized by the same Flag State.

    - A copy of the existing Safety Construction E-certificate should be forwarded to GR Head Office following the PM-TD12. Normally, GR Head Office would advise the attending surveyor to maintain the existing survey cycle. Any due or overdue survey to be carried out as per the existing cycle and any outstanding survey recommendations imposed by the previous society to be dealt with. The attending surveyor should verify during the agreed survey that no significant change in structure or arrangement has been made. In case of such changes being observed the GR CLASS Head Office should be informed immediately with a drawing or a sketch with all details showing the modification. It is be verified that the loading information (loading manual) and other necessary documents (cargo securing manual, documents required for ships subject to enhanced surveys such as tankers and bulk carriers, etc) are available onboard, in a language understood by the ships officers and approved by the administration or the previous society. The previous approval of the loading manual and cargo-securing manual may be accepted, and the manuals should be endorsed as appropriate – “This Manual has been accepted by on the basis of previous approval by (name of society)”. If manuals are not available on board, or the manuals on board are not approved, the Owners should be advised to submit the manuals for approval at the earliest. The e-certificate validityis to be restricted to maximum three months. Any service restriction imposed on the earlier e-certificate or by the previous society should be reflected in the new e-certificate without any change.

    - The date of previous inspection of the ships bottom is to be ascertained. Dates of previous dry-dockings indicated on previous Classification e-certificates or quarterly listings may be accepted. GR Head Officeshould be contacted for advice in other cases.

  - Previous E-certificates not issued by other classification society RO duly authorized by the same Flag State Administration or from other

    - PM-TD12 Transfer of Class and Certification must be followed. A copy of any previous certificate should be forwarded to GR CLASS Head Office. GR Head Office will advise whether the existing survey cycle can be followed and also whether any existing approval of manuals can be accepted. If

GR CLASS Head Office does not accept previous cycle, then a complete renewal survey should be carried out to ensure that the vessel complies with the applicable regulations. The necessary manuals (loading manual, cargo securing manual etc.) should be submitted to GR CLASS Head Office for approval along with a copy of any previous approvals. On completion of survey a CONDITIONAL E-certificate may be issued valid for three months with the approval of manuals remaining as an outstanding item. Any known service restriction should be reflected in the new e- certificate. Conditional E-certificates may only be issued provided that GR CLASS Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional e-certificate.

    - The date of previous inspection of the ships bottom is to be ascertained and GR CLASS Head Office advised. Depending upon when and where the dry-docking was carried out and under whose survey, GR Head Office will advise whether the earlier surveys can be accepted. When previous dry-dock survey records are not available or when previous surveys are not accepted by GR CLASS Head Office, a dry-docking survey should be carried out as part of the PM-TD12. Postponement of dry docking for bottom survey outside the water is approved by GR Head Office once the flag state has provided written authorization. Owner’s proposals /request for postponement of dry-docking surveys should be forwarded to GR CLASS Head Office. A e- certificate should not be issued in such cases until instructed by GR CLASS Head Office.

# SECTION 3

## Annual Surveys

  - The mandatory annual survey should include the following:

    - Verification that the statutory e-certificates onboard are valid, and all surveys are up to date (CCC, LL, CEC, CRC, IOPP and others as applicable). Verification that there is proof of current valid registration onboard.

    - Verification that no un-approved modifications have been carried out to the ship or equipment, which will affect the issue of CCC e-certificate.

    - Verification that the Classification e-certificate onboard is valid (if vessel is Classed) and all surveys (hull, docking, machinery, boilers, inert gas systems, etc) are up to date.

    - Verification that approved copy of loading/stability information is onboard. Verification that approved cargo-securing manual is onboard (if vessel carries any cargo other than bulk cargoes).

    - For ships subject to Enhanced Survey (bulk carriers, tankers etc) verification that the required ESP documentation is on board.

    - Verification that all log entries are being made as per SOLAS for steering gear drills, including emergency steering drills. Verification that the changeover procedures for steering gear are posted on the bridge and in the emergency steering position.

    - The condition of the hull and its closing devices should be examined as far as practicable. The examination should include weather decks, hatchways, air/vent/sounding pipes, casings, sky

lights, deckhouses and companionways, weather-tight doors, windows, side scuttles and sidelights, scuppers, sanitary discharges and valves, guard rails and bulwarks, freeing ports, gangways, walkways and lifelines, bow/stern/side doors and chutes and openings.

- The anchoring and mooring equipment is to be checked as far as possible.

- The watertight doors are to be carefully examined and operationally tested with local and remote controls as applicable. The condition of watertight bulkhead penetrations must be examined as far as practicable.

- The structural fire protection arrangements are to be examined to ensure that no changes have been made and the condition is satisfactory, (refer to the fire control plan for the structural fire protection details). Any manual and/or automatic fire doors are to be operationally tested to confirm that the doors close completely. Verify that holdback arrangements are not fitted (hooks/wedges etc. to keep doors in open position).

- Verify the operation of the loading instrument (if available) and ensure that the instrument and software are approved.

- Any mechanically operated cargo hatch covers are to be operationally tested to ensure weather- tightness.

- If the vessel is not classed, representative water ballast tanks are to be internally examined to establish satisfactory condition of the structure and protective coatings. At least one of each type of tank (peak tank, side/topside and double bottom tanks) must be internally examined. If protective coatings are in poor condition or not applied, or if the representative examination reveals defects, then all tanks have to be examined. If there is any doubt regarding integrity of the tank boundaries, a pressure test must bedone, by filling up the tank to the vent pipe and observing all the boundaries for leaks.

- If the vessel is not classed, representative cargo holds must be examined. If the survey reveals any deficiencies, then all cargo holds must be examined.

- If required by the attending surveyor, thickness measurements are to be carried out by an approved firm.

- Any special arrangements such as permanent fittings for timber deck cargoes or container-securing arrangements are to be examined as far as practicable.

- The machinery and boiler spaces are to be generally examined for possible fire and explosion hazards (excessive oil in bilges or under generators/purifiers, no protection of hot surfaces-exhaust pipes, no shielding of high-pressure oil pipes, oily rags etc). The emergency escape routes must be well marked, well-lit and free of any obstructions.

- All essential machinery to be generally examined and tested as far as practicable – auxiliary engines, compressors, pumps etc.

- The main and auxiliary steering gear arrangements are to be examined including the associated equipment and control systems. The steering is to be tried out from hard over to hard over including emergency steering. The movement of the rudderstock should be observed to ensure that there is no undue vibration or clearances. All steering gear alarms must be tried out. Ensure that the rudder angle indicator is working correctly. Means of communication between the navigating bridge, machinery control and alternative steering position is to be tried out.

- The bilge pumping systems are to be tried out, including bilge wells, extended spindles, pumps, and level alarms where fitted. The bilge pumping out should be tried out from various comparGMents suchas holds, fore peak/forecastle spaces, steering gear/after peak spaces, chain lockers etc. The emergency bilge suction arrangements are to be examined.

  - All boilers and pressure vessels are to be externally examined to ensure the satisfactory condition including the condition of all safety devices, foundations, controls, relieving gear, insulation, gauges andpiping.

  - All electrical equipment and cabling forming part of the main and emergency electrical installations are to be generally examined under operating conditions as far as practicable. All electrical installations in areas which may contain flammable gases or vapor and /or combustible dust must be examined and to ensure that they are maintained in good order.

  - Main and emergency sources of electrical power are to be operationally tested. If emergency source of electrical power is supplied by batteries, confirm the condition and that they are fully charged. All emergency lights should be verified as operational.

  - Any automation equipment (un-manned machinery space etc) must be examined and operationally tested to the extent possible.

  - The fire control plan should be examined to ensure that it is up-to-date and legible and that the duplicates are properly stowed in the watertight containers outside the accommodation.

  - All fire and/or smoke detectors and alarm systems are to be operationally tested as far as possible.

  - The fire main system is to be operationally tested with each pump, including the emergency fire pump, running separately and tested for adequate pressure and leaks. All fire hoses, nozzles, applicators and necessary tools must be verified as being available and in good condition.

  - Fixed fire fighting system (for engine room, boiler room, cargo pump rooms, cargo holds etc) records must be verified to ensure that system has been properly serviced. A general examination of system controls, piping, storage bottles/containers, pumps etc should be carried out to ensure that system is being maintained properly. Instructions and markings are to be verified.

  - A general examination of all portable and semi-portable fire extinguishers should be carried out to ensure that the extinguishers are in their respective positions as per the fire plan and that all extinguishers are fully charged and have valid service dates.

  - The remote controls for stopping fans and machinery and shutting off fuel supplies in machinery spaces(where fitted) and the remote controls for stopping fans in accommodation spaces are to be operationally tested.

  - The closing arrangements for ventilators, annular spaces, skylights, doorways and tunnel (where applicable) should be operationally tested. Fire dampers must be examined to ensure that they are closing properly and easily.

  - Firemen’s outfits to be examined (including the self contained breathing apparatus) to ensure that

they are complete in all respects and in satisfactory condition.

  - For oil tankers the following items are to be examined:

    - Cargo tank openings including gaskets, covers, coamings, PV valves and flame screens to be generally examined. Flame screens to vents on all bunker, oily ballast and oily slop tanks and void spaces are to be examined.

    - Cargo, crude oil washing, bunker, ballast and vent piping systems including vent masts and headers are to be generally examined.

    - Portable instrument for measuring the flammable vapor concentration must be available onboard including necessary spares and means of calibration.

    - Whether the vessel complies with SOLAS 1996 amendments regarding the tank venting arrangements and safe access to tanker bows.

  - Examination of the SOLAS requirements for structural fire protection, hull structure and essential machinery systems on board the ship, with tests as necessary to ensure that the ship’s structure,

structural fire protection and essential shipboard engineering systems remain in satisfactory condition.

  - The owner representative should confirm that no unapproved modifications have been carried out since the last survey.

  - All necessary documentation should be available on board (cargo securing manual, ESP documents for ships to which enhanced survey procedures apply, proof of compliance with recent regulations such as emergency towing arrangements, arrangements for venting, gas freeing, arrangements for fuel oil etc,access arrangements to tankers bows etc as applicable).

  - Electrical Plants must be surveyed and Form No. EL-01 must be filled and a copy submitted to GR Head Office.

# SECTION 4

## Intermediate Surveys

4.1  The intermediate survey should, in addition to the requirements of an annual survey, be a thorough examination of the SOLAS requirements for ship’s structure, structural fire protection and essential shipboard engineering systems.

# SECTION 5

## Renewal Surveys

  - The renewal survey should be a thorough and complete examination of the SOLAS requirements for

ship’s structure, structural fire protection and essential shipboard engineering systems.

# SECTION 6

## Safe Access to Tanker Bows

  - Tankers, including oil tankers as defined in SOLAS Regulation II-1/2.12, chemical tankers as defined in regulation VII/8.2 and gas carrier as defined in regulation VII/11.2, should be provided with means to enable the crew to gain safe access to the bow even in severe weather conditions. For tankers constructed on or after 1 July 1998, the access should be by means of either a walkway on the deck or apermanent construction gangway of substantial strength at or above the level of the superstructure deck or the first tier of a deckhouse, which should:

    - Be not less than1 m in width, situated on or as near as practicable to the center line of the ship and located so as not to hinder easy access across working area of the deck;

    - Be fitted at each side throughout its length with a foot-stop and guard rails supported by stanchions. Such rails should consist of no less than 3 courses, the lowest being not more than 230 mm and the uppermost being at least 1 m above the gangway, and no intermediate opening should be more than 380 mm in height. Stanchions should be at intervals of not more than 1.5 m;

    - Be constructed of fire resistant and non-slip material;

    - Have openings, with ladders where appropriate, to and from the deck. Openings should not be more than 40 m apart;

    - If the length of exposed deck to be traversed exceeds 70 m, have shelters of substantial

constructionset in way of the gangway or walkways at intervals not exceeding 45 m. Every such shelter should be capable of accommodating at least one person and be so constructed as to afford weather protection on the forward, port and starboard side; and

    - If obstructed by pipes or other fittings of permanent nature, be provided with means of passage over such obstruction.

    - Every tanker constructed on or after 01 January 2007 shall be provided with the means to enable the crew to gain safe access to the bow even in severe weather conditions. Such means of access shall be approved by the Administration based on the guidelines developed by the Organization (Refer to the Guidelines for safe access to tanker bows, adopted by the Maritime Safety Committee by resolution MSC.62(67)).

  - The Administration may accept alternative or modified arrangements for tankers with space constraintssuch as small tankers or tankers with a large freeboard, such as gas carriers, provided that suchalternative or modified arrangements achieve an equivalent level of safety for access to the bow.

  - Arrangements already approved by the Administration for Tankers constructed before 1st July 1998 maybe accepted, provided that such existing arrangements achieve an equivalent level of safety for access to the bow. This should be interpreted as follows:

    - An existing permanent gangway or a permanent deck level walkway having a minimum width of

0.6 m, will be acceptable provided deck access arrangements and shelters, in accordance with IMO guidelines are fitted.

    - Single wire rope lifelines are not acceptable.

    - Where no permanent gangway or walkway exists, arrangements complying with 6.2 above are to be provided.

    - Where the provision of a permanent gangway or walkway involves hot work on deck, attention should be given to the maintenance of tank coatings.

    - Provision of a permanent gangway or walkway within the forecastle area on flush deck vessels, occupied by mooring machinery, may be impractical due to possible interference with the normal operation of the deck machinery. The provision of suitable means to enable the crew to access and deploy the emergency towing arrangements within the forward area is the Owners responsibility.

# SECTION 7

## Venting, Purging, gas freeing and Ventilation

  - SOLAS Chapter II-2 has been amended for items concerning the arrangements for venting, purging, gas freeing and ventilation of cargo and ballast spaces on tankers. The amendments have been introduced to reduce the risk of damage to cargo tanks due to malfunction or mal-operation of the cargo tank venting system during cargo handling operations.

  - The amendments require that a secondary means of allowing full flow relief of vapor, air or inert gas mixtures is available at all times during cargo operations, or pressure sensors are fitted to each cargo tank to detect

over / under pressure conditions with an alarm facility. The ship is to have a combustible gas indicator with sufficient spares and means of calibration will be required.

  - The plans and details of proposed modifications to achieve compliance must be approved by GR CLASS Head Office before commencement of work. The modifications should be carried out under survey and tested on completion to the Surveyors satisfaction.

  - At first surveys by GR Class Classified For Standard  (GR CLASS), if the vessel does not comply with the above requirements and the vessel has already been dry-docked after 1st July 1998, a Safety Construction E-certificate cannot be issued. GR CLASS Head Office should be contacted for advice in such cases.

# SECTION 8

## Arrangements for Oil Fuel, Lubricating Oil and Other Flammable Oils

  - SOLAS Chapter II-2 has been amended for items, concerning fire protection of systems containing oils. The amendments have been introduced to further reduce the risk of ignition of oil spray in the event of leakage from oil fuel piping systems.

  - All external high pressure fuel delivery lines between high pressure fuel pumps and fuel injectors shall be protected with a jacketed piping system capable of containing fuel from a high pressure line failure.A jacketed system incorporates an outer pipe into which the high-pressure fuel pipe is placed forming a permanent assembly. The jacketed piping system shall include a means for collection of leakages and arrangements shall be provided for an alarm to be given of a fuel line failure.

  - All surfaces with temperature above 2200C, which may be impinged as a result of a fuel system failure, shall be properly insulated.

  - Oil fuel lines shall be screened or otherwise suitably protected to avoid as far as practicable oil spray or oil leakages onto hot surfaces, into machinery air intakes, or other sources of ignition. The number of joints in such piping systems shall be kept to a minimum.

  - A suitable enclosure on engines having an output of 375 kW or less having fuel injection pumps serving more than one injector may be used as an alternative to the jacketed piping system.

  - The amendments require all piping systems containing oil fuel, lubricating oil or other flammable oils to be generally examined and operated as far as practicable, with particular attention being paid to tightness, fire precaution arrangements, flexible hoses and sounding arrangements. MSC Circular 647 provides guidance on fire protection and prevention in machinery spaces, particularly with reference to shielding, insulation and pipe joints.

8.8 At classification and CCC surveys, the Surveyor should examine the piping systems with particular attention to details of screening or other methods used to prevent oil spray on to sources of ignition

or into machinery air intakes.

# SECTION 9

## Emergency Towing Arrangements for Tankers

  - As per SOLAS 1974, as amended, new and existing tankers of 20,000 tons deadweight and above shall be fitted with emergency towing arrangement, the design and construction of which shall be approved by the Administration. The term tanker, as per SOLAS 1974 Ch.I/2, is a cargo ship constructed or adapted for the carriage in bulk of liquid cargoes of an inflammable nature.

  - For tankers constructed prior to July 1st, 2002, the design and construction of emergency towing arrangements shall be approved by the Administration, based on the Guidelines developed by the Organization (Refer to MSC. 35 (63), as amended).

  - At first surveys by GR Class Classified For Standard  (GR CLASS), if the vessel does not comply with the above requirements, a Safety Construction E-certificate should not be issued. GR Head Office should be contacted for advice in such cases.

  - When new equipment/system is to be fitted, the plans and details of proposed system must be approved by GR Head Office before commencement of work. The modifications should be carried out under survey and tested on completion to the Surveyors satisfaction. GR CLASS Head Office will forward detailed instructions in such cases.

# SECTION 10

## Reporting

  - The relevant survey checklist (Form No. CCC-01) should be used when carrying out surveys and a copy submitted to GR CLASS Head Office along with survey reports Survey Statement (P24). When dry-docking/tail shaft surveys are carried out, survey checklist Form No. BS-01, and Form BS- 02 should be filled up and a copy submitted to GR Head Office.

  - Electrical Plants must be surveyed and Form No. EL-01 must be filled and a copy submitted to GR CLASS Head Office.

  - If the vessel to be surveyed is a Tanker then additional checklists Form No. OT-01 and Form No. IG- 01 must be filled and a copy submitted to GR CLASS Head Office.

  - At first surveys by GR CLASS, copies of the previous society’s e-certificates and copy of approval for loading manual/grain loading manual/Cargo securing manual/etc and list of any outstanding deficiencies, recommendations etc should also be forwarded. A copy of the previous societies e- certificates/statement indicating compliance with applicable SOLAS amendments (Emergency Towing Arrangements/ Safe Access to Bows / Tank Venting Arrangements / Oil Fuel Piping arrangements etc) should also be forwarded. Copy of previous dry-docking reports should also be submitted (when dry- docking/tail shaft surveys are not carried out by GR Class Classified For Standard  (GR CLASS).

  - On completion of initial, renewal or transfer of class and certification surveys an Interim Cargo Ship Safety Construction E-certificate should be issued, valid for 5 months. When survey is incomplete or deficiencies are noted, the INTERIM E-certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the INTERIM E-certificate and on the Survey Statement Report (P24). The e-certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two months (except where only approval of manuals remain outstanding, in which case the validity should be restricted to three months). Conditional E-certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional e-certificate.

A copy of the CONDITIONAL e-certificate is to be sent to GR CLASS Head Office.

  - On completion of other surveys (annual, intermediate and inspection of the ships bottom), the existing full-term e-certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR CLASS seal. A copy of the endorsed e-certificate must be submitted to GR CLASS Head Office. At annual surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the Statement report form P24. The Full Term CCC e-certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the Statement report P24.

  - A Statement report P24 should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owners proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama MerchanGMarine Circulars or Administrative Resolutions properly posted on www.segumar.com or

  - www.amp.gob.pa. For Barbados https://barbadosmaritime.org/documents/bulletins/. In case of conflict, the surveyor should follow the instruction issued by the flag GR Class is working on behalf unless there is an express authorization in written from the flag to act otherwise.

  - REFERENCE

10.10.2 IMO RESOLUTION A.756(18)

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021 | 06/2022 | https://panamashipregistry.com/circular- category/circulares/ |
|  | MMC 277: Fire Control Plan graphical symbols and Verification. | 06/2022 |  |
|  | MMC 204: Outside Ship’s Bottom Inspection and Dry docking Interface Periods for Panamanian Flagged Vessels. | 02/2022 |  |
|  | MMC 378: Announcement of Resolution entry in force January 1st 2020 for Amendments to the International Code for Fire Safety System (FSS Code) | 01/2020 |  |
|  | MMC 281: Guide for the Maintenance, Inspection of Fire-Protection System and Appliances. | 07/2022 |  |
| Barbados | NA | NA | https://barbadosmaritime.org/bulletins/ |
| Belize | MMN-18-001: Outside Ship ́s Bottom Inspection Requirements for Belize Registered Vessels. | 15/01/2018 | https://www.immarbe.com/circularslist/ |
| Sierra Leone | MC No. 13: Extension of Bottom Survey, Special Survey or Statutory Certificates Policy. | 10/01/2017 | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentation/i nstruments-and-laws/ |
| Honduras | NA | NA | https://merchanGMarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disposicio nes |
| Nigeria | NA | NA | https://nimasa.gov.ng/press-center/marine- notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |