# SECTION 1

## General

  - SOLAS does not, unless expressly provided otherwise, apply to:

    - Ships of war and troop ships

    - Cargo ships of less than 500 gross tons

    - Ships not propelled by mechanical means

    - Wooden ships of primitive build

    - Pleasure yachts not engaged in trade and

    - Fishing vessels.

  - The following may be used for guidance while considering the applicability of various regulations to theships:

    - SOLAS 1948 applies to ships built before 26 May 1965

    - SOLAS 1960 applies to ships built before 25 May 1980

    - SOLAS 1974 applies to ships built after 25 May 1980

    - SOLAS protocol 1978 applies to tankers built after 1 January 1981

    - The 1981 amendments apply to ships built after 1 September 1984

    - The 1983 amendments apply to ships built after 1 July 1986

    - SOLAS 1988 protocol entered into force on 3 February 2000.

Further amendments to the SOLAS 1974 and protocol 1988 have been adopted up to date by IMO and the Contracting Governments and may apply to new and existing ships according to their keel laying date and/or built date. It should, however, be borne in mind that some regulations of a later version of SOLAS may apply to existing ships. In case of doubt, GR Head Office should be consulted.

  - Safety equipment survey is arguably the most difficult survey on board a ship. The number of items to be covered is high and each item could be time consuming. There are large numbers of items, which areportable. Since the safety equipment is not used in routine operation there is a tendency from the ship staff to neglect this equipment

  - Safety equipment deficiencies have caused most port state detentions. The port state control inspectors inspect these items thoroughly and even a small item missing will be treated as a cause for detention. Hence the surveyor is advised to be highly cautious and thorough during survey. It is recommended that wherever possible two surveyors attend the surveys if the port stay of the vessel is limited.

  - Surveyors carrying out Cargo Ship Safety Equipment surveys are to familiar with the applicable provisions of SOLAS as amended and the 1972 Regulations for the Prevention of Collisions at Sea.

  - An initial survey should be held before the ship is put in service and these surveys should include a complete inspection, with tests, when necessary, of the equipment to ensure that the requirements relevant to the Safety Equipment Certificate are complied with and the ship is fit for the service for which is intended. Reference should be taken from the Harmonized System of

Survey and Certification (HSSC) requirements as applicable. The checklist form ICEC should be properly filled.

  - For ships in service the procedure for change of class and certification (PM-TD12) should be followed. Among others the existing certificate onboard should be sighted to establish the type of survey due.Renewal surveys are required (every 5 years as per the HSSC). In addition, four annual surveys are required and one of them must be a periodical survey as per the HSSC. The periodical survey should be held three months before or after the third or the second anniversary date.

  - Mandatory annual/periodical surveys should include a verification of that the safety equipment certificate on board is in order and an examination of all the various items of safetyequipment, together with tests as considered necessary, all to an extent to which, in the Surveyor’s judgment indicates adequate condition, given proper maintenance, until the next due survey. Inaddition, the Owner’s representative should confirm that no unapproved modifications have been carried out to the safety equipment since the last survey.

  - The renewal survey should be a thorough and complete examination with tests where appropriate. On completion, the safety equipment should, given proper maintenance, be expected to remain in good condition and working order till the next renewal survey.

On completion of a renewal or initial survey, an INTERIM certificate may be issued valid for 5 months along with a Form E (Record of Equipment). GR Head Office will issue the full term certificates after review of the survey reports. If some items of a minor nature remain outstanding for completion of a renewal survey because the required services or replacement equipment are not available locally, a CONDITIONAL certificate should be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two months. The outstanding recommendations are to be endorsed on the reverse of any CONDITIONAL certificate issued and on the Survey Statement report P24, copies are to be sent to GR Head Office. GR Head Office permission should be obtained before issuing of the certificates. Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, HeadOffice should notify this fact to the flag state that authorized the conditional certificate.

Annual servicing of liferafts and fire extinguishers/fire extinguishing systems, renewal of pyrotechnics and the provision of incidental items of equipment not affecting the overall efficiency of the lifesavingor fire appliances may be treated as minor items, unless the cumulative effect of the deficiencies present an unacceptable risk. However, where the efficiency of the life- saving or fire appliances are seriously impaired due to major deficiencies or defects, e.g. unserviceable or damaged lifeboats or davits, inoperable emergency fire pump, inability to provide the required jets of water for fire fighting purposes due to defective fire pumps, fire main or hoses, etc. a certificate should not be issued. The matter should be reported to GR Head Office immediately  and  instructions  awaited.  Where  such  deficiencies  are  noticed  during

annual/periodical surveys, the full-term certificate must bewithdrawn and the Owners requested to correct the deficiencies immediately.

  - The various items of safety equipment to be surveyed are itemized on the Record of Approved Cargo Ship Safety Equipment Form (Form No. CEC-02) held on board the vessel. During first (initial surveys by GR Class-Certified For Standard (GR CLASS) and/or upon transfer of class and certification, it should be ensured that the vessel has the appropriate Record issued by the previous society, which issued the safety equipment certificates. If a copy is not available, or if the existing record does not contain all relevant information as given in Form No. CEC-02, a new Form No.CEC- 02 should be completed, a copy placed on board and a copy submitted to GR Head Office. When previous record is available a copy must be submitted to GR Head Office. Any repairs/renewals must be reflected in the Record of equipment. The survey checklist Form No. CEC-01 must be filled up during all surveys and a copy submitted to GR Head Office along with reports.

  - Where new installations of fire extinguishing or other systems are to be installed or major modifications are carried out, plans are to be submitted before work is commenced. In case of any modifications or where renewal or testing of equipment is involved, it should be ensured that the replacement/repaired equipment meets the relevant requirements as per regulations (in some cases the replacement equipment must meet current requirements even though the replaced equipment was not meeting the current requirements). In cases of doubt, GR Head Office should be contacted for advice.

  - Plans and Documents: All essential documentation onboard must be in a language, which is understoodby the crew of the ship and in should be legible and well maintained. Illegible, faded, stained or torn instructions/plans etc. must be replaced. It should be borne in mind that operating instructions for lifesaving and firefighting equipment are normally required for use in an emergency and therefore theyshould be posted in locations where emergency lighting is also available – e.g. CO2 system instructions

  - All ships should have an updated fire control plan posted at suitable locations including one copy stowed in a weather-tight container on each side of accommodation. The plan should be legible, up-to- date and all symbols should be according to IMO guidelines. During surveys it should be ensured that allitems are at their respective locations as per the fire control plan onboard. Any modifications (such as installation of new systems for paint lockers etc, change of equipment - fire pump etc) should be updated on the fire plans.

  - There should be certificates of approval or other confirmation that the following equipment and materials have been approved to IMO or other recognized standards. The non GR CLASS approvals should be from a flag administration or from IACS member societies. Approval certificates should be available onboard for lifeboats/rescue boats, liferafts, davits, davit winches, pilot ladder and hoist, accommodation ladder, lifebuoys, lifejackets with lights and whistles, immersion suits, thermal protective aids, pyrotechnics, retro-reflective material for life-saving equipment, navigation lights and sound signals (as per COLREGS 72) and for individual items of navigational equipment in accordance with relevant IMO performance standards.

  - During surveys, it should be ascertained from the master that all equipment is available onboard as per the record of equipment. If any equipment has been added or deleted, the record of equipment should be amended under cover of office stamp. Certificates of any replacement equipment should be verified before accepting the equipment. Any replacement or new life saving appliance should meet all current SOLAS requirements irrespective of age of ship (for example if a lifeboat is being fitted on a tanker it has to be enclosed type).

  - SOLAS training manual should be available on board. There should also be instructions available on board for operation and maintenance of all survival craft, fire-fighting equipment, personal protection equipment etc. Symbols should be posted throughout the vessel in accordance with IMO guidelines.

  - Operating instructions for survival craft, fire extinguishing equipment etc should be posted at required locations and in languages understood by crew. The instructions must be posted in areas adjacent to the equipment and in well-lit areas so that they can be read under main and emergency lights.

  - Muster lists and safety instructions should be posted at required locations (in language understood by crew).

  - Review of records and interview of Master: The survey should start with an interview of the Master.The following questions should be asked:

    - Has any unapproved equipment been fitted since the last survey?

    - Was there any incident of fire onboard since the last survey?

    - Was the vessel subject to any port state control detention/boarding since last survey?

    - Was the vessel involved in any accident/collision/grounding/contact or problem involving the crew? If the answer to any of the above is yes, the details including logbook extracts should be noted andreported.

  - Review of documentation & records: The following records should be verified:

    - Validity of all statutory certificates on board including required annual/periodical survey endorsements. If any certificate is invalid or surveys are overdue, the Master must be advised, and owner’s proposals should be obtained and GR Head Office informed.

    - Certificate of Registration. If the certificate of registry has expired, statutory certificates must not be issued or endorsed.

    - Record of safety equipment onboard (any change should be endorsed by the surveyor)

    - Service certificates for fixed fire extinguishing systems include contents check for cylinders, service report, foam sample analysis report, servicing/hydraulic testing/refilling certificates for portable and semi-portable fire extinguishers and SCBA bottles and equipment. The certificates must be from approved/recognized organizations (please refer to the procedures for approval of service providers)

    - Service reports for liferafts and hydrostatic release units. The certificates must be from approved/recognized organizations (please refer to the procedures for approval of service providers)

    - Certificates for lifeboat falls (if rope falls have been renewed since last survey)

    - Records of renewal/reversal of lifeboat falls (log book entries)

    - Certificates for any new equipment supplied

    - Logbook entries are to be verified for all drills (fire and boat drills, dates of lifeboats lowering/ maneuvering in water, lowering of rescue boat, abandon ship drills etc) and dates noted on survey checklist. If drills and inspections are not being regularly carried out, Master to be advised regarding therequirements of applicable SOLAS regulations as amended. (Regulations 18, 19 of Chapter III)

    - Any exemption /waiver granted by/or on behalf of the administration should be reflected in the certificate issued and a copy of the exemption/waiver is to be attached to any certificate issued.

    - If minimum safe manning certificate is for short international voyages the same restriction should be imposed.

  - The Surveyors should ensure that the total number of persons provided for by lifesaving appliances must never be less than the actual number of persons carried on the ship or the minimum number as per the manning certificate. In general, the number is to the minimum of the following: -

    - capacity of the port lifeboat

    - capacity of the starboard lifeboat

    - total capacity of the liferafts (excluding any additional liferaft forward or aft) – in the case of shipswithout lifeboats, the capacity of the liferafts on each side

    - total number of lifejackets (excluding lifejackets for use at remote stations) and

    - total number of immersion suits and thermal protective aids (excluding thermal protective aidsprovided as part of equipment of lifeboats, rescue boats and liferafts)

  - Safety equipment can be divided into three main areas - safety of navigation, life saving appliances and fire detection and extinction.

# SECTION 2

## Survey of Navigational Equipment

  - The amount and type of navigation equipment to be fitted is dependent on the gross tonnage, datekeel laid and types of the vessel. When equipment is to be demonstrated for survey purposes, only a duly authorized Owner’s representative must operate it. If any additional or replacement equipment is installed, it should of a type approved by the national authorities. If it is not possible for the surveyor to verify that all navigational equipment is in good working condition, a written confirmation should be obtained from the Master that all navigational equipment and electronic aids remain in good working condition.

  - Magnetic Compass: In most cases there is no steering magnetic compass, and the Standard Magnetic Compass is positioned on top of the wheelhouse, which can be viewed from the navigational control position via a reflector. The communication between the two positions is often by means of a voice pipe although a two-way radio could be used. Check that deviation chart is posted; the liquid level is satisfactory, no air bubbles, azimuth for taking bearing. Deviation chart to be not more than 12 months old, voice pipe from wheelhouse top, spare compass, or card, covers for exposed compass.

  - Day Light (Aldis) Signaling Lamp: Test using both main and emergency supply.

  - Echo-sounder, Radio Direction Finder: Check functioning, check records of reading if time is a constraint. Master's confirmation may be accepted for proper functioning

  - Gyrocompass: Check functioning, error against magnetic compass and error between master & repeaters

  - Heading information at emergency steering position: For ships built after Sept 1984 visual compass readings are to be available at emergency steering gear position. For ships built before this date voice communication may be accepted.

  - Navigational Lights: The generally accepted arrangement is to fit duplex type lights in the five steaming light positions and single type anchor and NUC lights. All navigation lights should have test certificatesas per COLREG 72. Navigation lights are to be tested on both main & emergency power. Attention should be paid to the electrical connections and any audible and visual warnings of lamp failure, locatedon the bridge, are to be tested for operational efficiency. There must be a complete set of spare bulbs (correct approved type) available onboard.

  - Auto Pilot: Verify that instructions for operation and emergency changeover is posted, check off course & power failure alarm functioning. Logbook entries may be consulted to ensure that equipment was working satisfactorily.

  - Radio Life Saving Appliances: These will normally be surveyed at the same time as the main radio installation. However, these are also safety equipment items, and it should be verified that the required equipment is available. Radar transponders and VHF radiotelephones have replaced the portable lifeboat radio, handheld VHF EPIRBS, and two-way radiotelephones, which were required earlier. Two- way VHF radiotelephones can be easily tested but testing of SART should preferably be carried out by atrained officer or a radio engineer (so that distress messages are not sent inadvertently).

  - RPM/Propeller Pitch Indicators: Master's statement may be accepted.

  - Rudder Angle Indicator: Check functioning; check that it is independent of steering control.

  - Navigational Charts and Publications: Check for availability of all required charts and publications for trading area intended. Evidence of updating of charts should be sighted (e.g., Maintenance of a chart correction logbook or check for updating from Notices to Mariners etc). Master should also be requested to produce the following: - weekly notices to mariners (not more than 6-week-old), sailing directions (entitled “Pilots”)), list of lights and fog signals, list of radio signals, current tide tables, nautical almanac of the year, international code of signals etc.

  - Radars: Check that the radar is compatible for SARTS (9 GHz) and it is fully functional within 4 minutesof switching on, day light functioning. Check that maker's manual is onboard.

  - Ship's Whistle: Check the operation. If only one electric whistle is provided, it should be tested on

both main and emergency supply.

  - Ship's Bell, Gong, and Navigational shapes: Check available on board (gong required only for ship's over 100m in length) no damage to them.

  - Distress signals and rockets: Pyrotechnic signals on board and in lifeboats, including rockets and cartridges are to be examined and should be replaced on or before the date of expiry (or within 3 years of date of manufacture if date of expiry is not specified). Pyrotechnics, which have deteriorated due to damp or mechanical damage, are to be renewed immediately. Pyrotechnics should be packed in watertight containers and stored in a convenient dry location.

  - Emergency lighting: Emergency and main lighting should be checked in embarkation areas, passageways, engine room, pump room, emergency exits and muster stations. Emergency escape routes are clearly marked with lights or fluorescent stickers.

  - Alarms: The fire & general alarms are to be tested and it should be audible at all locations where crew normally works (engine room, steering room, mess room, galleys, passageway, forward store etc)

## GMDSS Performance Standards

    - Ship earth station which forms part of the GMDSS, if designed to operate in a mobile satellite service recognized on or after 1 January 2021, complies with the relevant requirements of A.1001(25) and conforms to performance standards MSC.434(98). Surveyors to verify.

# SECTION 3

## Survey of Life Saving Appliances

  - Life Boats

    - Surveyors should ensure that non-approved replacement boats have not been placed on board (by sighting certificates or checking boat details against record on board). Lifeboats are perhaps the most important life saving appliance on board. A thorough visual examination as described below supplemented by tests wherever necessary should be carried out on lifeboats.

      - All lifeboats and rescue boats are to be launched or lowered to the water. Where lowering to the wateris not possible due to port/weather restrictions, the boats should at least be lowered to the embarkation deck.

      - Hull of the lifeboat should be examined for corrosion, wear & tear (especially in exposed boats) damage(in fiber glass boats) damage to buoyancy chambers inside the hull, condition of paint. Special attention to be paid in way of bearing surfaces or areas where contact with chocks could damage the structure. Repairs to GRP lifeboats should only be carried out by experienced workshops and GR Head Office should be contacted when any major repairs are carried out. When lifeboats are damaged and cannot be satisfactorily repaired before the ship sails, alternative arrangements should be discussed with GR Head Office.

      - Buoyancy arrangements: Portable air cases, if fitted, should be removed, carefully examined and tested. Total immersion in water will reveal any deficiencies. The structure in way of air cases to be

carefully examined and care taken to ensure that cases are correctly re-fitted. Where built-in buoyancy tanks are fitted, an air and soapy water test can verify the buoyancy.

      - Fittings: Check whether lifeline is properly becketed outside the hull and that grab rail/handrail are in good condition and properly fixed.

      - Lifting Hooks & Connection: Check for wastage where they pass through the deck and in way of keel attachment. The keel attachment should also be verified as being efficient.

      - Fire Extinguishers: Check service record and whether properly stowed and marked

      - Retro Reflective Material must be properly seen on boat side, bottom & top. Examine for wear & tearand fading.

      - Marking: Ship's name, port of registry & capacity must be marked clearly on bow & stern and must bevisible from top also for boats installed in ships built after 1986 & new installations.

      - Lifeboat Pyrotechnics: 6 Hand flares (no hand flares in tankers), 4 Parachute flares & 2 smoke signals.Check for Validity.

      - Ration (Food): Validity (as stated on container) or 5 years from the date of manufacture. Check that thepacking is not damaged.

      - Water: 3 litres per person. Validity 3 years, if not stated on the container.

      - Other life boat equipment: At random, check items from the record, if any thing is found missing go through the entire list. Mast and sails and exposure covers should be rigged, the sea anchor and line run out and all lines and ropes checked. The rudder and tiller should be shipped and operated, rowlocks checked for freedom and rotation, hand-propelling gear operated and bilge pumps should be tested.

      - Davits, Block & Gear: A thorough check of davits, the supporting structure, wires and running tackle, release mechanisms and winches should be carried out for evidence of damage, significant corrosion or lack of adequate maintenance of moving parts and tackle. Ends of wire rope falls should not be securedby wire rope grips as primary terminations Wire rope falls are to be reversed every 30 months and renewed every five years. When rope falls are renewed certificates must be reviewed to ensure that correct size and type (strength) of wire rope is being used. All rollers and guides, pivot pins and axles should be checked for free running, slackness due to wear, corrosion wear & tear, and misuse of paint. Functioning of all limit switches should be verified and lifeboat- hoisting arrangements are to be tried out.

      - Davit: Check for wastage (especially lower portion), damage, paint condition and attachment to deck(bolts).

      - Rope (embarkation) Ladders & Connection: Check overall condition and connection to deck structurefor wear & tear and corrosion.

      - Lifelines on Davit Spans: Check the condition of ropes (not required for totally enclosed self- rightingboats with direct boarding).

      - Skates or Fenders: These prevent damage to boat during launching, check that these are in goodcondition.

      - Painting: Check that the inside of the boat is painted in highly visible orange and outside either white ororange

      - Rudder & Tiller: Check that these are not damaged and can be moved easily

      - Safety Belts: In partially/totally enclosed boats to be checked for wear & tear.

      - Rigid covers/hatches, closing arrangements: In partially/totally enclosed boats - to be checked forcondition & proper closing.

      - Water Spray System in Fire Protected Boat: Check sea intake, nozzles, piping, self-priming pumps for function. Ensure that spray heads are clean. Ensure that there is no oil film on canopy, which will

disrupt even flow of water on the surface. The system should be flushed with fresh water following a test with seawater.

      - Air support system: Air cylinders should be examined for capacity. The system should be examined for security of stowage.

      - Weather tight storage of lifeboat equipment: Check for damage.

    - Testing of Lifeboats:

      - Release of Lashing: Check the operation, any wear & tear or corrosion of the lashing gear.

      - Turning out and lowering to Embarkation: Moving parts should be released easily and all movementsshould be smooth.

      - Lowering to Water: Check for smooth & fast operation.

      - Winch Brake: During lowering check that the boat is held in position using brakes alone without anyslip.

      - Engine Started: Check the easiness of starting and running in condition for at least 3 minutes. Examinefor oil/fuel leaks, exhaust gas leaks (enclosed lifeboats), contents of oil fuel tank.

      - Release mechanism (Remote & Local): Check both the release mechanisms on load and normal. Check mechanism to prevent accidental release. All safety precautions to be taken to ensure that accidental on- load release do not occur.

      - Control of Discharges in way of life boat: Check that all over board discharges (if any) in way of Lifeboat embarkation & lowering can be stopped remotely from the embarkation area. Stabilizer fins operation (if any in way of lifeboat locations) to be checked.

      - Lighting: Check that all the lighting at the lifeboat station including floodlights and emergency lights etc.works both on main & emergency power. Where electrical system is provided for lifeboats, check condition and charge of batteries, charging arrangements including ships supply, internal and external lights and search lights operation.

      - Running of Propeller in Ahead & Astern: Check the engaging and condition of Gear/Clutch and propelleraction. The steering arrangement is to be tested hard over to port and starboard.

      - Boat Connected to fall and Hoisted: Check for smooth operation (Observe prime mover of winch if power operated). Brakes may be checked again. Check the function of limit switch at upper most point (if fitted)

      - During initial survey - Depending on the condition of boat & davit & supporting documents available, the following tests may be carried out: -

      - Load test of boat, davit.

      - Opening up of lifeboat engine and inspection of fuel oil tanks.

      - Lowering test (speed) of Full load & Empty Boat: At full load 0.4-(0.02XH) m/s minimum speed highdistance to waterline from davit.

      - Maneuvering of Boat in Water may be necessary.

## Rescue Boat

    - If no approval certificate is available, the speed of the boat is to be checked (minimum 6 knots). The rescue boat equipment is to be verified. If a lifeboat is converted or adopted as a rescue boat, check that the rescue boat requirement of placing a stretcher etc are fulfilled. Check the recovery speed of rescue boat using davit power winch – minimum 0.3 m/s required in the loaded condition.

The rescue boats quick release device should be thoroughly examined and tested for smooth operation. Check thatthe boat can be lowered to one side at adverse condition of heel up to 20 degrees. Check the hull(inside & outside) for damage. Ensure that all required equipment are available and in good condition.

    - Inflatable boats should be examined to ensure that there has been no slippage of the seams and the material of the collars is in good condition and has not started to crack and perish. Surveyors should ensure that air pressure is being maintained in accordance with the maker’s instructions. Self-draining flaps should be checked for operation and engine mountings and the attachment of lifting strops shouldbe carefully examined. All repairs to inflatable rescue boats should be carried out at an approvedservice station only. Check the inflating arrangement in inflatable boats; check the starting & running (3 minutes) of the engine. Check the lowering & braking function. Check the marking of ships name & port of registry, fitting of retro-reflective tapes, check that towing arrangement is satisfactory. Check weather tight compartments for storing equipment.

  - Life Rafts

    - Rigid Life Rafts: These are rare nowadays. Check condition of rafts, accessories for completeness as per checklist. Liferaft equipment should be checked for completeness as per checklist. Check validity of water, food, first aid equipment and pyrotechnics. Raft markings should be checked (Name, port, capacity, maker's name, serial number, name of approving authority, SOLAS, type of emergency pack inside, length of painter, maximum stowage height and launching instruction). Check that the lamps on top of canopy and inside functions properly.

    - Inflatable Raft: Check maker's/Service firm certificate, marking and stowage (annual servicing). The servicing must be carried out at a station, which is approved by the liferafts manufacturer or the national authority of the country or by GR Head Office. Where required, ensure that the liferafts canbe transferred from side to side at single deck level. Check that stowage height above lowest water level does not exceed the permitted height.

    - Float Free Arrangement: Check that the life Raft is not tied down to prevent movement. The inboard end of the painter should be secured to the ship by a weak link. The painter itself should not be directly fastened to the ship. Check that hydrostatic releases are serviced (annually) unless they are of the disposable type. It should be ensured that no overhead awning or other obstruction has been introduced which would prevent the raft from floating free.

    - Marking: Check inspection sticker for maker's name, serial no., approving authority, capacity, SOLAS, type of emergency pack, last service date, maximum height of stowage & launching instructions.

    - Life Raft Davit: For davit-launched liferafts, check davit function of turning out lowering, prevention of premature release and release on being waterborne.

    - Additional Forward/Aft Life Raft: This liferaft is required only if the distance from the nearest survival craft to the fore or aft end exceeds 100 m. Check stowage and readiness for use and check availabilityof embarkation ladder and posted instructions. Hydrostatic release unit is not required for the additional forward/aft liferafts.

  - Life Buoys: Check the validity of smoke signals (two) and function of light signals (if inverting type or if there is a “press to test” facility). If water activated lights are fitted, expiry date should be checked. Check that lifebuoys on bridge wings are connected to smoke & light signals and arranged for quick release (weight of these to be minimum 4.5kg). Check that the lifebuoys with light or light & smoke signals are not fitted with Lifelines. Check that at least two other life buoys are fitted with lifeline. Checkall the life buoys have ship's name and port of registry marked on them and are of bright orange color and with retro-reflective tapes. Check the condition of grab lines and lifelines. Check condition of the buoys (including water resistance) and stowage.

  - Life Jackets: Lifejacket material and ties should remain sound and not tear easily. There should be adequate number of spare lifejackets, which are stowed at different locations for people on watch or atremote stations (e.g., in forward area, wheelhouse and in engine room). Check stowage in clearly marked space & easily retrievable in each cabin. Check number as per record (additional for watch duty & remote locations – immersion suits meeting the requirements of lifejackets may be included in the total). Check at random for condition & fittings (whistle & light) and marking (ship's name).

  - Pyrotechnics: 12 Parachute flares, check for validity - if nothing mentioned on cover, 3 years from date of manufacture. If manufacturing or expiry dates are not printed, the flares to be replaced. One line- throwing appliance with four rockets and lines should be available. Check whether of approved type & validity date marked inside the container.

  - Immersion suits & Thermal Protective Aids: There shall be provided for every person assigned to crew the rescue boat or assigned to the marine evacuation system party. If the ship is constantly engaged in warm climates, where, in the opinion of the Administration thermal protection is unnecessary, this protective clothing need not be carried. For cargo ships, an immersion suit of an appropriate size, complying with the requirements of section 2.3 of the LSA Code shall be provided for every person onboard the ship. If a ship has any watch or workstations which are located remotely from the place or places where immersion suits are normally stowed, including remotely located survival crafts, additional immersion suits shall be provided at these locations for the number of persons normallyon watch or working at these locations at any time. Check condition and number & proper stowage. If not worn with life jackets should have lights & whistle. Some of the modern types of immersion suits are also approved as lifejackets. In this case, their number can be included in the total for lifejackets. The administration can waive the requirements for Immersion suits and thermal protective aids for vessels engaged in voyages in tropical regions only (between 320 North and 320 S latitudes). However, PANAMA and BELIZE maritime administration require that an exemption be obtained from the Flag sate in such cases. GR Head Office must be contacted for instructions if the vessel does not have immersion suits and thermalprotective aids.

  - Retro-reflective material: All lifesaving appliances should be fitted with retro-reflective material (lifeboats, life rafts, lifebuoys, lifejackets, and immersion suits and thermal protective aids)

  - Pilot embarkation: Unlike LSA items, pilot embarkation arrangements will be in constant use and thus particularly subject to wear and tear. Check the storage of pilot ladder. The ladder should be rigged to ensure that arrangements fully comply with the regulations and that safe access to and from the ladder to the deck is achieved. Wooden steps and spreaders of the ladder should be examined for rot or mechanical damage and ropes for rot or chaffing. Gates in bulwarks or guardrails should be verified efficient and steps to bulwarks, platforms etc., secure. Mechanical hoists should be rigged and tested. Accommodation ladders, which form part of the pilot boarding arrangements, are surveyable items andshould be verified as being in efficient condition. Hidden parts of structure such as central pivot under the landing platform should be given particular attention. Surveyors should also check to ensure that the lighting arrangements systems are adequate. The life buoy and safety line should be checked.

  - Muster and embarkation arrangements: Alleyways, stairways and exits to muster and embarkation stations must be properly lighted (main and emergency) and free of any obstructions. Symbols are to beposted in prominent well-lit positions. Embarkation ladders and lifelines, where applicable, are to be run out to ensure that they are of sufficient length and effectively secured.

# SECTION 4

## Survey of Fire Fighting Equipment

  - Most of the casualties on board recorded so far have been due to fire, particularly engine room fire. Investigations reveal that on many occasions fire protection & extinguishing equipment failed to function, or crew were unable to operate them properly. Hence Surveyor should pay special attention tothe functional readiness of equipment & crew's awareness. The firefighting equipment should be in readily accessible position, clearly marked and crew should know the position of each equipment &how to use them. If a serious fire risk is discovered, the Surveyor should make the Owner aware, so thatcorrective measures may be taken. (E.g., excessive oil spills on the engine room floor plates or bilges, accumulation of oily rags, oil seeping from machinery, drip trays overflowing etc. are all fire hazards). Inflammable materials like paint drums, oxygen/acetylene cylinders are to be stored in spaces designated for them and are not to be kept in engine rooms, accommodation spaces etc.

## Preventive Equipment

    - Quick closing valves on fuel oil and lube oil tanks: Visually examine the valves, remote operating wires & layout for slackness, wear & tear, If pneumatic check the lines. Check functions at random. In some old ships these valves may be only remote-operating types with spindles and not quick closing. Check function.

    - Remote Stopping of Fuel oil pumps and ventilation fans in engine room: Should be stoppable from a location outside engine room, in small ships on main deck and in large ships inside accommodation near engine room entrance. Check functioning and also engine crew's awareness.

    - Closing arrangement for skylight, Dampers in Ventilation ducts for Engine room/pump room: Check for seized handles for dampers, marking for operation and operation of sky light from inside and outside. Check for any corrosion or perished packing.

    - Fire doors: Check that no holdback hooks are fitted, except when magnetic hooks with release mechanism are fitted. If release mechanism is fitted check functioning of remote operation and indicators of position of fire doors at control station (wheelhouse). Check the proper closure of doors.

    - Safe stowage of cylinders containing acetylene and oxygen, when not in use: Surveyors should establish if there are any acetylene or oxygen cylinders (usually part of portable welding / cutting equipment) on board. Such cylinders should be provided with permanent stowage arrangements. If central storage facilities are provided above the uppermost continuous deck for oxygen and acetylene, the portable cylinders are to be stowed in this space, when not in use. When central storage facilities are not provided, the cylinders should be rigidly stored in either:

- A lockable well- ventilated room or cabinet, on or above the uppermost continuous deck, separated from other spaces by gas tight divisions and, if possible, with direct access to the open deck, or

- An open deck area on or above the uppermost continuous deck provided with storage racks and protected against mechanical damage and direct exposure to the sun, wind, and weather. A tight roof or cover of steel is to be fitted above the storage position and, to avoid interference by unauthorized persons, enclosed by either a solid wire grill or similar device with lockable access.

Any electrical device in the storage space should be of certified safe type for use in dangerous environments. Gas cylinders are to be stored in an upright position and securely fastened with arrangement that permit the rapid disconnection of the cylinders. A protective cover is to be screwed to the head of each cylinder, when not in use. Empty bottles are to be stored, as if they are full. Storage spaces are to be properly marked with warning signs indicating the storage of dangerous gases. Storage of oxygen and acetylene cylinders in machinery spaces is not permitted.

## LSA Code Fitting of Retro Reflective Materials on LSA

      - The "Revised Recommendation on the Use and Fitting of Retro-Reflective Materials on Life-Saving Appliances", in order to provide updated guidance on accelerated weathering tests for such materials. The previous Recommendation given by resolution A.658(16) will be revoked and replaced by resolution

MSC.481(102). Administrations may accept life-saving appliances which are already fitted with retro- reflective materials in accordance with resolution A.658(16).

## Fire Extinguishing Arrangements

    - Fire Pumps & Fire Main: Check whether fire pump was opened & examined towards class Survey. If not surveyed for class & there is no record of overhauling, one main fire dump should be opened up. Check functioning of the pumps and the pressure in the line. The fire line should be checked for leakage with the valves closed and at the working pressure. Check that a throw of around 12m from two jets (select two hydrants one forward and one aft which are farthest from the pumps) is achieved simultaneously. This should be achieved in addition to supplying water for other services for which fire pump may be used (for sprinkler systems for paint locker, for foam systems for tankers etc.). All valves in the fire line should operate freely. Test the relief valve on the fire main. (Should lift at 1.5 times the working pressure).

    - Emergency Fire Pump: Same as above. Check remote operation where fitted, check instructions for operation is posted near the pump. Check that pump is connected to emergency generator and can be started & operated with emergency power (if electrically driven). Ensure that there is adequate suction when ship in light condition. Special attention should be paid to emergency fire pump as often the crew neglects this pump. In case of portable pump check that they are able to develop enough pressure and arrangements are made for proper suction and discharge connections.

    - Fire Hoses, nozzles & coupling: Check that these are stored in boxes and properly marked and as per fire plan. Hoses must be selected-one each from engine room, accommodation and deck for examination and testing. If any of them is found to be unsatisfactory all the hoses should be examined. Verify that complete set is available at each location. The new fire hoses should be of non-perishable material. Check that there is sufficient number of dual-purpose (spray & jet) nozzles, especially in machinery spaces. Ensure that tools are available at each fire station as required. Ensure that the connectors at the hydrants and on the hoses are matching and interchangeable.

    - International Shore Connection: Check that the international shore connection flange with it's gasket, 4 Bolts & nuts and 8 washers is stored & marked properly and crew is aware of its location & procedure for use. The connection should be capable of being used on both port and starboard sides.

    - Portable Extinguishers: Check that the required number of portable foam, CO2 and dry powder extinguishers are stored at their designated locations and ready for use and stamped with the service label. At random check the weight by lifting, check that spare charges/refill are available on board. For portable foam applicators and semi-portable foam extinguishers, the foam concentrate/solution is to be replaced every 10 years. Portable extinguishers other than CO2 should be emptied and hydraulically tested at approximately 5-year intervals or earlier if the surveyor considers necessary. Fireextinguishers must be serviced/hydraulically tested or refilled by approved companies only (approved by IACS member classification societies or national authority of the country concerned or by GR Head Office.)

    - CO2/Halon fixed Installation: Check that there is no corrosion to cylinders & piping and that all the cylinders are connected properly to the gang release mechanism. Check that the cylinders are properly connected to the manifold and properly mounted and secured, especially against rotation. The CO2/halon containers must be weighed every two years and any container showing more than 10% weight loss

should be emptied out, inspected, hydraulically tested, dried and re-charged with correct amount of gas. All containers are to be hydraulically tested after 20 years and every 5 years thereafter. The remote controls, local controls and automatic release controls, if fitted, are to be tested. Visual andaudible alarms are to be tested. The Surveyor should inspect and test the system as far as practicable. Distribution lines should be checked and blown through with compressed air, if possible (to ensure that air is discharged satisfactorily from each discharge nozzle) and special attention should be given to cleanliness of the nozzles discharging over the engine room bilges. The condition of the pipe work is to be checked. The system should be examined and tested by a specialized company at least once every two years. Check that operating instructions are posted in language understood by the crew. The control stations or storage locations for fixed firefighting equipment should not be used as storage space for other materials.

    - Fixed Foam Fire Fighting System: Check installation controls, operating valves, storage room is in order, whether there is corrosion or damage to the installation/piping. Check that operating instructions are posted. Quantity of foam is to be checked. Foam sample is to be analyzed every 2.5 years. Whenever possible, the piping should be blown through with compressed air, to ensure that pipes and nozzles are clear. The control stations or storage locations for fixed fire fighting equipment should not be used as storage space for other materials

    - Water Sprinkler System: Check pipelines & nozzles for corrosion & damages. Check the automatic starting of pumps & alarm by pressure drop in the system (this can be achieved by opening the test valve. Check that the pressure of about 4-5 kg/cm2 is maintained in the system by operating the pump. Blow through the nozzles by water where possible, by air in accommodation.

    - Fireman's Outfit: Checks number of outfits (minimum 2 in cargo ships, 4 in tankers). All outfits should have helmet, boots, gloves, intrinsically safe type safety lamp, axe, protective clothing. Safety lamp battery must be checked to ensure that adequate light is available, and the battery is properly charged. The units are to be stored in properly marked boxes. Check that the self-contained breathing apparatus is complete and serviced. Check spare air bottles are available (2400 liter) and fully charged. A member of the crew should be requested to demonstrate the use to check correct operation of the SCBA, ensure that contents gauge is operational, and that low level (pressure) alarm is operational.

    - Emergency Escape Breathing Devices (EEBD): Check number of EEBD (minimum 2 in the E/R and 2 in Accommodation spaces.) A member of the crew should be requested to demonstrate the use and his familiarity. Flag State requirements posted on Orion should be verified in order to assess the specific number required by the ship’s administration. Panama Maritime Administration (AMP) Merchant Marine Circular (MMC) 142 contains Panama policy on this subject.

    - Smoke & Fire Detection System: Check the function by simulating smoke or heat at the sensors (about 10% of total number). Manually operated alarm call points are to be tested. Audio-visual alarm for power failure to the system should be tried out. If the system serving cargo holds is of the sample extraction smoke type, which uses dedicated or CO2 distribution piping, it should be checked to ensure that smoke from various compartments can be detected visually or by the initiation of a visual and audible signal.

    - Fire extinguishing systems for paint lockers should be examined to confirm that they remain efficient. A portable fire extinguisher may be used if the area of the paint locker is less that 4.5 m2. For larger paint

lockers fixed fire-extinguishing systems are required (normally CO2 system or water sprinkler system connected to the fire main).

    - Galley exhaust extinguishing systems should be specially examined with particular attention to cleanliness of grease traps and operation of dampers.

    - For unattended machinery spaces, the fire detection and alarm arrangements are to be tested. Both visual and audible alarms in the bridge, engineer’s office and other locations are to be tested.

## Special Requirements for Tankers

      - Fixed Deck Foam System: Check quantity & quality (from certificate) of foam concentrate. Carry out visual examination of piping monitors & portable applicators for corrosion & damage. Test the system using water (throw not less than 30m), movement of monitors (360horizontally, 45-75vertically) check that instructions are posted at various stations and valves are clearly marked and operate freely. Operating instructions should be posted in language understood by crew. Foam concentrates to be analyzed every

2.5 years.

      - Inert Gas System: The inert gas system forms part of class surveys. Check the survey status. If vessel is not classed or if records of class surveys cannot be found, the system must be tested including all alarms and cutouts as required in the section under class machinery surveys.

      - Fire safety arrangements in cargo pump rooms: The fixed fire extinguishing system is to be examined as applicable. External switches to stop ventilation fans should be tested and closing arrangements to be checked. Fire detection and alarm systems should be tested.

# SECTION 5

## Periodic Servicing of Launching Appliances and On-Load Releasing Gear

  - Revised requirements regarding periodical servicing of launching appliances and on-load release gear in accordance with Regulation SOLAS Ch. III/20 as amended.

    - The launching appliances shall be subjected to a thorough examination at intervals not exceeding 5 years and shall upon completion of the above examination be subjected to a dynamic test of the winch brake.

    - The lifeboat on-load release gear shall be subjected to a thorough examination and test during the surveys, required by Regulation I/7 and I/8 by properly trained personnel familiar with the system. Furthermore, at least once every five years the gear shall be overhauled and subsequently operationally tested under load of 1.1 times the total mass of the lifeboat when fully loaded with full complement of persons and equipment.

  - In accordance with the above, the five yearly thorough examination and tests are to be done in the presence of the surveyor. The examination/tests are to be carried out at the earliest opportunity (preferably at the next safety equipment renewal/periodic survey) or when testing is possible (when vessel is dry-docked or when vessel is in a suitable port where facilities are available. During annual/periodical safety equipment surveys, it should be verified that the on-load release mechanism is checked by properly trained personnel (such as manufacturer’s representatives, shipyard engineers or a specialist vendor certified by the manufacturer of the on-load release mechanism. A written statement should be obtained from the person who inspects the mechanism and a copy kept on board. For Panamanian vessels services providers which carried out above mentioned verification, should be duly

authorized by the Administration according to IMO requirements and MMC 258. Copy of reports covering the five yearly thorough examinations and tests are to be forwarded to GR Head Office. The Master should also be recommended to make log entries when the examination/tests of the launching appliances and on-load release mechanisms are carried out.

# SECTION 6

## Reporting

  - The relevant survey checklist Form No.CEC-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. The record form No. CEC-02 is tobe filled up if one is not available on board during the surveys and a copy submitted to GR Head Office.

  - At first surveys by GR, copies of the previous society’s certificates, record of equipment and Form E and list of any outstanding deficiencies, recommendations etc. should also be forwarded. A copy of the certificates for major equipment such as lifeboats, davits, winches, rescue boat, navigation lights etc. should be submitted, if available.

  - On completion of renewal surveys or First Survey by GR Class-Classified For Standard , under a transfer of class or certification procedure, an INTERIM Cargo Ship Safety Equipment Certificate should be issued along with record of equipment (Form E), valid for 5 months. When survey is incomplete or deficiencies are noted, the Interim Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the Interim Certificate and on the Surveyor Site Survey Statement Report P24. The certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two months. A copy of the CONDITIONAL certificate is to be e- mailed/ GR Head Office. GR Head Office should obtain written authorization from the flag prior to authorizing the surveyor to issue a conditional certificate.

  - On completion of annual/periodical survey, the existing full-term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR seal. A copy of the endorsed certificate must be submitted to GR Head Office. At annual surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the Statement Report P24. The Full-Term certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the Statement Report P24.

  - A survey Statement report P24 should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owners’ proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

  - In case it is required by GR Head Office, necessary supporting documentation such asservicing reports for life rafts, portable fire extinguishers and breathing apparatus, servicing report for fixed fire extinguishing system etc., should be submitted to GR Head Office. The purpose is to avoid up loading these documents to Orion Data Base. The evidence of these services should be maintainedat Regional Offices. Furthermore, the name of the service provider should be submitted on the checklist in order to

verify its approval according to our service providers’ procedure.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa. In case of conflict, the surveyor should follow the instruction issued by the flag GR Class is working on behalf unless there is an express authorization in written from the flag to act otherwise.

- .9	REFERENCE

| MARITIME ADMINISRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | WEBSITE |
| --- | --- | --- |
| PANAMA | 258 Authorized service providers for maintenance, thorough examination, operational testing, overhaul and repair of lifeboats and rescue boats, launching appliances and release gear. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 156 Exemption, Conditional, Interim and Full- Term Certificates. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 161 Voyage Data Recorder (VDR) and Simplified Voyage Data Recorder (S- VDR) annual test, and its Performance Standards | https://panamashipregistry.com/circular- category/circulares/ |
|  | 281 Guidelines for the Maintenance, Inspection of Fire-Protection System and Appliances. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 277 Fire Control Plan graphical symbols and Verification. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 112 EPIRB Programming and Registration. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 195 Long Range Identification and Tracking of Ships (LRIT). REPLACING THE MMC-197 AND MMC-249 | https://panamashipregistry.com/circular- category/circulares/ |
|  | 378 Announcement of Resolution entry in force January 1st, 2020, for Amendments to the International CODE for Fire Safety SYSTEM (FSS CODE) | https://panamashipregistry.com/circular- category/circulares/ |
|  | 142 Emergency Escape Breathing Devices (EEBD) | https://panamashipregistry.com/circular- category/circulares/ |
|  | 110 Radio Direction Finder Apparatus Exemption Certificate Ch. V, Reg. 12(p) of the 1997 Edition of SOLAS 74/78. | https://panamashipregistry.com/circular- category/circulares/ |
|  | 139 Automatic Identification System (AIS) | https://panamashipregistry.com/circular- category/circulares/ |

| PANAMA | 165 Guidelines for Periodic Testing of Immersion Suits and Anti-Exposure Suit Seams and Closures. | https://panamashipregistry.com/circular- category/circulares/ |
| --- | --- | --- |
|  | 250 Lifeboat Release and Retrieval System | https://panamashipregistry.com/circular- category/circulares/ |
|  | 158 EPIRB Registration | https://panamashipregistry.com/circular- category/circulares/ |
| BARBADOS | Bulletin 11: Minimum Safe Manning Requirements | https://barbadosmaritime.org/bulletins/ |
|  | Bulletin 160: LRIT – General Information | https://barbadosmaritime.org/bulletins/ |
|  | Bulletin 185: Inspection of Lifeboat Launching Equipment | https://barbadosmaritime.org/bulletins/ |
|  | Bulletin 194: Rescue Boat – Davit Equivalence | https://barbadosmaritime.org/bulletins/ |
|  | Bulletin 301: Escape Route signs and Equipment Location Markings | https://barbadosmaritime.org/bulletins/ |
| SAINT KITTS | MC 116 22 Compliance with Long-Range Identification and Tracking (LRIT) | https://www.skanregistry.com/en/maritim e-circulars |
|  | MC 18 09 Carriage of immersion suits | https://www.skanregistry.com/en/maritim e-circulars |
|  | MC 11 09 EPIRB’S | https://www.skanregistry.com/en/maritim e-circulars |