# SECTION 1

## General

  - Surveyors carrying out Cargo Ship Safety Radio surveys are to be familiar with the applicable provisions of SOLAS as amended particularly the amendments concerning radio communications for the Global Maritime Distress and Safety System.

  - Radio surveys are to be carried out by specialist radio technicians, with expertise in maritime radio, owing to the specialist nature of ship’s radio. Only those Radio technicians, who are approved by GR Head Office and/or Companies engaged in servicing and testing of radio communication equipment accepted by GR Head Office, according to procedure GR-PM-TD07 on Service Providers, must be allowed to carry out all radio surveys on behalf of GR Class  Classified For  Standard  (GR CLASS) . Radiosurveys, which are not independent surveys, such as surveys of radio installations on passenger ships orships complying with Caribbean Code for vessels less than 300 GRT, must also be carried out by approved radio technicians. The radio technician is expected to have a detailed knowledge of the ship’s radio equipment and its operation and should be able to carry out radio survey to InternationalRegister’s specification. The GR Class  Classified For  Standard  (GR CLASS) surveyor must take an active interest in the radio technician’s examination and tests be satisfied that the radio survey has been competently carried out and reported before accepting it as an GR Class  Classified For  Standard  (GR CLASS) survey and issuing Safety Radio E-certificates. Requirements for the approval of radio technicians are given in Annex to this chapter.

  - SOLAS (GMDSS requirements) does not, unless expressly provided otherwise, apply to:

    - Ships of war and troop ships

    - Cargo ships of less than 300 gross tons

    - Ships not propelled by mechanical means

    - Wooden ships of primitive build

    - Pleasure yachts not engaged in trade and

    - Fishing vessels.

  - The various items of safety radio equipment are itemized on the Record of Approved Cargo Ships Radio Installation (Form R) held on board. For all surveys the checklists (Forms No. CRC-01-04) should be filled up and signed and stamped by the radio technician and the surveyor.

# SECTION 2

## Surveys

  - Initial surveys: Is a complete inspection before a ship is put in service in order to verify all items related to the Safety Radio E-certificate are complied with and that these items are satisfactory for the service which the ship is intended.

When the survey is being done by GR Class  Classified For  Standard  (GR CLASS) for the first time, the surveyor/radio technician should fill up and submit Form R (Record of Equipment for GMDSS ships) and

Form No. CRC-01 (Survey Checklist). For initial survey, a Form ICRC must be completed accordingly. A copy of the completed Form CRC-01 should be left on board the vessel for use at subsequent surveys. During initial surveys it must be ensured that all equipment must be type approved by a Flag Administration and should conform to the appropriate performance standards adopted by IMO. When completing Form CRC-01, the equipment serial number should be indicated, in addition to the maker, type and location.

  - Renewal surveys are required every year (every five years as per HSSC, with periodical surveys every year). However, if a ship, at the time when its e-certificate is about to expire, is not in a port of the country in which it is registered or is unable to undergo survey either due to lack of facilities or other justifiable reason, the e-certificate may be extended for a period up to a maximum of five months. In all cases, GR Head Office will grant the exemption only when it appears reasonable and proper, for the purpose of allowing a ship to complete its voyage to a port with adequate facilities where a Safety Radio renewal/periodical survey can be carried out. It must be noted that such extension must be approved before the expiry of the e-certificate, as an already expired e-certificate cannot be extended.

  - Renewal surveys: The renewal survey is to verify that the installation continues to be in accordance with convention requirements. In case of modifications or renewal/replacement of equipment, the surveyor must update the record of equipment on board and forward a copy to GR Head Office. At all surveys, the Record (Form R) should reflect the actual arrangements and equipment situation on board at the time of survey and therefore the record should be amended whenever any modificationsor additions to radio equipment are carried out. The survey checklist (-01) is to be completed and signed by both radio technician and the GR Class  Classified For  Standard  (GR CLASS) surveyor.

  - Provided the ship is capable of performing all distress and safety signals, malfunction of the equipment for providing the general radio communications required by the regulations shall not be considered as making a ship un-seaworthy or as a reason for delaying the ship in ports where adequate repair facilities are not available. In such cases, a CONDITIONAL e-certificate may be issued with validity adequate to allow the vessel to reach the next port but not exceeding two (2) months. The items remaining outstanding are to be endorsed on the back of the CONDITIONAL e-certificates. The conditional e-certificate can only be issued once written authorization from the flag state is received by GR Head Office. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional e-certificate. In all other cases an INTERIM e-certificate valid for five (5) months is to be issued along with the (Form R) (record of Equipment) on completion of initial or renewal surveys.

  - Radio Station License: A Cargo ship safety Radio E-certificate is to be issued only if the vessel has a valid radio station license onboard. Copy of the valid radio station license must be submitted along with survey reports.

  - Exemption E-certificate: When an exemption is issued by the Flag Administration, all the requirements set out in the exemption e-certificate must be completely fulfilled before any e-certificate is issued. The exemption e-certificate is to be attached to any radio e-certificate issued. Any exemption e-certificate issued by a previous government will be invalid if a ship changes flag and new exemption will be required

from the new Flag State. In such cases, a radio e-certificate is not to be issued unless authorized by the Flag State and/or GR HO. Normally exemption is issued from GMDSS requirements if the coastal stations in which the ship operates do not possess adequate GMDSS facilities or if the vessel is to be taken out of service by 1st February 2001. In either case, the vessel has to comply with all the requirements given in SOLAS, Chapter IV, Regulation 4. In such cases, the survey checklist must indicate compliance with each of the requirements of Regulation 4. If the exemption e-certificate has a validity indicated, care should be taken to ensure that the validity of the Cargo Ship Safety radio e-certificate is limited to the expiry date of the exemption e-certificate. Any requests for exemption from GMDSS requirements must be forwarded to GR Head Office along with supporting documents (statement from the owner indicating the trading areas of the vessel along with documents from relevant coast stations/authorities in the area indicating lack of GMDSS facilities)

  - GMDSS Personnel: There are three categories of GMDSS radio competence. These are Radio Electronics E-certificate (REC – 1st or 2nd class), GMDSS General Operators E-certificate (GOC) and GMDSS Restricted Operator’s E-certificate (ROC). The SOLAS convention requires that there be on board one certified GMDSS radio operator. For Sea Areas A2, A3 and A4, the qualification level may be REC or GOC. However, if a ship’s voyages are confined to Sea Area A1, then the qualification level may be REC, GOC or ROC. Whenever on-board maintenance is specified, the qualification must be REC. The GMDSS personnel should have e-certificates issued by the relevant Flag State (or acceptable to the Flag State) andthe number of operators should be as per the minimum safe manning e-certificate. E-certificates issued by organizations other than Flag States or their authorized radio organizations (proof of passing e- certificates etc) should not be accepted. If satisfactory radio operator e-certificates are not submitted, only CONDITIONAL E-certificate must be issued. The number of operators shown in Form No. SAFRA-02 (Form R) should be the minimum statutory requirement. The actual on board may exceed this number. For PANAMA flag vessels there must be two qualified operators (GOC or ROC as appropriate) who must be navigating officers, or one exclusive qualified operator who is also a certified radiotelegraphy officer or a suitably qualified electronics officer. Only PANAMANIAN e-certificates should be accepted. There have been many cases where GMDSS operators were not familiar with the GMDSS radio equipment or able to operate the equipment competently despite holding valid GMDSS GOC e-certificates. In view of this, the radio technician and attending Surveyor should check the competence of the operators during the surveys.

  - Ship Identity Encoding: It should be ensured during survey that the ship identification is properly encoded and activated, such as MMSI (Maritime Mobile Service Identifier) and IMN (INMARSAT Mobile Number).

  - Equipment maintenance: The methods of equipment maintenance (duplication of equipment, shore- based maintenance or at-sea maintenance capability) should be acceptable to the Flag Administration and must be in accordance with the guidelines contained in IMO Resolution A.702 (17). When shore- based maintenance is used, a copy of the contract should be forwarded to GR HO. If at-sea maintenance capability is included, it should be ensured that the necessary technical documentation, tools, test equipment and spare parts are available on board. Additional requirements regarding radio personnel (see above) should also be complied with. For ships sailing in Sea Areas A1 and A2, any one of the methods may be adopted and in A3 and A4 any two may be adopted.

  - Two-way VHF radio telephones: There should be minimum 3 two-way VHF (hand held portable) radiotelephones for cargo ships of 500 GT and over and for all passenger ships and minimum 2 for cargo ships of less than 500 GT. The equipment should comply with the technical requirements of IMO Resolutions A.605 (15) and must be a highly visible yellow or orange color.

  - Ship Radio Station Documents: The documents on board should include the radio log book, listings of call signs (such as ITU or British Admiralty list) and coast and ship radio stations as well as the appropriate manuals for the services appropriate to the ship radio station. The logbook entries should be verified and evident omissions or departures from the specified requirements alerted to the Master.

  - Emergency position Indicating Radio Beacon (EPIRB): While the convention offers choice between the VHF, COSPAS/SARSAT and the INMARSAT EPIRBs, invariably a COSAS/SARSAT 406 MHz type EPIRB is fitted on all ships. The general deficiencies noted during surveys include EPIRB left in a de-activated condition, float free arrangements improper, Overdue batteries, HRU servicing or replacement overdue and Equipment registered with another ship or not registered at all

  - Navtex receiver: This is required for all convention ships operating in waters served by a NAVTEX broadcast service. The general deficiencies noted during surveys include the following:

    - Equipment being switched off when in a NAVTEX coverage area

    - Faint print on the paper message roll

    - Inadequate paper roll supply

    - A model of equipment, which does not include a printer.

# SECTION 3

## Reporting

  - The relevant survey checklist Form CRC-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. The Form R is to be filled up if one is not available on board during the surveys and a copy submitted to GR Head Office.

  - At first surveys by GR, copies of the previous society’s e-certificates, record of equipment (Form R) andlist of any outstanding deficiencies, recommendations etc. should also be forwarded.

  - On completion of renewal surveys or First Survey by GR CLASS , an INTERIM e-certificate should be issued along with record of equipment Form R, valid for five (5) months. When survey is incomplete or deficiencies are noted, the Interim E-certificate should be marked CONDITIONAL, and the outstanding items/deficiencies must be entered at the reverse of the e-certificate and on the Report Statement (P24). The e-certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two months. A copy of the CONDITIONAL e-certificate is to be sent to GR Head Office.

  - In case it is required by GR Head Office, necessary supporting documentation such as copy of radio station license, approval letter for radio technician carrying out surveys, copies of radio operatorlicenses, copy if shore-based maintenance contract, copies of equipment test print outs etc. should be submitted to GR Head Office. The purpose is to avoid uploading these documents to GR Class Data Base. The evidence of these services should be maintained at Regional Offices. Furthermore, the name of the service provider should be submitted on the checklist in order to verify its approval according to our service providers’ procedure.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa. For Barbados Maritime, https://barbadosmaritime.org/documents/bulletins/ In case of conflict, the surveyor should follow the instruction issued by the flag GR Class is working on behalf unless there is an express authorization in written from the flag to act otherwise.

4.0	REFERENCE

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021. | 06/2022 | https://panamashipregistry.com/circular- category/circulares/ |
|  | MMC 18: Required Documents for Ship Radio Station | 10/2011 |  |
|  | MMC 94: Panama Policy - GMDSS Flag Vessels | 06/1995 |  |
|  | MMC 103: GMDSS equipment requirements | 08/1998 |  |
|  | MMC 105: Global Maritime Distress and Safety System (GMDSS) Fines | 01/1999 |  |
|  | MMC 109: Application of GMDSS in accordance with International Tonnage Certificate ITC-69. | 08/2022 |  |
| Barbados | Bulletin 160: LRIT – General Information | -- | https://barbadosmaritime.org/bulletins/ |
| Belize | MSN-0036: Implementation of the LRIT System. | 25/07/2008 | https://www.immarbe.com/circularslist/ |
| Sierra Leone | MC No.25: Long – Range Identification and Tracking of Ships (LRIT) | 30/07/2018 | https://slmarad.com/maritime-circulars |
| Palau | MN 172.1: Required documents and publications for Ship Radio Stations. | 12/2018 | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentation /instruments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |

| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disposici ones |
| --- | --- | --- | --- |
| Nigeria | NA | NA | https://nimasa.gov.ng/press-center/marine- notice-adverts/ |
| Togo | Circular No.006C/TG/03/13: LRIT Data | 21/03/2013 | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |