|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

INSTRUCTIVE FOR THE SURVEY AND ISSUANCE OF THE CERTIFICATE AND CARGO GEAR RECORD BOOKLET

|  | POSITION | DATE | SIGNATURE |
| --- | --- | --- | --- |
| PREPARED BY |  |  |  |
| REVISED BY |  |  |  |
| APPROVED BY |  |  |  |

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 1 of 7 |
| --- | --- | --- | --- |

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)

E-mail: info@grclass.com Web: www.grclass.com

|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

# OBJECTIVE

To establish an appropriate mechanism to accomplish and control systematically the surveys and completion of reports and the Cargo Gear Booklet.

# SCOPE

The content of this Instructive applies to all cargo gear of cargo ships.

# RESPONSIBILITIES

  - It is responsibility of the personnel of the Technical Department to assure that all the processes contained in this INSTRUCTIVE are fulfilled for the surveys and issuance of the Certificates and Cargo Gear Booklet.

  - It is responsibility of the personnel of the Technical Department to support in the monitoring for the compliance of the mechanisms for surveys and issuance of Certificates and Cargo Gear Booklet.

  - It is responsibility of the surveyors to comply with the procedures contained in this instructive when carrying out the surveys for Certificates and Cargo Gear Booklet.

# DEFINITIONS

GR Class	GR Class-Classified  For Standard ., is an Recognised Organization and Recognised Security Organisation to perform inspection and certification work on ships registered under a Flag Administration.

Classification Rules

They are referred to those issued by a classification society with which GR CLASS has signed contractual agreement for sharing such rules in ship surveys and certification activities.

ILO	International Labor Organization: It is a specialized agency of the United Nations dedicated to labor matters.

ILO CONVENTION NO. 32

ILO CONVENTION NO. 152

Maritime Administration National Regulations

related to the protection against accidents of workers dedicated to the loading and unloading of the ship (1932).

in reference to the protection against accidents of the personnel engaged in the loading & unloading of the ships (1979).

is the Authority responsible to regulate all aspects related to the marine requirements of the flag.

Are those established by each Maritime Administration to implement IMO Regulations or to adopt standards not envisaged in International Conventions.

Applicable Regulations

The surveys for the test loading certificates and endorsement of the Cargo Gear

Booklet will be carried out to ships according to ILO Regulations, specially the ILO Convention NO. 152.

GR Class will issue a Cargo Gear Booklet to cargo ships so that the Captain can show same to the authorities, including the surveyors of this company when requested.

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 2 of 7 |
| --- | --- | --- | --- |

|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

To the surveys that reflect compliance with the requirements of the ILO Conventions, GR CLASS will issue a corresponding Cargo Gear Certificates.

Types of Surveys

Initial survey	will be carried out in any of the following cases:

- For the issuance of load test certificate CG-IL for examination for interchange able components and loose gear, before they are taken into service.

- For the issuance of load test certificate CG-LA for examination of lifting appliances (for Derricks or Cranes) before they are taken into service.

- For the issuance of the load test certificate CG-R for examination wire rope before they are taken into service.

- For the issuance of load test certificate CG-UP for examination Derricks used in union purchase.

Annual surveys are carried out after the initial survey during the three (3) years following to the initial survey of the derricks and permanent attachments (including supporting chains) to derricks, masts and decks (see part I of the Register of ship’s cargo gear CG1).

Quadrennial throughout survey Annual throughout inspection

will be carried out to equipment listed.

will be carried out to winches, lifts and their accessories.

Annual throughout inspection of Cargo Gear exempted of heat treatment.

Periodical inspection of six (6) or twelve (12) months, or every	two (2) years to equipment or accessories subject to heat treatment.

Types of Certificates

Interim Load

Test certificate

Full Term Load Test Certificate

with	a maximum	validity	of five	(5)	months	counted	from the date of

inspection,  is  issued  by  the surveyor to  the  ship  whose survey to the equipment to his criteria complies with the requirements.

has an indefinite validity, it is issued by the Head Office of GR CLASS, to the ships whose survey and recognition of the equipment demonstrates a satisfactory result.

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 3 of 7 |
| --- | --- | --- | --- |

|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

# ACTIVITIES

  - GENERAL

    - Documents Necessary For Approval Surveyors Under Cargo Gear:

      - Certification or course of lifting Appliance inspection.

      - Supporting documents of equipment which will be used for this inspection, example dynamometer. Send us certification of calibration and documentation which prove the familiarization of the personal with the equipment which will be used.

      - Please inform how many inspections of CG have been done and name of vessels.

      - Certificate of familiarization with ILO 32 and ILO 152

      - Which Procedures will be used for TEST and though examination of cargo handling gear before being taken into use

      - Which procedure will be used for Test and through examination on derricks used in union purchase

      - Which procedure will be used for Test and through examination of wire rope

      - Which procedure will be used for Re-test and through re-examination of cargo handling gear

# PROCEDURE OF SURVEYS FOR LOAD TEST CERTIFICATE

    - The Surveyor verifies that all tests are carried out and instructions for each type of equipment and material are complied with, before being taken into service in conformity with the indications of the Cargo Gear Booklet CG1.

    - The Surveyor obtains when proceeds, copy or certification and Load Test from manufacturer.

    - The Surveyor carries out visual inspection or tests required for annual or quadrennial endorsement.

    - The Surveyor completes the form Cargo Gear Survey Report CG-SR in which will be noted the actual condition of the equipment and accessory inspected.

    - The Surveyor writes down the deficiencies in the Cargo Gear Booklet.

    - The Surveyor, on the cases of detecting or been reported that a ship is found with deficiencies of any type during initial, annual, periodical, renewal, or by Port State Control - Flag State Control, shall proceeds according with the Service Procedure

    - (GR-PM-TD03).

    - The Surveyor complies with the established Service Procedure (GR-PM-TD03), concerning the notification via e-mail to the Technical Department of GR CLASS Head Office for the surveys carried out and the mailing of copies of the documentation that supports the survey and the issued or endorsed certificates to the ship.

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 4 of 7 |
| --- | --- | --- | --- |

|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

# PROCEDURE OF CERTIFICATION AND ENDORSEMENT OF CARGO GEAR BOOKLET

    - The Surveyor  follows the Service Procedure  (GR-PM-TD03) for  the issuance of  interim certification and issues an interim certificate CG-IL, CG-LA, CG-UP or CG-R whose validity in no case will exceed five (5) months counted from the date of the inspection upon concluding the initial survey and satisfies the requirements of the ILO Convention NO. 152.

    - For the flag Administrations that do not allow the issuance of a Certificate or Statement of Compliance (Panama flag) under their authority, the Surveyor must issue an interim certificate at Owner’s request. In no case the validity of an interim statement of compliance should exceed five months.

    - The Surveyor endorses the Cargo Gear Booklet CG1 to the inspection of the Cargo Gear and its accessories (annual or quadrennial) with a satisfactory result.

    - The Surveyor issues a statutory Load Test certificate of Cargo Gear with indefinite validity to all surveys that satisfy the general procedures and what is indicated in this Instructive.

    - The Technical Manager issues a Full Term Load Test Certificate, whose validity will not exceed five (5) years to all surveys that satisfies the general procedures and what is indicated in this instructive.

    - For the flag Administrations that do not allow the issuance of a Certificate or Statement of Compliance (Panama flag) under their authority, the Technical Manager shall issue a Full Term Certificate at Owner’s request. In no case the validity of an interim statement of compliance should exceed five years.

    - The Surveyor provides a Register of Ship’s Cargo Gear CG1 to attach The Load Equipment Test Certificates and for the recording of endorsements of annual survey, annual throughout survey or quadrennial throughout survey of the cargo gear.

    - The Surveyor complies with the established Service Procedure (GR), concerning the notification via e-mail to the Technical Department of GR Class Head Office for the surveys carried out and the mailing of copies of the documentation that supports the survey and the issued interim certificate or endorsement of the full term of the Cargo Gear Book CG1.

# 5.4	Flag Administration requirements

5.4.1 The Flag Administration requirements should also be taken into consideration when carrying out any type of survey on board no matter the scope. The Flag Administration requirements are listed through GR-PM-TD12.

# RELATED DOCUMENTS

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 5 of 7 |
| --- | --- | --- | --- |

|  | SERVICE PROCEDURE | I-12-CG |
| --- | --- | --- |
|  | INSTRUCTIVE | Revision 00 |
|  |  | 00/00/0000 |

Certificate Of Test And Thorough Examination For

Interchangeable Components And Loose Gear (Interim)	CG-IL-IC Certificate Of Test And Thorough Examination

Of Lifting Appliances (For Derricks Or Cranes) (Interim)	CG-LA-IC Certificate Of Test And Thorough Examination

For Wire Rope (Interim)	CG-R-IC

Certificate Of Test And Thorough Examination

(For Derricks Used In Union Purchase) (Interim)	CG-UP-IC Certificate Of Test And Thorough Examination For Interchangeable

Components And Loose Gear	CG-I

Certificate Of Test And Thorough Examination Of Lifting Appliances

(For Derricks Or Cranes)	CG-LA

Certificate Of Test And Thorough Examination For Wire Rope	CG-R Certificate Of Test And Thorough Examination

(For Derricks Used In Union Purchase)	CG-UP Certificate Of Test And Thorough Examination For

Interchangeable Components And Loose Gear (Interim) (At Owner’s request) CG-IL-IC-OR Certificate Of Test And Thorough Examination

Of Lifting Appliances (For Derricks Or Cranes) (Interim) (At Owner’s request)CG-LA-IC-OR Certificate Of Test And Thorough Examination

For Wire Rope (Interim) (At Owner’s request)	CG-R-IC-OR Certificate Of Test And Thorough Examination

(For Derricks Used In Union Purchase) (Interim) (At Owner’s request) CG-UP-IC-OR Certificate Of Test And Thorough Examination For Interchangeable

Components And Loose Gear (At Owner’s request)	CG-IL-OR Certificate Of Test And Thorough Examination Of Lifting Appliances

(For Derricks Or Cranes) (At Owner’s request)	CG-LA-OR

Certificate Of Test And Thorough Examination For Wire Rope (At Owner’s request) CG-R-OR Certificate Of Test And Thorough Examination

(For Derricks Used In Union Purchase) (At Owner’s request)	CG-UP-OR CG-SR  Survey Report

Cargo Gear Booklet	CG1

Control of Documentation	CONTROL LIST-CG

# REFERENCES

Procedure for the Control of Documents and Records	GR-PM-QD01

Flag Administration requirements	GR-PM-TD12

Request for survey	GR-PM-TD03

Service Procedure	GR-PM-TD07

| Prepared by Quality Manager | Revised by Technical Manager | Approved by General Manager | Page 7 of 7 |
| --- | --- | --- | --- |