# SECTION 1

- General

  - Some administrations require compliance with International Labor Organization Conventions 92 & 133 (ILO 92 and ILO 133), while others have their own national requirements. Some administrations have no mandatory requirements. In many cases Owners often require compliance with ILO standards on a voluntary basis. Crew accommodation surveys are required to be carried out on PANAMANIAN vessels.

  - ILO 92 applies to all sea going cargo and passenger ships of 500 gross tons and over and ILO 133 to cargo and passenger ships of 1000 tons gross and over. ILO 133 is supplementary to ILO 92. For vessels over 200 tons but below the limit for ILO 92 or 133, the Regulations should be applied where reasonable and practicable. Fishing vessels are required to comply with Part III of ILO Convention No. 126.

  - General arrangement drawings of accommodation, galleys and provision stores are to be sent to GR Head Office for verification and approval. Plans should preferably be drawn to scale 1:100, but other scales showing similar or better detail will be accepted. At initial surveys (new buildings or cases where the vessel does not hold a previous crew accommodation certificate), the surveyor should check the applicable plans (accommodation layout, including galleys, hospitals, medicals, and sanitary arrangements together with plans of awnings for ships engaged in voyages through tropics) to verify compliance with the requirements. The surveyor should satisfy himself that the actual arrangements on board correspond with those indicated in the plans. This stage should ideally be completed before initial survey is conducted but may take place immediately after the survey if time constraints prevent doing otherwise.

  - The survey report is self-explanatory and should be used when carrying out surveys. A summary of details of the crew accommodation, galleys, provision stores and fresh water supply is to be compiled in the Record Report (Form No. OMCA-02). This format includes specific references to all items which are required by ILO conventions and supplementary provisions.

  - The survey will consist of a detailed visual inspection of the crew accommodation, galleys, and food storage areas to ascertain the maintenance, cleanliness and structural integrity of these spaces and particulars, according to the approved plans and record. At this time the surveyor shall test as applicable, heating systems, ventilation systems, air conditioning (if required); artificial lighting, sanitary pumps, piping, and fixtures (seawater, fresh water and drinking water systems), sanitary drains, laundry equipment, galley equipment, and refrigerated stores machinery. These aspects are to be reported in the Survey

  - Report (Form No. OMCA-01), which includes references to all items to be verified in the course of the survey. The survey report shall state any outstanding items left at the end of the survey. The correction of pending items shall be verified by a re-survey.

  - At the end of the initial survey, the surveyor will issue an Interim certificate valid for five (5) months, and a copy of the Record Report (Form No. OMCA-02)

- Issuance of Full-Term Certificate

  - Specific requirements

    - Panama and Bolivia: When the ship and relevant documentation is considered by GR Head Office to be in full compliance with the applicable regulations and flag state provisions, GR Head Office will issue the full-term certificate and, within seven working days, a request will be sent to the Administration in Panama for the endorsement of the Full-term certificate of crew accommodation. Once available, this certificate will be forwarded by GR Head Office to the Owner or GR Representative. The full-term certificate will be valid for four (4) years from the date of the completion of the survey. The certificate will cease to be valid if the ship undergoes alterations, which are

considered to result in a substantial departure from the conditions of issue of the certificate.

    - Belize: When the ship and relevant documentation is considered by GR Head Office to be in full compliance with the applicable regulations and flag state provisions, the full-term certificate will be issued valid for five (5) years from the date of completion of the survey. The certificate will cease to be valid if the ship undergoes alterations, which are considered to result in a substantial departure from the conditions of issue of the certificate.

  - Renewal surveys are required every four years. This survey is similar to the initial survey described in 1.5, plus a check that any alteration to the ship, if relevant to the contents of the certificate, being also noted in the Record of Approved Crew Accommodation Details (Form No. OMCA-02). The survey should be such as to ensure that the accommodation is maintained in a clean and habitable condition and all equipment and installations are maintained in good working order. The survey checklist Form No. OMCA-01 is to be filled up and submitted at all renewal surveys. Form No. OMCA-02 should be submitted only if there has been a change on the record from previous survey. On completion of surveys, an INTERIM Crew Accommodation certificate is to be issued for five months. GR Head Office will issue the full-term certificate after review of the reports. Any deficiencies noted during surveys must be entered into “Survey Statement” into GR CLASS MIS, if they are not rectified during the surveys.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa. In case of conflict, the surveyor should follow the instruction issued by the flag state that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

- Exemptions

  - Specific Requirements

    - Belize: This Administration may grant exemptions from all or any of the provisions of these Conventions (as may be specified in the exemption) on such terms (if any) as we may so specify and may, subject to giving reasonable notice, alter or cancel such exemption. Vessels applying for an Exemption Certificate to this Administration will need to provide in addition to a written application, the rationale for the particular request endorsed by the Recognized Organization which performed the related survey onboard the vessel.

    - Panama: The General Directorate for Seafarers may issue an Exemption or Dispense Letter pursuant to the provisions of ILO Convention 92 and 126 of the International Labor Organization by means of Note addressed to the General Director for Seafarers, on which shall be detailed: The provisions of the ILO Convention 92 and 126 that shall not be complied with anymore.

- Reasons to justify the waiver or exemption

  - Alternative compliance measures.

    - Copy of the floor area of the vessel where the alteration or modification applies.

    - In the event that a dispense/exemption is granted, a copy shall be attached to the Certificate of Inspection of Crew Accommodation (CICA) to be endorsed, making the appropriate annotation in the Remarks Box (Number and date of issue of the exemption), and to enclose the Survey Report and the Record of Approved Crew Accommodation Details (RACAD), to evidence that it complies with the rest of the provisions of the ILO Convention 68, 92 and 126, if applicable.

  - Other Flag States: Crew Accommodation Surveys and Certification under a Flag State not included in this procedure will be processed according to instructions from the Flag State. This procedure will be revised as necessary to include said instructions.

# REFERENCES

- www.imo.org

- www.ilo.org

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 396: Regulation for the issuance of the Certificate of Inspection of Crew Accommodation (CICA) and its exemptions, dispensations, and authorizations of the Ship Registry of the Republic of Panama. | March 2022 | https://panamashipregistry.com/circulars/ |
| Barbados |  |  | https://panamashipregistry.com/circular- category/circulares/ |
| Sierra Leone |  |  | Maritime Circulars - Sierra Leone Maritime Administration (slmarad.com) |
| Vanuatu |  |  | Vanuatu Maritime Services Safety Forms (vanuatumaritimeships.com) |
| St. Kitts & Nevis |  |  | Maritime Circulars (skanregistry.com) |
| Nigeria |  |  | https://nimasa.gov.ng/?s=notice |