# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

Table B

IMSBC CODE-INITIAL CHECKLIST

(For COAL and BROWN COAL BRIQUETTES)

| Name of ship | Distinctive number or letters | Port of registry | Gross Tonnage |
| --- | --- | --- | --- |
|  | IMO Number: |  |  |

(The following requirements govern the application of Table 2.3)

|  | Requirements | Y,N, N/A |
| --- | --- | --- |
| 1 | Boundaries of cargo spaces are to be resistant to fire and liquids |  |
| 2. | Electrical equipment fitted in the cargo holds are to be of safe type having an explosion protection grade of IIAT4 or upwards. Not suitable explosion protected type electrical equipment are to be capable of being positively isolated from outside of the spaces and have the enclosure having a protection degree of IP55 or upwards, and caution plates to ensure isolation of electrical equipment are to be provided. |  |
| 3 | Suitable means for measuring following gases, etc. in cargo spaces without entry into such spaces are to be provided. Electrical measuring device is to be of safe type having an explosion protection grade of IIAT4 or upwards. Methane Oxygen Carbon monoxide pH value Temperature( 0 - 100ºC) |  |
| 4(*) | Two sets of self-contained breathing apparatus are to be provided. (Note: The apparatus required by SOLAS Reg.II- 2/17(00E) or Reg.II-2/10(00N) may be used for this purpose) |  |
| 5 | “No Smoking" signs are to be posted in conspicuous places. |  |
| 6(*) | Natural ventilation system is to be provided for cargo spaces and air holes should be provided at the upper part of web plates of longitudinal and transverse girders fitted to deck plates with appropriate spacing. Note: Air holes should not be located at any part that may be subject to stress concentration. |  |
| 7 | Natural or mechanical ventilation systems are to be provided for adjacent enclosed working spaces, such as store rooms, carpenter's shops, passage ways and tunnels. In the case of mechanical ventilation, only the equipment which is safe type for use in an explosive atmosphere can be used in cargo area. |  |
| 8 | Two sampling holes per hold, one on the port side and one on the starboard side of the hatch cover or upper parts of hatch coamings are to be provided with threaded stub and sealing cap. |  |

Note:   1.   The items marked with (*) are not applicable to brown coal (lignite) briquettes.

2.   The results of confirmation survey on board have been shown in the right columns.

## Issued at	On the

## GR CLASS REPRESENTATIVE