- General

  - BC Code was developed at IMO with primary aim to promote safety of stowage and shipment of solid bulk cargoes. IMO Maritime Safety Committee 85th session (MSC85) held from November to December 2008 adopted the amendments of BC Code, of which individual schedules of solid bulk cargoes had been reviewed and the name had been changed to IMSBC (International Maritime Solid Bulk Cargoes) Code, and the amendments to the SOLAS Convention which makes the IMSBC Code becomes mandatory. With these amendments, the IMSBC Code was scheduled to be mandatory on or after 1 January 2011.

  - A number of states have been requiring that ships carrying solid bulk voluntary application date of the amendments to the SOLAS Convention mentioned in the above into consideration, it is estimated that a number ofstates will require that ships carrying solid bulk cargoes in their waters should have a Certificate of Compliance with the International Maritime Solid Bulk Cargoes Code (hereinafter the certificate as “IMSBC Code fitnesscertificate”).

  - GR Class conducts when requested, a document examination and/or a survey, and issue a certificate offitness of a ship for the carriage of solid cargoes in bulk in accordance with the provisions of the IMSBC Code.

  - The certification is conducted for the following periods based on the issuance of the certificate.

    - Period: 01 January 2009 – 31 December 2010……BC Code or IMSBC Code fitness certification

    - Period: 01 January 2011 – ……. IMSBC Code fitness certification

  - Comparing the requirements on the vessel´s construction and equipment under the BC Code and those of the IMSBC code, some requirements are found to be added, for quick reference, the said cargoes are shown in the “Table G1 – Cargoes for which some requirements on construction/equipment are added”.

  - Under these Code, materials are classified into those likely to liquefy, those having chemical hazard and other. The materials having chemical hazard can be divided into those classified as dangerous goods in IMDG Code and materials hazardous only in bulk (MHB). Ships intended for the carriage of dangerous goods and constructed on or after 1 September 1984 are required to be complied with the requirements of SOLAS ‘74 Reg.II-254 (Reg.II-2/19 under SOLAS 2000) as amended.

  - The materials having chemical hazard can also be divided into non- combustible or low-fire-risk materials and other materials. Ships carrying cargoes other than those of non-combustible or low-fire- risk cannot be exempted from the requirement of SOLAS Reg.II-2/53.1.1 (Reg.II-2/10.7 under SOLAS2000) for a fixed fire extinguishing system in cargo spaces. Ships exempted from the requirement are required to have an exemption certificate to which a list of cargoes permitted is to be carried, which are to be non-combustible or low-fire-risk cargoes.

  - BC Code or IMSBC Code fitness certification is conducted conditional upon that the ship is complied with therequirements of the rules of GR Class and SOLAS ‘74 as amended applicable for the carriage of the cargoes.

# Table G1 – Cargoes for which requirements on construction/ equipment are added Additional requirements are shown in red

  - The contents of each column in the Table G1 are as follows.

    - Materials (column “a”)

Bulk Cargo Shipping Names are expressed in capital letters and identifies a bulk cargo during transport by sea.

    - IMO class (column “b”)

Group B: cargoes are categorized into the following classes. Class 4.1: Flammable solids

Class 4.2: Substances liable to spontaneous combustion

Class 4.3: Substances which, in contact with water, emit flammable gases Class 5.1: Oxidizing substances (agents)

Class 7: Radioactive materials

Class 9: Miscellaneous dangerous substances and article

MHB: Materials which may possess chemical hazards when transported in bulk other Than materials classified as dangerous goods in the IMDG Code.

    - UN No. (Column “c”)

This is a 4-digit number assigned to a particular dangerous substance included in the dangerous substance list (approximately 3,000 items) within the United Nations Committee of Experts on the Transport of Dangerous Goods.

    - Group (column “d”)

A: Group A consist of cargoes which may liquefy if shipped at moisture content inexcess of their transportable moisture limit.

B: Group B consist of cargoes which possess a chemical hazard which could give rise to a dangerous situation on aship.

C: Group C consists of cargoes which are neither liable to liquefy (Group A) nor to Possess chemical hazards(Group B).

    - Stowage (column “e”)

A: Bulkheads to the engine room are to be insulated to A-60 standard.

F: Boundaries of components are to be resistant to fire and passage of water. G: Bulkheads to the engine room are to be of gastight.

    - NO SMOKING sign (column “f”)

Yes: “NO SMOKING” signs are to be posted on decks and in areas adjacent to cargo compartments

    - Ventilation (column “g”)

N: Natural ventilation system is to be provided for cargo holds.

Nm: Natural or mechanical ventilation system is to be provided for cargo holds. M: Mechanical ventilation system is to be provided for cargo holds.

ML: At least two mechanical ventilation fans are to be provided for cargo holds.

The total ventilation is to be at least six air changes per hour. Ventilation openings are to comply with the requirements of the Load Line convention as amended for openings not fitted with means of closure.

Sa: Ventilation fans are to be safe for use in a flammable atmosphere.

Sp: Spark-arresting screens (wire mesh guards with max. 13mm X 13mm) are to be Fitted to ventilation openings.

    - SCBA (column “h”)

Yes: Two self-contained breathing apparatuses with 200% spare cylinders are to be additionally provided

    - Protective clothing resistant t chemical attack (column “i”)

Yes: Four sets of protective clothing which consists of a pair of gloves, boots, a protective Clothing, and helmet with goggles are to be additionally.

    - Bilge line (column “j”)

F: In case where bilge lines are led to machinery space, bilge line is to be isolated either by fitting a blank flange or by a closed lockable valve.

N: a notice is to be placed adjacent to the valve warning against opening without the master´s permission.

    - Electrical equipment (column “k”).

Not suitable explosion protected type electrical are to be disconnected (by removal of links in the system, other than fuses) from the power source at a point external to the space.

IIAT2: Electrical equipment having an explosion protection grade of IIAT2 or upwards are considered as suitableexplosion protected type electrical equipment.

IIAT3: Electrical equipment having an explosion protection grade of IIAT3 or upwards are considered as suitableexplosion protected type electrical equipment.

IIAT4: Electrical equipment having an explosion protection grade of IIAT4 or upwards are considered as suitableexplosion protected type electrical equipment.

IICT1: Electrical equipment having an explosion protection grade of IICT1 or upwards are considered as suitableexplosion protected type electrical equipment.

IICT2: Electrical equipment having an explosion protection grade of IICT2 or upwards are considered as suitableexplosion protected type electrical equipment.

IS: Intrinsically safe type electrical equipment is considered as suitable explosion protected type electricalequipment.

    - Dual purpose nozzles (column “l”)

Yes: Nozzles provided with fire hoses are to be of dual-purpose type (i.e., spray/jet type).

    - 4 jets of water (column “m”)

Yes: The quantity of water delivered is to be capable of supplying four nozzles at pressure as specified in SOLAS regulation and being trained on any part of the cargo space when empty.

    - Requirements of SOLAS Reg.II-2/54.2 (Reg.II-219.3 on or after 2000 amendments) (column “n” ~ “u”)X: Applicable.

    - FFEA (SOLAS Reg.II-2/10.7.1.3) (column “v”)

Yes: Fixed CO2 fire extinguishing system for cargo holds are required by SOLAS Reg.II- 2/10.7.1.3. (Yes): Fixed gas fire-extinguishing system is ineffective and for which a fixed fire extinguishing

system giving equivalent protection shall be available. According to the Unified interpretation of IMO, water supplies defined in SOLAS Reg.II-2/19.3.1.2 are considered as the alternative of a fixed gas fire- extinguishing system in cargo spaces.

  - General notes:

    - For the detailed requirements of the IMSBC Code, the relevant part of the Code should be referred to.

    - The application of the requirements of SOLAS Reg.II-2/54.2 or 19.3 is shown just for ready reference. For the detailed requirements, the relevant part of the SOLAS should be referred to.

    - Blank columns mean “Not applicable”

  - Important Notes:

    - CASTER MEAL, CASTER POMACE and CASTER FLAKE shall not be carried in bulk.

    - For the planned voyage not exceeding 5 days from the commencement of loading to the completion ofdischarge, the vessel may be exempted from the requirements of FFEA.

    - Consideration shall be given to providing the vessel with the means to top up the cargo spaces with additional supplies of inert gas taking into account the duration of the voyage. The ship´s fixed CO2 fire extinguishing system shall not be used for this purpose.

    - Only applicable to Industrial sand coated with resin.

    - Only applicable to Seedcake containing solvent extractions only.

    - Fine grained sulphur (flowers of sulphur) shall not be transported in bulk.

    - With moisture content of 15% or more, the vessel may be exempted from the requirements of FFEA.

    - To be stowed out of direct contact with a metal engine room boundary.

    - Only suitable wire mesh guards are required.

- Guidance for IMSBC Code fitness certification

  - General

    - Under the IMSBC Code, materials are classified into those likely to liquefy (Group A), those having chemical hazard (Group B) and others (Group C). In this guidance, they are referred to as “Group A cargoes”, “Group B cargoes” and“Group C cargoes” respectively.

  - Requirements for construction and equipment

    - A loading manual and stability information booklet are required to be provided onboard regardless of the types of cargoes intended to be carried.

    - No special construction and equipment are required for the carriage of Group A and C cargoes except that specially designed portable divisions or permanent structural boundaries to confine any shift of cargo to an acceptable limit are required for the carriage for Group A cargoes without appropriate restrictions on their moisture contents.

    - For the carriage of Group B cargoes, ships are to be complied with the requirements for special construction and/or equipment specified in IMSBC Code. The requirements for the carriage of Group B cargoes except coal and brown coal (lignite) briquettes are summarized in Table 2.1 and

2.2. The requirements for the carriage of coal and brown coal (lignite) briquettes are shown in Table 2.3.

# Notes – 1

The Code provides special requirements for construction and equipment for fire protection and personnel protection as well as operational precautions and information on properties of each material.

The applications of the requirements of SOLAS74 Reg.II-2/53 and 54 (Reg.II- 2/10.7 and 19 under SOLAS2000) are also shown in Table 2.1 for ready reference.

  - Application

    - Applicant, the shipowner, or their representative should submit an application containing the information onthe item listed below to Head Office, Marine Operations Department (MOD), prior to the survey onboard the ship.

      - Cargoes for which an IMSBC Code fitness certificate is to be issued (for example, “Group A and C

cargoes” or“Group A and C cargoes and (name of Group B cargoes)”

      - In case where a survey onboard the ship is required, expected date and place of the survey and local agent tobe contacted.

      - A list of documents submitted together with the application and of those expected to be submitted

later, if any.

    - In case where dangerous goods are included in the cargoes, the applicant should also apply for the issue of a certificate of compliance with the requirements of SOLAS74 Reg.II-2/54 (Reg.II-2/19 under SOLAS 2000) as necessary.

    - In case where a conversion or an alteration affecting a class requirement including the case mentioned in 2.3.2. above is made, the applicant should also apply for a class survey.

  - Submission of documents

    - In case where the certification is requested for the carriage of Group B cargoes, the applicant should submit thedocuments as shown in Table 2.4 (other than coal and brown coal (lignite) briquettes) and/or Table

2.5 (coal and brown coal (lignite) briquettes). For the specific construction and equipment of existing ships, if the ship´s compliance with the requirements, in the opinion of the surveyor, is capable of being checked by only the survey on board, submission of documents and document examination may be omitted.

    - In case where the certification is requested for the carriage of Group A cargoes without appropriate restrictionson their moisture contents, the applicant should submit relevant structural drawings, stability calculations and other documents considered necessary by GR Class.

  - Document examination, survey and issue of certificate

    - Certification for cargoes other than Group B cargoes

      - For the certification for the carriage of other cargoes than Group B cargoes, either document examination or survey onboard the ship is not required in principle. Head Office or the Surveyor Site’s Office should issue an interim certificate and checklist (if any).

        - In case where any Group A cargo is intended to be carried without appropriate restrictions on its moisture content, GR Technical Department should be contacted as examination of drawings of subdivision is required.

      - The Surveyor Site’s Office should forward a copy of the interim certificate and checklist (if any) to GR Technical Department.

      - For certification for the carriage of cargoes including Group B cargoes (except where Group B cargo is only coal and/or brown coal (lignite) briquettes), (2.5.1.3.1.) and (2.5.1.3.2.) below should be followed.

        - As shown in Table 2.4, the Head Office (Technical Department) or the Surveyor Site’s Office should examine or check the documents. The Technical Department should inform the applicant and the Surveyor Site’s Office of the results of examination, and the Surveyor Site of the time required for examination, if carried out by the Head Office.

        - The Surveyor Site’s Office should:

- based on the above (1) results of examination and the those of examination into the documents which should be submitted to the Surveyor Site’s Office according to table 2.4, carry out a survey on board using Table 2.2 as checklist.

- Finalize and issue an interim certificate (See note – 2) including the attachments and checklist. The checklist should not be attached to the certificate but kept on board for the periodical survey; and

- Forward a copy of the interim certificate and checklist to GR Technical Department via database.

      - For certification for the carriage of cargoes including only coal and/or brown coal (lignite) briquettes as Group B cargo, (2.5.1.4.1) through (2.5.1.4.3) below should be followed.

        - Document examination on the Marine Operations Department (MOD) is not required. The Surveyor Site’s Office should prepare an interim certificate (Refer to Note – 2) and checklist.

        - Based on the results of examination into the documents listed in Table 2.5, the Surveyor Site should carry out a survey on board using Table 2.3 as checklist and issue an interim certificate. The checklist should not be attachedto the certificate but kept on board for the periodical survey.

        - A copy of the Interim certificate and checklist should be forwarded to GR Technical Department.

  - Renewal and rewrite of the certificate

    - Change to IMSBC Code fitness certificate from BC Code fitness certificate Document examination at the Head Office is not required. The Head office or the Surveyor Site’s Office when requested, prepare IMSBC Code fitness certificate, and checklist. A copy of the interim certificate and checklist (if any) should be forwarded to GR Technical Department.

    - Change to IMSBC Code fitness certificate and renewal of the BC Code fitness certificate Document examination at the Head Office is not required. The Surveyor Site’s Office should prepare IMSBC Code fitness interim certificate and the checklist. The survey on board should be carried out using the before-mentioned checklist, and certificate should be finalized and issued. The checklist should not be attached to the certificate but kept on board for the periodical survey. A copy of the interim certificate and checklist (if any) should be forwarded to GR Technical Department. (Refer to note

-2)

  - Renewal of IMSBC code fitness certificate

    - Document examination at the head office is not required. The Surveyor Site’s office should carry out a survey usingthe checklist kept on board and based on the results of the survey, prepare an interim certificate, valid for 5 months. A copy of the interim certificate should be forwarded to GR Technical Department. (Refer to note – 2)

    - Rewrite of IMSBC code fitness certificate due to flag change or change of ship´s name.

    - Document examination at the Head Office is not required. The Surveyor Site’s Office should carry out a survey using the checklist kept on board and based on the results of the survey, prepare an interim certificate, (Refer to Note – 2). In case where the expiry date of new certificate is not changed from the previous one, the survey is not required. A copy of the interim certificate should be forwarded to GR Technical Department.

Note – 2: Interim certificate will be issued valid until 5 months. When survey is incomplete or deficiencies are noted, the interim Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be entered on the “Survey Statement” into GR CLASS MIS. The certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two (2) months (except where only approval of mandatory manual/plans/booklet remain outstanding, in which case the validity should be restricted to three (3) months). A copy of the CONDITIONAL certificate is to be sent to GR Head Officevia database. Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

    - The period of validity of the IMSBC Full Term certificate should be five (5) years after the date of

issued.

Note – 3: Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. In case of conflict, the surveyor should follow the instruction issued by the flag state that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

# REFERENCE

  - IMSBC Code

  - Class Rules and Regulations

  - www.imo.org

  - www.ilo.org

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 214: Mandatory Implementation of the Amendments to the IMSBC Code. | May 2020 | https://panamashipregistry.com/circulars/ |
| Barbados |  |  | https://barbadosmaritime.org/documents/bulletins/ |
| Sierra Leone |  |  | Maritime Circulars - Sierra Leone Maritime Administration (slmarad.com) |
| Vanuatu |  |  | Vanuatu Maritime Services Safety Forms (vanuatumaritimeships.com) |
| St. Kitts & Nevis |  |  | Maritime Circulars (skanregistry.com) |
| Nigeria |  |  | https://nimasa.gov.ng/?s=notice |

# RECORD FORMS

  - Form IMSBC-01

Table 2.2. For cargoes other than COAL and BROWN COAL BRIQUETTES

  - Form IMSBC-02

Table 2.3 For COAL and BROWN COAL BRIQUETTES

  - Form IMSBC-03

Table 2.4. Documents/Information to be submitted

  - Form IMSBC-03

Table 2.5. Documents/information to be submitted for COAL/BROWN COAL BRIQUETTES

- APPENDIXES

Appendix 1

Table 2.1. Requirements of construction and equipment for individual cargoes under the provisions of the IMSBC Code and SOLAS Reg.II-2/54.2 (Reg.II-2/19.3 on or after 2000 amendments)

Table 2.1

Requirements of construction and equipment for individual cargoesunder the provisions of the IMSBC Code and SOLAS Reg.II-2/54.2 (Reg.II-2/19.3 on or after 2000 amendments)

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MATERIALS | IMO class | UN No. | Group | Stowage | NO SMOKING sign | Ventilation | SCBA | Protective clothing | Bilge line | Explo sion | Dual purpose nozzles | 4 jets of water | SOLAS Reg.II-2/54.2 or 19.3 |  |  |  |  |  |  |  | FFEA (SOLAS Reg.II- 2/10.7.1.3) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Remote control of fire pump | 4 jets of water | Explosion protected | Mechanical ventilation | Safe type fan | Natural ventilation | Personnel protection | A-60 insulation |  |
| ALFALFA |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALUMINA |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALUMINA, CALCINED |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALUMINA, SILICA |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALUMINA SILICA, pellets |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALUMINIUM FERROSILICON POWDER | 4.3 | 1395 | B | A, G | Yes | ML,Sa | Yes |  |  | IICT2 |  |  |  |  | X | X | X | X | X | X |  |
| ALUMINIUM NITRATE | 5.1 | 1438 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| ALUMINIUM SILICON POWDER, UNCOATED | 4.3 | 1398 | B | A, G | Yes | ML,Sa | Yes |  |  | IICT2 |  |  |  |  | X | X | X | X | X | X |  |
| ALUMINIUM SMELTING BY-PRODUCTS or ALUMINIUM REMELTING BY-PRODUCTS | 4.3 | 3170 | B | A, G | Yes | ML,Sa | Yes |  |  | IICT2 |  |  |  |  | X | X | X | X | X | X |  |
| AMMONIUM NITRATE | 5.1 | 1942 | B | A | Yes |  | Yes | Yes |  | IS |  | Yes | X | X | X |  | X9 | X | X | X | (Yes) |
| AMMONIUM NITRATE BASED FERTILIZER (Type A) | 5.1 | 2067 | B | A | Yes |  | Yes | Yes |  | IS |  | Yes | X | X | X |  | X9 | X | X | X | (Yes) |
| AMMONIUM NITRATE BASED FERTILIZER (Type B) | 9 | 2071 | B | A | Yes |  | Yes | Yes |  | IS |  | Yes | X | X | X |  | X9 | X | X | X | (Yes) |
| AMMONIUM NITRATE, BASED FERTILIZER (non-hazardous) 8 |  |  | C |  | Yes |  | Yes | Yes |  | IS |  | Yes |  |  |  |  |  |  |  |  |  |
| AMMONIUM SULPHATE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTIMONY ORE AND RESIDUE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BARIUM NITRATE | 5.1 | 1446 | B |  |  | Nm | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| BARYTES |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BAUXITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BIOSLUDGE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BORAX, ANHYDROUS, crude or refined |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BORAX (PENTAHYDRATE CRUDE) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BROWN COAL BRIQUETTES | MHB |  | B | See Table 2.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CALCIUM NITRATE | 5.1 | 1454 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| CALCIUM NITRATE FERTILIZER |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CARBORUNDUM |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CASTOR BEANS 1 | 9 | 2969 | B |  |  | Nm | Yes | Yes |  |  | Yes |  | X | X |  |  |  | X | X |  | Yes |
| CEMENT |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CEMENT CLINKERS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CEMENT COPPER |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

pump

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MATERIALS | IMO class | UN No. | Group | Stowage | NO SMOKING sign | Ventilation | SCBA | Protective clothing | Bilge line | Explosion protected | Dual purpose nozzles | 4 jets of water | SOLAS Reg.II-2/54.2 or 19.3 |  |  |  |  |  |  |  | FFEA (SOLAS Reg.II- 2/10.7.1.3) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Remote control of fire | 4 jets of water | Explosion protected electrical | Mechanical ventilation | Safe type fan | Natural ventilation | Personnel protection | A-60 insulation |  |
| CHAMOTTE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHARCOAL | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| CHOPPED RUBBER AND PLASTIC INSULATION |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes 2 |
| CHROME PELLETS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHROMITE ORE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CLAY |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COAL |  |  | A and B | See Table 2.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COAL SLURRY |  |  | A |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COARSE CHOPPED TYRES |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes 2 |
| COKE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COKE BREEZE |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COLEMANITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COPPER CONCENTRATE |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COPPER GRANULES |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COPPER MATTE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COPRA (dry) | 4.2 | 1363 | B | A | Yes | Nm |  |  |  |  |  |  | X | X |  |  |  | X | X | X | Yes |
| CRYOLITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAMMONIUM PHOSPHATE (D.A.P.) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIRECT REDUCED IRON,(A) Briquettes, hot-moulded | MHB |  | B | F | Yes | Nm, Sp |  |  |  | IICT2 |  |  |  |  |  |  |  |  |  |  |  |
| DIRECT REDUCED IRON, (B) Lumps, pellets, cold moulded briquettes 3 | MHB |  | B | F | Yes |  |  |  |  | IICT2 |  |  |  |  |  |  |  |  |  |  | Yes |
| DIRECT REDUCED IRON, (C) (By product fines) 3 | MHB |  | B | F | Yes |  | Yes |  |  | IICT2 |  |  |  |  |  |  |  |  |  |  | Yes |
| DOLOMITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FELSPAR LUMP |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERROCHROME |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERROCHROME, exothermic |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERROMANGANESE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERRONICKEL |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERROPHOSPHORUS (including briquettes) | MHB |  | B |  |  | M, Sa | Yes |  |  | IICT1 |  |  |  |  |  |  |  |  |  |  |  |
| FERROSILICON with 30% or more but less than 90% silicon (including briquettes) | 4.3 | 1408 | B | A, G | Yes | ML.Sa | Yes | Yes | F.N | IICT1 |  |  |  |  | X | X | X | X | X | X |  |

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MATERIALS | IMO class | UN No. | Group | Stowage | NO SMOKING sign | Ventilation | SCBA | Protective clothing | Bilge line | Explosi on | Dual purpose nozzles | 4 jets of water | SOLAS Reg.II-2/54.2 or 19.3 |  |  |  |  |  |  |  | FFEA (SOLAS Reg.II- 2/10.7.1.3) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Remote control of fire pump | 4 jets of water | Explosion protected | Mechanical ventilation | Safe type fan | Natural ventilation | Personnel protection | A-60 insulation |  |
| (Including briquettes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FERROUS M E T A L BORINGS, SHAVINGS, TURNINGS or CUTTINGS | 4.2 | 2793 | B | A | Yes |  | Yes |  |  |  |  |  | X | X |  |  |  | X | X | X | Yes |
| FERTILIZERS WITHOUT NITRATES (non-hazardous) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FISH (IN BULK) |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FISHMEAL (FISHSCRAP), STABILIZED | 9 | 2216 | B |  |  | Nm | Yes |  |  |  |  |  | X | X |  |  |  | X | X |  | Yes |
| FLUORSPAR | MHB |  | A and B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FLY ASH |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GRANULATED SLAG |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GRANULATE TYPE RUBBER |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes 2 |
| GYPSUM |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ILMENITE CLAY |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ILMENITE SAND |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRON ORE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRON ORE PELLETS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRON OXIDE, SPENT or IRON SPONGE, SPENT | 4.2 | 1376 | B | A |  | Nm | Yes | Yes |  | IIAT2 | Yes |  | X | X |  |  |  | X | X | X | Yes |
| IRONSTONE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LABRADORITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEAD NITRATE | 5.1 | 1469 | B |  |  | N | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| LEAD ORE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LIME (UNSLAKED) | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LIMESTONE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LINTED COTTON SEED | MHB |  | B |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| MAGNESIA (DEADBURNED) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAGNESIA (UNSLAKED) | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAGNESITE, natural |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAGNESIUM NITRATE | 5.1 | 1474 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| MANGANESE ORE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MARBLE CHIPS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| METAL SULPHIDE CONCENTRATES | MHB |  | A and B |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Mineral Concentrates |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MONOAMMONIUM PHOSPHATE (M.A.P.) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PEANUTS (in shell) |  |  | C | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Approved by: GM	 		Page 12 of 17

pump

| PEAT MOSS | MHB |  | A and B |  |  | Nm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MATERIALS | IMO class | UN No. | Group | Stowage | NO SMOKING sign | Ventilation | SCBA | Protective clothing | Bilge line | Explosi on | Dual purpose nozzles | 4 jets of water | SOLAS Reg.II-2/54.2 or 19.3 |  |  |  |  |  |  |  | FFEA (SOLAS Reg.II- 2/10.7.1.3) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Remote control of fire | 4 jets of water | Explosion protected | Mechanical ventilation | Safe type fan | Natural ventilation | Personnel protection | A-60 insulation |  |
| PEBBLES (sea) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PELLETS (concentrates) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERLITE ROCK |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PETROLEUM COKE, calcined or uncalcined | MHB |  | B |  |  |  | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| PHOSPHATE, defluorinated |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHOSPHATE ROCK, calcined |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHOSPHATE ROCK, uncalcined |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PIG IRON |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PITCH PRILL | MHB |  | B |  |  | Nm | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| POTASH |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POTASSIUM CHLORIDE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POTASSIUM NITRATE | 5.1 | 1486 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| POTASSIUM SULPHATE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUMICE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PYRITE (containing copper and iron) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PYRITES, CALCINED (Calcined Pyrites) | MHB |  | A and B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PYROPHYLLITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QUARTZ |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QUARTZITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-1) | 7 | 2912 | B |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-1) | 7 | 2913 | B |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RASORITE (ANHYDROUS) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RUTILE SAND |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SALT |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SALT CAKE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SALT ROCK |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SAND |  |  | C | A 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SAWDUST | MHB |  | B |  |  | Nm |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| SCRAP METAL |  |  | C |  |  | Nm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEED CAKE Type (a) | 4.2 | 1386 | B | A |  |  | Yes |  |  |  |  |  | X | X |  |  |  | X | X | X | Yes |
| SEED CAKE Type (b) | 4.2 | 1386 | B | A 5 | Yes | Nm, Sp | Yes |  |  | IIAT3 5 |  |  | X | X | X5 | X5 | X5 | X | X | X | Yes |
| SEED CAKE | 4.2 | 2217 | B | A | Yes | Nm, Sp | Yes |  |  | IIAT3 |  |  | X | X | X | X | X | X | X | X | Yes |
| SEED CAKE (non-hazardous) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Approved by: GM	                                         	                                                                                                                  Page 13 of 17

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MATERIALS | IMO class | UN No. | Group | Stowage | NO SMOKING sign | Ventilation | SCBA | Protective clothing | Bilge line | Explosion protected electrical | Dual purpose nozzles | 4 jets of water | SOLAS Reg.II-2/54.2 or 19.3 |  |  |  |  |  |  |  | FFEA (SOLAS Reg.II-2/10.7.1.3) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Remote control of fire pump | 4 jets of water | Explosion protected electrical equipment | Mechanical ventilation | Safe type fan | Natural ventilation | Personnel protection | A-60 insulation |  |
| SILICOMANGANESE (low carbon) (with known hazard profile or known to evolve gases) (with silicon content of 25% or more) | MHB |  | B |  | Yes | M, Sa | Yes |  |  | IICT1 |  |  |  |  |  |  |  |  |  |  |  |
| SODA ASH |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SODIUM NITRATE | 5.1 | 1498 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| SODIUM NITRATE AND POTASSIUM NITRATE MIXTURE | 5.1 | 1499 | B |  |  |  | Yes | Yes |  |  | Yes | Yes | X | X |  |  |  | X | X |  | (Yes) |
| STAINLESS STEEL GRINDING DUST |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STONE CHIPPINGS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUGAR |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SULPHATE OF POTASH AND MAGNESIUM |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SULPHUR (crushed lump and coarse grained) 6 | 4.1 | 1350 | B | A | Yes | Nm, Sp | Yes |  |  | IIAT4 |  |  | X | X | X |  | X9 | X | X | X |  |
| SULPHUR (formed, solid) |  |  | C |  |  | Nm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUPERPHOSPHATE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUPERPHOSPHATE (triple, granular) |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TACONITE PELLETS |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TALC |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TANKAGE | MHB |  | B |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| TAPIOCA |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UREA |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VANADIUM ORE | MHB |  | B |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VERMICULITE |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WHITE QUARTZ |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WOODCHIPS | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes 7 |
| WOOD PELLETS | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |
| WOOD PULP PELLETS | MHB |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes 7 |
| ZINC ASHES | 4.3 | 1435 | B | A | Yes | ML,Sa | Yes | Yes |  | IICT2 |  |  |  |  | X | X | X | X | X | X |  |
| ZIRCON SAND |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Approved by: GM	                                     		                                                                                                                    Page 14 of 17

The contents of each column in the Table G1 are as follows.

- MATERIALS (column “a”)

Bulk Cargo Shipping Names are expressed in capital letters and identifies a bulk cargo during transport by sea.

- IMO class (column “b”)

Group B cargoes are categorized into the following classes.

  - Class 4.1: Flammable solids

  - Class 4.2: Substances liable to spontaneous combustion

  - Class 4.3: Substances which, in contact with water, emit flammable gases

  - Class 5.1: Oxidizing substances (agents)

  - Class 7: Radioactive materials

  - Class 9: Miscellaneous dangerous substances and articles

  - MHB: Materials which may possess chemical hazards when transported in bulk other than materials classified as dangerous goods in the IMDG Code.

- UN No. (column “c”)

This is a 4-digit number assigned to a particular dangerous substance included in the dangerous substance list (approximately 3,000 items) within the United Nations Recommendations on the Transport of Dangerous Goods issued by the Unite Nations Committee of Experts on the Transport of Dangerous Goods.

- Group (column “d”)

  - A: Group A consists of cargoes which may liquefy if shipped at moisture content in excess of their transportable moisture limit.

  - B: Group B consists of cargoes which possess a chemical hazard which could give rise to a dangerous situation on a ship.

  - C: Group C consists of cargoes which are neither liable to liquefy (Group A) nor to possess chemical hazards (Group B).

- Stowage (column “e”)

  - A: Bulkheads to the engine room are to be insulated to A-60 standard.

  - F: Boundaries of components are to be resistant to fire and passage of water.

  - G: Bulkheads to the engine room are to be of gastight.

- NO SMOKING sign (column “f”)

  - Yes: “NO SMOKING” signs are to be posted on decks and in areas adjacent to cargo compartments.

- Ventilation (column “g”)

  - N: Natural ventilation system is to be provided for cargo holds.

  - Nm: Natural or mechanical ventilation system is to be provided for cargo holds. M : Mechanical ventilation system is to be provided for cargo holds.

  - ML: At least two mechanical ventilation fans are to be provided for cargo holds. The total ventilation is to be at least six air changes per hour.

  - Ventilation openings are to comply with the requirements of the Load Line Convention as amended for

openings not fitted with means of closure.

  - Sa: Ventilation fans are to be safe for use in a flammable atmosphere.

  - Sp: Spark-arresting screens (wire mesh guards with max. 13mm X 13mm) are to be fitted to ventilation openings.

- SCBA (column “h”)

  - Yes: Two self-contained breathing apparatuses with 200% spare cylinders are to be additionally provided.

- Protective clothing resistant to chemical attack (column “i”)

  - Yes: Four sets of protective clothing which consists of a pair of gloves, boots, a protective clothing and helmet with goggles are to be additionally provided.

- Bilge line (column “j”)

  - F: In case where bilge lines are led to machinery space, bilge line is to be isolated either by fitting a blank flange or by a closed lockable valve.

  - N: A notice is to be placed adjacent to the valve warning against opening without the master’s permission.

- Electrical equipment (column “k”)

  - Not suitable explosion protected type electrical equipment are to be disconnected (by removal of links in the system, other than fuses) from the power source at a point external to the space.

  - IIAT2: Electrical equipment having an explosion protection grade of IIAT2 or upwards are considered as suitable explosion protected type electrical equipment.

  - IIAT3: Electrical equipment having an explosion protection grade of IIAT3 or upwards are considered as suitable explosion protected type electrical equipment.

  - IIAT4: Electrical equipment having an explosion protection grade of IIAT4 or upwards are considered as suitable explosion protected type electrical equipment.

  - IICT1: Electrical equipment having an explosion protection grade of IICT1 or upwards are considered as suitable explosion protected type electrical equipment.

  - IICT2: Electrical equipment having an explosion protection grade of IICT2 or upwards are considered as suitable explosion protected type electrical equipment.

  - IS: Intrinsically safe type electrical equipment is considered as suitable explosion protected type electrical equipment.

- Dual purpose nozzles (column “l”)

  - Yes: Nozzles provided with fire hoses are to be of dual-purpose type (i.e., spray/jet type).

- 4 jets of water (column “m”)

  - Yes: The quantity of water delivered is to be capable of supplying four nozzles at pressure as specified in SOLAS regulation and being trained on any part of the cargo space when empty.

- Requirements of SOLAS Reg.II-2/54.2 (Reg.II-2/19.3 on or after 2000 amendments) (column “n” ～ ”u”) X: Applicable.

- FFEA (SOLAS Reg.II-2/10.7.1.3) (column “v”)

  - Yes: Fixed CO2 fire extinguishing system for cargo holds are required by SOLAS Reg.II-2/10.7.1.3.

(Yes): Fixed gas fire-extinguishing system is ineffective and for which a fixed fire-extinguishing system giving equivalent protection shall be available. According to the Unified Interpretation of IMO, water supplies defined in SOLAS Reg.II-2/19.3.1.2 are considered as the alternative of a fixed gas fire-extinguishing system in cargo spaces.

General notes:

- For the detailed requirements of the IMSBC Code, the relevant part of the Code should be referred to.

- The application of the requirements of SOLAS Reg.II-2/54.2 or 19.3 is shown just for ready reference.

- For the detailed requirements, the relevant part of the SOLAS should be referred to.

- Blank columns mean “Not applicable”.

Notes:

- CASTER MEAL, CASTER POMACE and CASTER FLAKE shall not be carried in bulk.

- For the planned voyage not exceeding 5 days from the commencement of loading to the completion of discharge, the vessel may be exempted from the requirements of FFEA.

- Consideration shall be given to providing the vessel with the means to top up the cargo spaces with additional supplies of inert gas taking into account the duration of the voyage. The ship’s fixed CO2 fire extinguishing system shall not be used for this purpose.

- Only applicable to Industrial sand coated with resin.

- Only applicable to seed cake containing solvent extractions only.

- Fine grained sulphur (flowers of Sulphur) shall not be transported in bulk.

- With moisture content of 15% or more, the vessel may be exempted from the requirements of FFEA.

- To be stowed out of direct contact with a metal engine room boundary.

- Only suitable wire mesh guards are required.