# SECTION I

## General

- Carriage of chemicals in bulk is covered by regulations in SOLAS Chapter VII - Carriage of dangerous goods and MARPOL Annex II - Regulations for the Control of Pollution by Noxious Liquid Substances in Bulk. Both Conventions require chemical tankers built after 1 July 1986 to comply with the International Code for the Construction and Equipment of Ships carrying Dangerous Chemicals in Bulk (IBC Code).

- The IBC Code provides an international standard for the safe carriage by sea of dangerous and noxious liquid chemicals in bulk. To minimize the risks to ships, their crews and the environment, the Code prescribes the design and construction standards of ships and the equipment they should carry, with due regard to the nature of the products involved. In December 1985, by resolution MEPC.19(22), the Code was extended to cover marine pollution aspects and applies to ships built after 1 July 1986.

- In October 2004, IMO adopted revised MARPOL Annex II Regulations for the control of pollution by noxious liquid substances in bulk. This incorporates a four-category categorization system for noxious and liquid substances, and it entered into force on 1 January 2007.

- Consequential amendments to the International Bulk Chemical Code (IBC Code) were also adopted in October 2004, reflecting the changes to MARPOL Annex II. The amendments incorporate revisions to the categorization of certain products relating to their properties as potential marine pollutants as well as revisions to ship type and carriage requirements following their evaluation by the Evaluation of Hazardous Substances Working Group.

- Ships constructed after 1986 carrying substances identified in chapter 17 of the IBC Code must follow the requirements for design, construction, equipment, and operation of ships contained in the Code.

- Ships subject to the Code shall be designed to one of the following standards:

  - A type 1 ship is a chemical tanker intended to transport chapter 17 products with very severe environmental and safety hazards which require maximum preventive measures to preclude an escape of such cargo.

  - A type 2 ship is a chemical tanker intended to transport chapter 17 products with appreciably severe environmental and safety hazards which require significant preventive measures to preclude an escape of such cargo.

  - A type 3 ship is a chemical tanker intended to transport chapter 17 products with sufficiently severe environmental and safety hazards which require a moderate degree of containment to increase survival capability in a damaged condition.

- Thus, a type 1 ship is a chemical tanker intended for the transportation of products considered to present the greatest overall hazard and type 2 and type 3 for products of progressively lesser hazards. Accordingly, a type 1 ship shall survive the most severe standard of damage and its cargo tanks shall be located at the maximum prescribed distance inboard from the shell plating.

  - Surveyors conducting Chemical Code surveys are to be familiar with the applicable provisions of the International Code for the Construction and Equipment of Ships Carrying Dangerous Chemicals in Bulk

(IBC Code), the Code for the Construction and Equipment for Ships Carrying Dangerous Chemicals in Bulk (BCH Code) and Annex II of the International Convention for the Prevention of Pollution from ships 1973 and the Protocol of 1978 relating thereto.

  - Ships for which keel was laid before 1st July 1986 are subject to provisions of BCH code while ships constructed on or after 1st July 1986 are subject to the provisions of the IBC code. All chemical tankers carrying products of pollution category X, Y, Z and other substances must comply with the provisions of Marpol 73/78 Annex II. Chemical Code and Annex II surveys are integral and the Certificate of Fitness covers both aspects.

  - Renewal surveys are required every five years before the certificate in force has expired. Four annual surveys and one intermediate survey are required during the duration of the certificate. Annual surveys are to be held within three months before and three months after the anniversary due date. Intermediate survey should be held within the three months before and after the due date of the second or third anniversary date, then this survey can take the place of that particular annual survey.

  - The surveys should be held within the permitted range dates only.

  - The existing certificate onboard should be sighted to establish the category of survey due.

  - The various arrangements of Chemical Code equipment to be surveyed will be identified in the Chemical Code Record of Construction (IBC/BCH Code) onboard and various items of Marpol 73/78 Annex II equipment to be surveyed will be identified in the “Procedures and Arrangements” (P&A) manual on board. At each survey, the survey checklist should be completed in conjunction with the Record of Construction and P&A manual and a copy submitted to GR Head Office along with survey reports.

  - Where new installations or major modifications are proposed, plans and P&A manuals should be submitted to GR Head Office for approval before the work is commenced.

  - During the first survey by GR Class – CLASSIFIED FOR STANDAR (GR) onboard, as part of the transfer of class and certification procedure, it should be verified whether the ship already has an existing certificate. If the vessel is already in possession of a certificate issued by a signatory Flag State or Administration, or a Recognized Organization (RO) duly authorized by the same Flag State or Administration, the certificate should be sighted to establish whether any survey is due or overdue. The relevant due/overdue survey is to be carried out. If no survey is due, a general examination, equivalent, to an annual survey is to be carried out. The P&A manual may be accepted if it is approved by a major society or the flag administration. In such cases, the manual must be endorsed “This manual been accepted by GR Class – CLASSIFIED FOR STANDAR (GR) on behalf of “flag state” on the basis of previous approval by “name of society”. A copy of the endorsed page, copy of any approval letter by the previous society and a copy of the full-term certificate along with record of equipment should be forwarded to GR Head Office along with survey reports. If a previous certificate is not available or

not issued by a signatory Flag State or Administration or a Recognized Organization (RO) duly authorized by the same flag state or Administration, then a complete renewal survey should be carried out. The owner should be advised to submit the P&A manual for approval and place a copy on board. The copy on board is to be endorsed “provisionally accepted pending approval by GR CLASS CLASSIFIED FOR STANDARD (GR CLASS) GR Head Office”. The validity of the interim certificate issued is to be for three (3) months and the certificate should be marked CONDITIONAL” with the approval of the P&A manual endorsed as an outstanding item. Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

  - On completion of the renewal surveys or the initial survey by GR CLASS CLASSIFIED FOR STANDARD (GR CLASS), INTERIM OR PROVISIONAL e-certificate is to be issued with validity of five (5) months. GR Head Office will issue the full term certificate after review of the survey report. On completion of annual or intermediate surveys, the certificate on board should be endorsed.

# SECTION 1.1 - AMENDMENDS

    - Resolution MEPC.315(74) dated May 17, 2019 related to amendments to MARPOL Annex II - Cargo residues and tank washing of persistent floating products.

    - Resolution MEPC.318(74) dated May 17, 2019 and Resolution MSC.460 (101) dated June 14, 2019, related to amendments IBC Code Chapter 1 - Definitions, Chapter 15 - Special requirements, Chapter 16 - Operational requirements, Chapter 17 - summary of minimum requirements, Chapter 18 – List of products to which the Code does not apply, Chapter 19 – Index of Products Carried in Bulk, Chapter 21 – Criteria for assigning carriage requirements for products subject to IBC Code,

    - Resolution MEPC.319(74) dated May 17, 2019, and Resolution MSC.463(101) related to BCH Code, Chapter IV, V, VI – Special, operational and minimum requirements.

    - The amendments introduce new requirements as a result of the assessment of products. Amendments which applies to new and existing ships will take effect on January 1, 2021, therefore chemical tankers and offshore supply vessels and other vessels carrying Noxious Liquid Substances shall be Certified in compliance with the new amendments

    - As the amendments introduce changes of ship type, tank type, other carriage requirements for some cargoes, and requirements for pre-washing for some products. Therefore, the range of products a ship is certified to carry will differ before and after January 1, 2021.

    - The new Certificate of Fitness based on the new amendments shall be issued before the entry into force date of the amendments (must be on board from January 1, 2021) and kept on board with the existing certificate. New Certificates will have the same expiry date as the existing Certificate and is effective, and supersedes the existing certificate, at the entry into force of the amendments (on January 1, 2021).

    - Offshore supply vessels with Certificates of Fitness in compliance with the “LHNS Guidelines” - IMO Resolution A.673(16) or “OSV Chemical Code” - IMO Resolution A.1122(30), are also subject to these amendments.

# SECTION 2

## Annual Surveys

  - The purpose of the annual survey is to carry out an examination of the various items of equipment together with tests as considered necessary, all to an extent which, in the Surveyor’s judgment, indicates adequate condition and standard of maintenance until the next due survey.

  - An annual survey within 3 months before or after each anniversary date of the Certificate including a general inspection of the structure, equipment, systems, fittings, arrangements, and material to ensure that they have been maintained and that they remain satisfactory for the service for which the ship is intended.

  - The Surveyors should verify that the certificate and P&A manual on board are in order. The P&A manual should be written in a language understood by the ship’ personnel engaged in the operations described therein, and it is the Owner’s responsibility to provide a copy written in an appropriate language. Similarly, system operating instructions and on board maintenance manuals etc., should also be in a language understood by the crew.

  - The owner’s representative must confirm that no unapproved modifications have been carried out to

the pollution prevention arrangements and equipment since the last survey.

  - The stringency of the survey should be to the Surveyor’s satisfaction and in general will depend upon the condition of the equipment and the level of maintenance.

  - Surveys in respect of the safety considerations of the chemical codes are to be carried out in conjunction with and in accordance with such procedures and instructions as laid down for the classification surveys of chemical tankers.

  - For fire extinguishing arrangements, the relevant section under safety equipment surveys should be referred to. Care should be taken to ensure that when foam concentrate for fixed foam fire extinguishing systems is being renewed, the correct type of foam concentrate suitable for the cargoes on the approved cargo lists should be used. Filter type respiratory protection is not acceptable and where required, the emergency escape breathing apparatus must be of the self-contained type.

# SECTION 3

## Intermediate Surveys

  - An intermediate survey should, in addition to the requirements of an annual survey, be a thorough examination and be sufficiently extensive to ensure that the equipment and the associated piping and pumping systems, including any efficient stripping and line blowing arrangements, and tank washing

systems as applicable, comply with the requirements of the Regulations of Chemical Code and Marpol 73/78 Annex II, are in accordance with the P&A manual and are in good working order sufficient until the next renewal survey.

# SECTION 4

## Renewal Surveys

  - The renewal survey should be a thorough and complete examination with efficient stripping tests and other tests where appropriate. On completion the equipment, systems, fittings and arrangements should, given adequate routine maintenance, be expected to remain in good condition and working order until the next renewal survey. At renewal surveys of chemical tankers having products of pollution category should carry out practical efficient stripping tests using water to verify the continued efficiency of the stripping arrangements in at least two cargo tanks. The tanks should be selected at random and the Surveyor may require systems in other tanks to be tested. The tests are to be carried out in accordance with the procedures described in the P&A manual and should also include any cargo pipeline purging arrangements and the measurement of residues remaining in the lines.

# SECTION 5

## Reporting

  - The relevant survey checklist Form No. IBC.BCH-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports.

  - At first surveys by GR, as part of the transfer of class and certification procedure, copies of the previous society’s certificates, record of equipment, if any, and copy of approval for P&A manual and list of any outstanding deficiencies, recommendations etc should also be forwarded.

  - Interim IBC or BCH Certificate should be issued, valid for five (5) months on satisfactory completion of Renewal and initial Surveys or First Survey by GR CLASS CLASSIFIED FOR STANDARD (GR CLASS) under transfer of class and certification. When survey is incomplete or deficiencies are noted, the Interim Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the CONDITIONAL Certificate and on the Statement Report (former P24).

  - On completion of annual and intermediate surveys, the existing full-term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR seal. A copy of the endorsed certificate must be submitted to GR Head Office along with the Form No. IBC.BCH-01. If during surveys, it is observed that items required for a specific group of products are defective in some tanks, it may be possible, with the agreement of the Owners, to issue or endorse a Certificate of Fitness, but to prohibit the carriage of these products from the tanks in question, until repairs are affected (for example products requiring overflow control could be excluded from tanks with defective high level alarms). GR Head Office should be contacted for advice on these cases.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa. and Barbados Maritime In case of conflict, the surveyor should follow the instruction issued by the flag state that GR is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

REFERENCES

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021 | 06/2022 | https://panamashipregistry.com/circular- category/circulares/ |
|  | MMC 147: Acceptance of the Exemption provision for the Vegetable Oils in the Revised MARPOL Annex II and IBC/BCH Code as amended | 12/2006 |  |
|  | MMC 200: Revision of the IBC/BCH Code and MARPOL 73/78 Annex II | 09/2020 |  |
| Barbados | Bulletin 331: Getting Ready for the Upcoming IBC Code & MARPOL Amendments | 07/2020 | https://barbadosmaritime.org/bulletins/ |
| Belize | NA | NA | https://www.immarbe.com/circularslist/ |
| Sierra Leone | NA | NA | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentation /instruments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disposici ones |
| Nigeria | NA | NA | https://nimasa.gov.ng/press-center/marine- notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |