# SECTION I

## General

  - Surveyors conducting Gas Code surveys are to be familiar with the applicable provisions of the International Code for the Construction of Ships Carrying Liquefied Gases in Bulk (IGC Code), the Code for the Construction of Ships Carrying Liquefied gases in Bulk (Gas Carrier Code) and the Code for Existing Ships carrying Liquefied Gases in Bulk (existing Gas Code) as applicable.

  - Ships for which keel was laid before 1st July 1986 are subject to provisions of Gas Carrier Code or Existing Gas Code as applicable, while ships constructed on or after 1st July 1986 are subject to the provisions of the IGC code, as amended. Gas ships carrying products subject to the provisions of Marpol 73/78 Annex II are to be provided with an NLS certificate. The pollution survey is not an integral part of the gas code survey.

  - An initial survey should be held before the ship is put in service and should include a complete inspection of all the items relating to Fitness for the Carriage of Liquefied Gases in Bulk certificate, to ensure that the relevant requirements are complied with and that these items are satisfactory for the service for which the ship is intended. During Initial survey Form IGC-01 must be used and reported to GR HO. Renewal surveys are required every five years before the certificate in force has expired. Four annual surveys and one intermediate survey are required during the duration of the certificate, as per HSSC (Harmonized System of Survey and Certification). Annual surveys are to be held within three months before and three months after the due anniversary date.

  - If the intermediate survey is held within the three months before and after the due date of the second or third anniversary date and should take the place of one of annual surveys

  - The surveys should be held within the permitted range dates only.

  - The existing certificate onboard should be sighted to establish the category of survey due.

  - At each survey, the survey checklist should be completed, and a copy submitted to GR Head Office along with survey reports.

  - Where new installations or major modifications are proposed, plans should be submitted to GR Head Office for approval before the work is commenced.

  - During the first survey by GR Class Classified for Standard (GR CLASS) onboard, under the transfer of class and certification, it should be verified whether the ship already has an existing certificate. If the vessel is already in possession of a certificate issued by IACAS member or the national administration or, a Recognized Organization (RO) duly authorized by the same flag state, the certificate should be sighted to establish whether any survey is due or overdue. The relevant due/overdue survey is to be carried out. If no survey is due, a general examination, equivalent, to an annual survey is to be carried out.

  - If a previous certificate is not available or not issued by a major society or not issued by a major society, or the flag administration or, a RO duly recognized by the same flag state then a complete renewal

survey should be carried out.

  - On completion of the renewal surveys or the first survey by GR Class- Classified for Standard (GR CLASS), under the transfer of class and certification procedure, an INTERIM certificate is to be issued with validity of five (5) months. GR Head Office will issue the full-term certificate after review of the survey report. On completion of annual or intermediate surveys, the certificate on board should be endorsed.

# RESOLUTION MSC.492(104) – JANUARY 1, 2024

  - After January 1, 2021, the final waterline after flooding due to damage specified by the IGC Code is not be above the lower edge of any opening through which progressive down flooding takes place. This amendment expands the current exclusion provisions by including three specific doors that may now be permitted to be submerged after flooding: 1) remotely operated sliding watertight doors; 2) hinged watertight access doors of the quick-acting or single-action type with open/closed indication locally and at the navigation bridge; and 3) hinged watertight doors that are permanently closed at sea

  - In additions, the IGC Code amendments extend the cross-weld tensile strength to materials such as aluminum alloys

# SECTION 2

## Annual Surveys

  - The purpose of the annual survey is to carry out an examination of the various items of equipment together with tests as considered necessary, all to an extent which, in the Surveyor’s judgment, indicates adequate condition and standard of maintenance until the next due survey.

  - The Surveyors should verify that the certificate of Fitness on board is in order.

  - The owner’s representative must confirm that no unapproved modifications have been carried out to the arrangements and equipment since the last survey.

  - The stringency of the survey should be to the Surveyor’s satisfaction and in general will depend upon the condition of the equipment and the level of maintenance.

  - Surveys in respect of the safety considerations of the gas codes are to be carried out in conjunction with and in accordance with such procedures and instructions as laid down for the classification surveys of gas carriers.

  - For fire extinguishing arrangements, the relevant section under safety equipment surveys should be referred to. Filter type respiratory protection is not acceptable and where required, the emergency escape breathing apparatus must be of the self-contained type.

  - The ship’s cargo hoses should be approved by a major society or a National Administration.

  - It should be verified that spare parts are carried for each type of vent fan provided for enclosed spaces

containing cargo-handling equipment or where cargo-handling operations are performed.

# SECTION 3

## Intermediate Surveys

  - An intermediate survey should, in addition to the requirements of an annual survey, be a thorough examination and be sufficiently extensive to ensure that the equipment and the associated piping and pumping systems, as applicable comply with the requirements of the Regulations and are in good working order sufficient until the next renewal survey.

# SECTION 4

## Renewal Surveys

  - The renewal survey should be a thorough and complete examination with tests as appropriate. On completion the equipment, systems, fittings, and arrangements should, given adequate routine maintenance, be expected to remain in good condition and working order until the next renewal survey.

# SECTION 5

## Reporting

  - The relevant survey checklist Form No. IGC-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. In case of initial surveys, the Form IGC-01 should be used instead and copy submitted to GR Head Office.

  - At first surveys by GR, copies of the previous society’s certificates, record of equipment, if any, and list of any outstanding deficiencies, recommendations etc. should also be forwarded.

  - Interim IGC or GC Certificate should be issued, valid for five (5) months on satisfactory completion of Renewal or initial Surveys or at First Survey by GR Class Classified For  Standard (GR CLASS). When survey is incomplete or deficiencies are noted, the interim Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the CONDITIONAL Certificate and on the Statement Report (previous P24). Conditional certificate can only be issued once received authorization at head Office from the FlagState.

  - On completion of annual surveys, the existing full-term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR seal. A copy of the endorsed certificate must be submitted to GR Head Office along with the Form No. IGC-01. At annual surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the Statement report (previous P24).

  - When previous recommendations are examined and completed satisfactorily, the items must be deletedon the Survey Statement report (previous P24).

  - Survey Statement report (previous P24) should be submitted whenever a survey is incomplete (indicating

the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owner’s proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

# DEFINITIONS

    - IGC Code 1983: means the International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk set out in the Annex to Resolution MSC.5(48) adopted by the Maritime Safety Committee of IMO on June 17, 1983, as amended by Resolutions MSC.17(58), MSC.103(73) and MSC.177(79) adopted by the Committee.

    - IGC Code 1993: means the International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk set out in the Annex to Resolution MSC. 5(48) adopted by the Maritime Safety Committee of IMO on June 17, 1983, as amended by Resolutions MSC.17(58), MSC.30(61), MSC.32(63), MSC.59(67), MSC.103(73), MSC.177(79) and MSC.220(82) adopted by the Committee.

    - IGC Code 2014: means the International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk set out in the Annex to Resolution MSC.370(93) adopted by the Maritime Safety Committee of the IMO on May 22, 2014, as occasionally revised or amended by any revision or amendment

    - IGC Code 2016: means the International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk set out in the Annex to Resolutions MSC.411(97); MSC.441(99); MSC.447(99); adopted by the Maritime Safety Committee of the IMO on May 22, 2014, as occasionally revised or amended by any revision or amendment.

REFERENCE

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 392: International Code of the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk (IGC Code) | 05/2021 | https://panamashipregistry.com/circular- category/circulares/ |
|  | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021 | 06/2022 |  |
| Barbados | NA | NA | https://barbadosmaritime.org/bulletins/ |
| Belize | NA | NA | https://www.immarbe.com/circularslist/ |
| Sierra Leone | NA | NA | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentati on/instruments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/dispos |

|  |  |  | iciones |
| --- | --- | --- | --- |
| Nigeria | NA | NA | https://nimasa.gov.ng/press- center/marine-notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |