GRAIN LOADING MANUAL CHECK LIST

# The booklet must be prepared in accordance with guidance provided by Regulation VI-C.8 of the 1974 Convention for the Safety of Life at Sea (SOLAS).

The IMO International Code for the Safe Carriage of Grain must be used for the approval of the booklet.

| Name of Ship: | Port of Registry: | Gross Tonnage: | Type of Ship: |
| --- | --- | --- | --- |
| Call Letters: | CCC N°: | LL N°: | IMO Number: |
| Name and address of owners: |  | Name and address of operators: |  |
| (1) Indicate year of keel laying or of last major conversion, if applicable. |  |  |  |

| # | DESCRIPTION | YES | NO |
| --- | --- | --- | --- |
| 1 | Is the Plan written in the working language of the crew |  |  |
| 2 | Does the booklet include instructions on the use of the booklet, including a worked example for the guidance of the Master. |  |  |
| 3 | Does the booklet include |  |  |
| A | Ship´s particular |  |  |
| B | Lightweight displacement and the KG position |  |  |
| C | Table of liquid free surfaces corrections |  |  |
| D | Capacities and centres of gravities |  |  |
| E | Curves or tables of angle of flooding, where less than 40º, at all permissible displacements |  |  |
| F | Curves or table of hydrostatic properties suitable for the range of operating drafts. |  |  |
| G | Cross curves of stability which are sufficient for the purpose of the requirements given in GRALO Part A.7 and which include curves of 12º and 40º |  |  |

| # | DESCRIPTION | YES | NO |
| --- | --- | --- | --- |
| H | Curves or tables of volume, vertical center of volumes, and assumed vertical heeling moments for every compartment, filled or partially fulfilled, or combination thereof, including the effects of temporary fittings |  |  |
| I | tables or curves of maximum permissible heeling moments for varying displacements and varying vertical centres of gravity to allow the master to demonstrate compliance with the requirements of GRALO Part A 7.1; this requirement shall apply only to ships the keels of which are laid on or after the entry into force of this Code; |  |  |
| J | Details of the scantlings of any temporary fittings and, where applicable, the provisions necessary to meet the requirements of GRALO A 7, A 8 and A 9 |  |  |
| K | Loading instructions in the form of notes summarizing the requirements of GRALO Code; |  |  |
| M | Typical loaded service departure and arrival conditions and where necessary intermediate worst service conditions. Note: It is recommended that loading conditions be provided for three representative stowage factors, e.g. 1.25, 1.50, and 1.75 cubic metres per tonne. |  |  |
| 4 | The intact stability characteristics of any ship carrying bulk grain shall be shown to meet, throughout the voyage, at least the following criteria after taking into account in the manner described in part B of GRALO Code and, in figure A 7, the heeling moments due to grain shift: Note: Requirements for ships constructed before 1 January 1994 |  |  |
| A | the angle of heel due to the shift of grain shall not be greater than 12° |  |  |
| B | in the statical stability diagram, the net or residual area between the heeling arm curve and the righting arm curve up to the angle of heel of maximum difference between the ordinates of the two curves, or 40° or the angle of flooding (θ1), whichever is the least, shall in all conditions of loading be not less than 0.075 metre-radians; and |  |  |
| C | the initial metacentric height, after correction for the free surface effects of liquids in tanks, shall be not less than 0.30 m. |  |  |
| 5 | The intact stability characteristics of any ship carrying bulk grain shall be shown to meet, throughout the voyage, at least the following criteria after taking into account in the manner described in part B of this Code and, in figure A 7, the heeling moments due to grain shift: |  |  |
| A | the angle of heel due to the shift of grain shall not be greater than 12° or in the case of ships constructed on or after 1 January 1994 the angle at which the deck edge is immersed, whichever is the lesser; |  |  |
| B | in the statical stability diagram, the net or residual area between the heeling arm curve and the righting arm curve up to the angle of heel of maximum difference between the ordinates of the two curves, or 40° or the angle of flooding (θ1), whichever is the least, shall in all conditions of loading be not less than 0.075 metre-radians; and |  |  |
| C | the initial metacentric height, after correction for the free surface effects of liquids in tanks, shall be not less than 0.30 m. |  |  |
| 6 | For “existing ships”: Note: Ships for which the keel was laid before 25 May 1980 |  |  |

| # | DESCRIPTION | YES | NO |
| --- | --- | --- | --- |
| A | If loaded in accordance with documents previously approved under regulation 12 of chapter VI of SOLAS 1960, IMO resolutions A.184(VI) or A.264(VIII) are considered to and there are intact stability characteristics at least equivalent to the requirements of GRALO Part A.7. Documents of authorization permitting such loadings are accepted for the purposes of GRALO Part A 7.2. |  |  |
| B | In case of not having on board a document of authorization issued in accordance with GRALO Part A.3 of this Code, the provisions of Part A.9 without limitation of the deadweight which may be used for the carriage of bulk grain is being applied. |  |  |
| 9 | Does the severe wind and rolling criterion has been used to asses the stability in weather conditions (For ships of 24 m in length and over) |  |  |

Remarks:

The undersigned certifies that the Grain Loading Stability Booklet has been reviewed and it is in compliance with the requirements set forth by applicable regulations as described in Page 1, and that it has been developed according with the applicable guidelines. Furthermore, recommends approval of the booklet on behalf of the Administration.

| Issued at: |  | Date of Issue: |
| --- | --- | --- |
|  | (Place of issue of certificate) | (DD-MM-YYYY) |

# GR CLASS Surveyor Name  - ID No.