# Grain loading Booklet(s)

General Information

A ship engaged in the carriage of grain in bulk shall carry on board at all times its approved grain loading booklet together with the certificate (document of authorization)

The booklet shall be approved by the Administration or an organization recognized by it or by a Contracting Government on behalf of the Administration.

# Applicable Regulation

- For a ship registered in a State which is a Party to the 74 SOLAS, and the ship applying to have the approval of the Grain Loading Booklet on and after 1 January 1994, shall comply with the requirements of “International Grain Code” (IMO Res.MSC23(59)) in accordance with the Regulation 8 & 9 of Chapter VI, 74 SOLAS (91 Amendments).

For a ship loaded in accordance with documents previously approved under Chapter VI of 60 SOLAS, IMO Res. A.184 (VI), IMO Res.A.264 (VIII) or Chapter VI of old 74 SOLAS, it is not necessary to have the grain loading booklet which is based on the International Grain Code.

However, the ship shall carry onboard the approved grain loading booklet and the document of authorization in accordance with the International Grain Code, if so required by the Contracting Government of the country of the port of loading.

Where the nationality of the ship is changed, the grain loading booklet shall be re-approved under the authority of the Government of new flag state, even if the booklet has been previously approved under the former nationality.

# Approval of Grain Loading Booklet

Initially, the grain loading booklet is examined and approved by Head Office.

Certificate for Grain Loading (Document of Authorization) is issued by Head Office pursuant to this approval

# Reference Documents and Information

The following documents shall be submitted to Head Office for the initial approval of grain loading booklet.

- Grain loading booklet

Number of copies to be submitted = 2 + number of copies to be returned. This document shall include the following items.

  - Ship’s particulars

Name of ship, Shipbuilder and hull No; Date of construction (Date of Keel laid, Launching & Delivery),Class Notation, Call letters, Official number, Port of registry, Gross tonnage, Net tonnage, Principal dimensions, Draught corresponding to summer load water line,

Full

displacement	at	summer	load	water	line, Deadweight.

  - Lightship displacement and the center of gravity

  - Guidance for the carriage of grain cargo (loading instructions in the form of notes summarizing the requirements of the regulations, Method of using of date, Method of stability calculation for grain loading, A worked example for the guidance of the master, and others)

  - Curves or tables of volumes, vertical centers of volumes, and assumed volumetric heeling moments for every compartment, filled or partly filled, or combination thereof, including

the effects of temporary fittings

  - Tables listing the capacities and the centers of gravity (LCG, VCG) of each Compartment which may be carried for cargo, fuel oil, drinking water, general service water, ballast

water

  - Tables or curves showing the capacity, center of gravity and the surface effects of liquids in all compartments which may be partly filled. It shall be expressed as the functions of loading depth or ullage of each compartment.

  - all information related shall be upload at GR CLASS DATABASE.

- Tables or curves of maximum permissible heeling moments for varying displacements and varying vertical centers of gravity to allow the master to demonstrate compliance with the requirements of the regulations.

- Typical loaded service departure and arrival conditions and where necessary intermediate worst service conditions

- Table summarizing the calculation of item e) above

- Cross curves of stability which are sufficient for the purpose of the requirements of the regulations and which include curves at 12º and 40º

- Curve or table of the angle of flooding

- Curve or table of the angle of deck edge immersion

- Hydrostatic curves or tables

- Blank Sheet

- Where bagged grain or strapping / lashing is utilized, details of the securing method

- Where the temporary fittings are utilized, details of the arrangement and the scantling of the fittings (Number of copies to be submitted: (Same as -1. above)

- Reference documents (one copy)

  - General arrangement

  - Midship section

  - Lines plan

  - Hatch cover construction

  - Hatch coaming construction

  - Frame body plan or frame offset g) Hold construction

- Result of inclining experiment

- Calculation sheets for -1. d) and g) above

- Other reference plans and data for examination of the booklet, if deemed necessary

# Additional documents and Information for Approval of Dispensation from Trimming

For ships carrying bulk grain cargo without trimming of grain surface in filled compartments, the following drawings and documents shall be submitted in addition to those prescribed in 0103.

- For existing ship having an approved grain loading booklet under the condition of trimming of the surface of bulk grain in filled compartments.

An addendum which includes the following information shall be submitted for approval. Number of copies to be submitted = 2 + number of copies to be returned.

  - Title, Drawing number and approved date of the basic grain loading booklet shall be

specified on the cover sheet of the addendum.,

  - Following notice shall be indicated into the booklet.

    - This addendum has been prepared in accordance with the provision of A10.3 of International Grain code (IMO Res. MSC.23 (59));

    - The data on capacity, KG and volumetric heeling moment furnished in this addendum shall be used for the cargo hold specified when the hold is filled to the top of the main deck coaming with the ends untrimmed;

    - This information is to be used in conjunction with the approved basic grain loading booklet.

  - Tables showing the following items

    - Volumetric heeling moment of each filled compartment, Ends Untrimmed;

    - Volumetric centre of gravity of whole cargo compartment, or centre of gravity of cargo spaces considering the void spaces

    - Net grain capacity (volume of whole cargo compartment – volume of assumed voids in forward and after end of hatch end beams) of each compartment

  - Typical loaded service departure and arrival conditions and where necessary intermediate worst service conditions.

- For new ships

  - Information for dispensation from trimming shall be indicated in a separation chapter of the booklet.

  - The items described in -1. b), c) and d) shall be included.

- Other reference plans and data for examination of the booklet (One copy) As to an existing ship, the following documents shall be submitted.

As to a new ship, -1. c) shall be submitted in addition to the documents described in 0103. a) General arrangement

- Midship section

- Lines plan (including offset tables)

- Hatch cover construction

- Hatch coaming construction

- Frame body plan or frame offset g) Cargo hold construction

h) Calculation sheets for -1. c) above.

| GR CLASS |  |
| --- | --- |
| GR-PM -GRALO | GRALO |

# Chapter 2	Re-approval of Grain Loading Booklet (s) for Flag Change at Representative Office

Application and Re-approval Fee

- The application  or similar is to be submitted by the owner or master of the ship.

- Re-approval fee is to be charged by Head Office.

# Applicable Grain Loading Booklet (hereinafter called “The Booklet”)

The booklet which has been already approved by the Society in accordance with 60 SOLAS, IMO Res.A.184(VI), IMO Res.A.264(VII), 74 SOLAS or International Grain Code (IMO Res.MSC.23(59)) is to be re-approved by the Representative Office.

Note: - The booklet which has not been approved by the Society is to be examined and approved by Technical Department in the Head Office.

# Procedure of Re-approval

- The booklet is to be examined and be confirmed that it has been already approved by the Society in accordance with applicable regulations.

- The “Approved” label is to be affixed on the cover page of the booklet so that it covers the existing approval stamp.

- Approved date, ship’s flag, etc. are to be entered on the “Approved” label with Surveyor’s signature.

- GR Stamp is to be stamped on the boundary of the “Approved” label.

- The ship’s names, flag, etc. in the booklet are to be corrected accordingly and “[GR CLASS]” seal is to be stamped as their respective locations.

- In case that the grain loading with ends untrimmed has been approved by the Society, the same re-approval procedure is to be followed as instructed in above.

It is able to confirm whether the grain loading with ends untrimmed has been approved by the Society or not by the statement in the Document of Authorization (Grain Loading Certificate) in each case as follows.

Case 1: When approved by IMO Res.A.184(VI) or IMO Res.A.264(VII)

“Approved pursuant to: Other (Specify) IMO paper BC XIX /INF.4 dated June 13, 1978”

Case 2: When approved by Chapter VI, 74SOLAS

“Approved pursuant to: Other (Specify): Reg.3(c) of Part A, Chapter VI,SOLAS 1974”

Case 3: When approved by International Grain Code

“Remarks: Dispensation form trimming of (ends of) fully filled holds in granted under

A10.3 (orB6) of International Grain Code (IMO Res.MSC.23(59))”

- If a copy of the approved booklet has been kept in the Head Office, a copy of the re- approved booklet is not necessary to be sent to the Head Office.

| GR CLASS |  |
| --- | --- |
| GR-PM -GRALO | GRALO |

# Reporting

After completion of the re-approval of the booklet, the following documents are to be uploaded to GR CLASS Data baseTechnical Department in the Head Office by facsimile or e-mail. Upon receipt of the documents by facsimile or e-mail, the Head Office issues the “Document of Authorization” (Grain Loading Certificate) and sends it to the Owner or Representative Office.

- Application for Approval of the Grain Loading Plan

- Copy of the Registry Certificate (Final or Provisional)

- Copy of the previous “Document of Authorization” (Grain Loading Certificate)

- Copy of the cover of the booklet re-approved by the Representative Office

# Certificate for Grain Loading (Document of Authorization)

A ship without a certificate for grain loading (Document of Authorization), cannot load the bulk grain cargo, even if the ship has an approved grain loading booklet. Consequently, the relative information for re-approval shall be immediately sent to Head Office for issuance of the certificate. (This certificate is issued by Head Office.)