# SECTION 1

## General

  - The Torremolinos Protocol of 1993 relating to the Torremolinos International Convention for the Safety of Fishing Vessels 1977 applies to fishing vessels including vessels also processing their catch.

  - The Protocol has not yet been adopted by sufficient number of countries and is therefore not internationally mandatory. However owners may request for such certificates where the Port State Control in their trading areas requires compliance. Any such requests should be forwarded to GR Head Office and the Head Office will forward specific instructions in each case.

  - Fishing vessel safety certificates must be issued in accordance with national safety regulation for Fishing Vessels of 24meters in length and above. Panama administration has its own regulation.

  - Certificates are issued with validity of five years with annual surveys required every year. The fishing vessel safety certificate is a combined certificate, which covers items of Safety construction, Safety equipment and Safety radio. Surveyors carrying out fishing vessel safety surveys on Panamanian ships should be familiar with the applicable provision of the Panama Rules for Fishing Vessels.

# SECTION 2

## Initial Survey

  - During the initial survey checklist Form No. FISVEL-01 and Form No. FISVEL-02 must be filled out and submitted to GR Head Office. Other documents are required in order to process the Full Term Certificates, Please refer to TD-DOC-01S, Fishing Vessel Safety Certificate Part.

  - At any case RPT-01 and RPT-02 Reports must be used by the surveyor to inform GR HO if any minor deficiency is found. Deficiencies listed on RPT-01 report, must be discussed with the Captain of the ship, owner or representative in order to have a complete understanding of them, as well as, to be informed about the period of time to be corrected. In the case that Deficiencies are not cleared during the period

allowed by the surveyor, the associated certificate must be withdrawn and authorities as port state control and flag state must be informed.

  - Short term Fishing Vessel Safety Certificate (FISVEL) should be issued after a satisfactory survey has been carried out, with a maximum validity of Five (5) months from the date of survey. Please be advised that Short term Certificate shall not be issued if a non-satisfactory survey completion has been performed.

  - For fishing vessels of 75 meters or more in length installation of GMDSS equipment is required. For that reason a GMDSS survey (CRC) must be performed and forms CRC-01 & CRC-02 must be filled and submitted to GR HO. For survey instructions refer to procedure TD-SP-CRC.

  - For fishing vessel below 75 meters in length a SARCE survey must be performed and forms SARCE-01 & SARCE-02 must be filled and submitted to GR HO.

# SECTION 3

## Annual Survey

  - Annual Survey must be carried out within the specify window (three months before or after the anniversary date). Documents to process Full Term Certificate have been specified in our TD-DOC-1S procedure. At any case RPT-01 and RPT-02 Reports must be used by the surveyor to inform GR HO if any minor deficiency is found. If the vessel is not surveyed within this period, the certificate will be not valid anymore.

  - For fishing vessels of 75 meters or more in length installation of GMDSS equipment is required. For that reason a GMDSS survey (CRC) must be performed and forms CRC-01 & CRC-02 must be filled and submitted to GR HO. For survey instructions refer to procedure TD-SP-CRC.

  - For fishing vessel below 75 meters in length a SARCE survey must be performed and forms SARCE-01 & SARCE-02 must be filled and submitted to GR HO.

# SECTION 4

## Renewal Surveys

  - Renewal surveys are required every 5 years and annual surveys required every year. The survey report (Form No. FISVEL-01) is to be filled up and submitted as applicable for the relevant surveys. Form No. FISVEL-02 should be submitted only if there has been a change on the record from previous survey.

  - For fishing vessels of 75 meters or more in length installation of GMDSS equipment is required. For that reason a GMDSS survey (CRC) must be performed and forms CRC-01 must be filled and submitted to GR HO. For survey instructions refer to procedure TD-SP-CRC.

  - For fishing vessel below 75 meters in length a SARCE survey must be performed and forms SARCE-01 must be filled and submitted to GR HO.

  - On completion of an initial/renewal survey, an SHORT TERM Fishing Vessel Safety certificate is to be issued for five (5) months.

  - GR Head Office will issue the full term certificate after review of the reports. Any deficiencies noted during surveys should be itemized on the Surveyor Site Report (RPT-01), if they are not rectified during the surveys.

# SECTION 5

## Reporting

  - The relevant survey checklists Form No. FISVEL-01, Form No. FISVEL-02 and CRC-01 & CRC-02 or SARCE-01 & SARCE-02 (As applicable) should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports.

  - Short term Fishing Vessel Safety Certificate should be issued, valid for five (5) months on satisfactory completion of Renewal Surveys or First Survey by GR Class Classified For Standard (GR CLASS). When survey is incomplete or deficiencies are noted, the Short term Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the CONDITIONAL Certificate and on the Surveyor Site Report (RPT-01).

  - On completion of annual surveys, the existing full term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR seal. A copy of the

endorsed certificate must be submitted to GR Head Office along with the Form No. FISVEL-01 & CRC-01 or SARCE-01 (As applicable). At annual surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the surveyor site report (RPT-01). The Full Term FISVEL certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the surveyor site report (Form No. RPT-01).

  - A narrative survey report (Form No. RPT-02) should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owner’s proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.