| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-GRB | MARPOL ANNEX V (GARBAGE) SURVEYS |

# SECTION 1

General

  - Marpol 73/78 Annex V contains the regulations for the prevention of pollution by garbage. The Regulations apply to all ships and addresses the disposal of all kinds of victual, domestic and operational waste (excluding fresh fish) generated during the normal operation of the ship and not subject to other parts of the convention.

  - There is no certification required for compliance with Annex V and the disposal of garbage according to the limits set by the Annex is entirely the responsibility of the Owner. However, GR offers the certification process.

  - The storage of garbage on board may cause stowage space problems, which may be alleviated by the use of incinerators, compactors, comminutor’s and other devices, which may also make it possible to discharge certain garbage at sea.

  - When requested by owners, Surveyors may issue an interim e-certificate describing any equipment installed to facilitate judicious compliance with the Annex. There should be a Garbage Management Plan onboard the ship and a copy must be submitted along with survey reports. A garbage record book/log must be maintained and the surveyor, at all surveys, should examine this.

# SECTION 2

Reporting

  - At first surveys by GR, copies of the previous class certificates and copy of approval for garbage management plan and list of any outstanding deficiencies, recommendations etc. should also be forwarded.

  - On completion of renewal surveys or First Survey by GR Class Classified For Standard (GR), a Full-Term e-certificate should be issued.

  - On completion of annual surveys, the full-term e-certificate on board is not endorsed as is not subject for annual endorsement, being voluntary

  - Formats to use to be obtained from GR CLASS as follows:

    - Survey report for certification of prevention of pollution by garbage from ships	GRB-01

    - GR Survey Statement Report	P24

REFERENCE

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-GRB | MARPOL ANNEX V (GARBAGE) SURVEYS |

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | NA | NA | https://panamashipregistry.com/circular- category/circulares/ |
| Barbados | NA | NA | https://barbadosmaritime.org/bulletins/ |
| Belize | NA | NA | https://www.immarbe.com/circularslist/ |
| Sierra Leone | NA | NA | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentat ion/instruments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos -legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/dispo siciones |
| Nigeria | NA | NA | https://nimasa.gov.ng/press- center/marine-notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |