| G R C L A S S |  |
| --- | --- |
| GR-PM-IAPP & EIAPP | INTERNATIONAL AIR POLLUTION PREVENTION (IAPP) ENGINE INTERNATIONAL AIR POLLUTION PREVENTION (EIAPP) |

# SECTION 1

General

  - Annex VI of MARPOL entered into force on 19 May 2005. The Annex was latest amended in 2018, effective on 1 March 2020.

  - The provisions of the Annex apply to all ships, except where expressly provided otherwise.

  - Under Regulation 13, the control of emissions of Nitrogen Oxides (NOx) only applies to each marine diesel engine of more than 130 kW installed on a ship; and each marine dieselengine or more than 130 kW that undergoes a major conversion on or after 1 January 2000;

  - Under Regulation 13, the control of emissions of NOx applies to a marine diesel engine ofmore than 5,000 kW and a per cylinder displacement at or above 90 L installed on a ship constructed on or after 1 January 1990 but prior to 1 January 2000, provided that an approved method for that engine has been certified by an Administration.

  - Each diesel engine which is required to comply with Regulation 13 (Nitrogen Oxides) of revised Annex VI must be issued an Engine International Air Pollution Prevention(EIAPP) Certificate and aNOx Technical File.

  - An IAPP Certificate is required for all ships of 400 GT or above, and platforms and drilling rigs, engaged in international voyages. Ships below 400 GT are not required to be provided with theIAPP Certificate.

# NOx REGULATIONS

  - Regulation 13 of revised Annex VI concerns NOx emissions from marine diesel engines and shall apply to:

    - Each marine diesel engine with a power output of more than 130 kW installed on a ship;

    - Each marine diesel engine with a power output of more than 130 kW which undergoes a major conversion on or after 1 January 2000; and

    - Each marine diesel engine with a power output of more than 5000 kW and a per cylinder displacement at or above 90 L which is installed on a ship constructed on or after 1 January 1990 but prior to 1 January 2000

# 2.2           TIER APPROACH

2.2.1 Tier Iemission limits apply to all marine diesel engines with a power output ofmore than 130 kW installed on ships constructed on or after 1 January 2000 and prior to 1 January2011.

    - Emission limits equivalent to Tier I may apply to marine diesel engines with a power output

of more than 5,000 kW and a per cylinder displacement at or above 90 liters installed on a ship constructed on or after 1 January 1990 but prior to 1 January 2000 according to regulation VI/13.7.

| G R C L A S S |  |
| --- | --- |
| GR-PM-IAPP & EIAPP | INTERNATIONAL AIR POLLUTION PREVENTION (IAPP) ENGINE INTERNATIONAL AIR POLLUTION PREVENTION (EIAPP) |

    - Tier II emission limits apply to all marine diesel engines with a power output of more than 130kW installed on ships constructed on or after 1 January 2011.

    - Tier III emission limits apply to all marine diesel engines with a power output of more than 130 kW installed on ships constructed on or after 1 January 2016, operating in the North American or the United States Caribbean Sea NOxemission control area.

    - Tier III emission limits apply to all marine diesel engines with a power output of more than 130 kW installed on ships constructed on or after 1 January 2021, operating in the North Sea or the Baltic Sea NOx emission control area.

2.1         NOX TECHNICAL CODE

2.1.1 The Technical Code on Control of Emission of Nitrogen Oxides from Marine Diesel Engines adopted by Conference resolution 2, as may be amended by the Organization, provided that such amendments are adopted and brought into force in accordance with the provisions of article 16 of the present Convention concerning amendment procedures applicable to an appendix to an Annex.

  - MAJOR CONVERSION

  - In relation to regulation 13 of revised Annex VI (Nitrogen oxides), means a modification on or after 1 January 2000 of a marine diesel engine that has not already been certified to the standards set forth as per Tier I, Tier II or Tier III of this regulation where:

    - the engine is replaced by a marine diesel engine or an additional marine diesel engineis installed, or

    - any substantial modification*, as defined in the revised NOx Technical Code2008, is made to the engine, or

    - the maximum continuous rating of the engine is increased by more than  10% compared to the maximum continuous rating of the original certification of the engine.

  - SURVEYS

  - Every ship of 400 gross tons and above and every fixed and floating drilling rig and other platforms shall be subject to the following periodic surveys by  GR to ensure that the equipment, systems, fittings, arrangements, and material fully comply with applicable requirements of chapter 3 (Control of Emissions) of revised Annex VI:

    - An initial survey before the ship is put into service or before the IAPP Certificate isissued for the first time;

    - An annual survey within three months before or after each anniversary date of the certificate;

| G R C L A S S |  |
| --- | --- |
| GR-PM-IAPP & EIAPP | INTERNATIONAL AIR POLLUTION PREVENTION (IAPP) ENGINE INTERNATIONAL AIR POLLUTION PREVENTION (EIAPP) |

    - A renewal survey at intervals not exceeding five years, except where Regulation 9.2,9.5, 9.6 or 9.7 of revised Annex VI is applicable;

    - An intermediate survey within three months before or after the second anniversary date or within three months before or after the third anniversary date of the certificate which shall take the place of one of the annual surveys specified in .2 of this paragraph.

    - An additional survey either general or partial, according to the circumstances, shall be made whenever any important repairs or renewals are made that conform with the provisions of revised Annex VI or after a repair resulting from an accident or a defect that substantially affects the efficiency or completeness of the equipment covered under revised Annex VI.

    - Surveys of engines and equipment for compliance with Regulation 13, Nitrogen Oxides, shall be conducted in accordance with the revised NOx Technical Code 2008.

    - When GR determines that the condition of the equipment does not correspond substantially with the particulars of the certificate, they shall ensure that corrective action is taken and shall immediately notify the Administration. If such corrective action is not taken, the certification will be withdrawn by the Administration.

# Issuance and form of International Air Pollution Prevention (IAPP) Certificate

  - An IAPP Certificate shall be issued after the successful completion of initial and renewal surveys in 3.1 above to:

    - Any ship of 400 gross tons and above engaged in voyages to ports or offshore terminals under the jurisdiction of other Parties; and

    - Platforms and drilling rigs engaged in voyages to waters under the sovereignty or jurisdiction of other Parties.

    - The form of the IAPP is specified in Appendix I and new Appendix VIIIto revised Annex VI or MARPOL respectively.

# Issuance and form of Engine International Air Pollution Prevention (EIAPP) Certificate

  - Basically, every engine shall have its NOx emissions measured and pre-certified, and an EIAPP Certificate or a Statement of Compliance “STATEMENT OF COMPLIANCE FOR ENGINE AIR POLLUTION PREVENTION” shall be issued to the engine before the engine is installed on board a ship. In cases where on-board diesel engines do not have an EIAPP Certificate nor the Statement of Compliance, please confirm with the engine manufacturer or shipowner.

| G R C L A S S |  |
| --- | --- |
| GR-PM-IAPP & EIAPP | INTERNATIONAL AIR POLLUTION PREVENTION (IAPP) ENGINE INTERNATIONAL AIR POLLUTION PREVENTION (EIAPP) |

  - At Initial survey, it is to be confirmed that the EIAPP Certificate (or Statement of Compliance) and Technical file are retained on board ships, and it is also to be confirmed that the NOx emissions remain within the limits (on-board NOx verification), in accordance with the procedures described in the approved Technical file, after adjustment of injection timing etc. which are generally conducted on board ships.

  - For on-board NOx verification, engine parameter check method and simplified measurement method etc. are available. In the parameter check method, the Engine parameter record book which contains records of change or adjustment of NOx critical components is to be checked and confirmation of ID number of actual components etc. is to be carried out.

  - Therefore, at Initial survey, please prepare the EIAPP Certificate (or Statement of Compliance), Technical file and Engine parameter record book for the relevant engines. When carrying out the parameter check method, to confirm ID numbers around combustion chamber, an open up inspection may be required.

# Certificates and other documents which are to be prepared at Initial survey

  - It  is  to  be  ensured  that  the  following  Certificates  etc.  are retained  on board  the ship, and are appropriate.

7.1.1     Bunker delivery notes (for any bunkering on or after 19 May 2005)

    - EIAPP Certificate or equivalent Statement of Compliance (when the requirements for NOxare applied)

    - Record book of engine parameters (when the requirements for NOx are applied and Parameter check method is adopted)

    - Logbook (when the requirements for fuel oil with the sulphur contents of less than1.5%m/m are applied)

    - Operation manual for vapour collection system (when the requirements for VOCs are applied)

    - Operation  manual  for  shipboard  incinerator  (when  the  requirements  for onboard incinerator are applied)

    - Type   approval   certificate	etc.    for    shipboard    incinerator    (when   the requirements for onboard incinerator are applied)

    - Manufacturer’s certificate that an incinerator  has been constructed in accordance with IMOMEPC Resolution 76(40) (when the requirements for onboard incinerator are applied)

    - Product verification certificate for shipboard incinerator (when possessed)

    - Installation examination certificate for shipboard incinerator (when possessed)

| G R C L A S S |  |
| --- | --- |
| GR-PM-IAPP & EIAPP | INTERNATIONAL AIR POLLUTION PREVENTION (IAPP) ENGINE INTERNATIONAL AIR POLLUTION PREVENTION (EIAPP) |

8.0	REFERENCES

| MARITIME ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021 | 06/2022 | https://panamashipregistry.co m/circular- category/circulares/ |
|  | MMC 375: Ship Implementation Plan for the Consistent Implementation of the 0.50% Sulphur Limit under MARPOL Annex VI. | 11/2021 |  |
|  | MMC 376: Phase out and prohibition on installation of equipment containing ODS, including hydrochlorofluorocarbons (HCFCs) | 08/2019 |  |
| Barbados | Bulletin 168: MARPOL Annex VI – Eng. Log book Supplement | NA | https://barbadosmaritime.org /bulletins/ |
|  | Bulletin 281 : Fuel Oil Consumption Data | NA |  |
|  | Bulletin 306 : Sulphur Limit 0.50% – Jan 2020 | NA |  |
|  | Bulletin 316 : Fuel Oil Sulphur Cap | NA |  |
| Belize | MMN-19-001 Global 0.50 mm Sulphur Limit under MARPOL Annex VI | NA | https://www.immarbe.com/cir cularslist/ |
| Sierra Leone | MC 41: Compliance to requirements of MARPOL ANNEX VI regarding low Sulphur fuel oil | 04/2020 | https://slmarad.com/maritime -circulars |
| Palau | NA | NA | https://www.palaureg.com/inf ormation-centre/online- library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/d ocumentation/instruments- and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.h n/aspectos-legales/circulares- y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web /site/disposiciones |
| Nigeria | NA | NA | https://nimasa.gov.ng/press- center/marine-notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.co m/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/nor matividad |
| Guinea Bissau | NA | NA | NA |