| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

# SECTION 1

General

  - Surveyors conducting Marpol 73/78 Annex I surveys are to be familiar with the applicable provisions of the International Convention for the Pollution of Prevention from Ships 1973 as amended by the Protocol of 1978 and as further amended by the protocol of 1997, and subsequent amendments relating thereto.

  - MARPOL (IOPP) surveys are important as lot of port state detentions are due to deficiencies related to these certificates/surveys. Most detentions are due to lack of awareness on the part of the crew about the equipment and regulations - tend to neglect the equipment or violate rules.

  - When requested to carry out a survey, the surveyor must first check the existing certificate onboard to establish which survey is due/overdue.

  - Renewal surveys are required every five years and before the certificate in force have expired.

  - Four annual are required and one of them shall be an intermediate survey is required during the duration of the certificate, as per the Harmonized System of Survey and Certification (HSSC). Annual surveys are to be held within three months before and three months after the due date (anniversary date). Intermediate surveys are to be held within three months before or after the second or third anniversary date of the NLS Certificate or Annex I Certificate All surveys should be held only within the permissible range dates.

  - The pollution prevention arrangements and equipment and the documents approved for compliance with the regulations are included in the record of construction and equipment (Form A), attached to the current certificate onboard.

  - For first survey by GR Class Classified for Standard (GR), the previous certificate onboard is to be sighted to establish the survey due: the procedures for transfer of class and certification  should be followed. If the previous certificate as issued by a classification society or the flag administration, the existing survey cycle may be followed. Any due or overdue survey is to be carried out and if no surveys are due/overdue, a general examination, equivalent to an annual survey, is to be carried out. Copies of the existing certificate, any  exemption/waiver certificate and  Form  A/B are to be attached to the reports submitted to GR Head Office. If previous certificate is not available or not issued by asociety, a complete renewal survey is to be carried out. At (first survey by GR Class Classified for Standard (GR)), the status of approval of the various manuals – Oil Discharge Monitoring and Control Systems Operations (ODM) manuals, Crude Oil Washing Operations and Equipment (COW) manuals, Dedicated Clean ballast Tank operation (DCBT) manuals and Shipboard Oil pollution Emergency Plan (SOPEP) – must be verified. If the manuals/plan are approved by a society or a recognized organization duly authorized by the same flag state, the previous approval may be accepted. In the previous case but, in addition, involving a change of class, the manuals/plan is to be endorsed  “This  manual/plan has been accepted by GR Class Classified for Standard (GR) on behalf of (name of new flag state) on the basis of approval by (previous society or flag state name)” and, in any of the last two cases, a copy of the endorsed page is to be submitted to GR Head Office. If there are any changes to the ship’s Owners/management or operations, which will affect the mandatory sections of the SOPEP, the Owners should be advised to submit urgently a revised plan for approval to GR Head Office. If the manuals/plans have not been approved by a society or not approved at all, copies of the plans must be submitted for approval. The copy on board is to be

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

endorsed “Provisionally accepted pending approval by GR Class Classified for Standard (GR) Head Office”. During the first survey by GR Class Classified for Standard (GR), Surveyors should ensure that correct equipment is fitted (100 ppm oily water separating equipment is not acceptable and have been replaced by 15 ppm oil filtering equipment from 6th July 1998) and that for tankers the correct instantaneous discharge rate of oil content in oily water mixtures is used in the Oil Discharge Monitoring and Control Systems (previous limit of 60 liters per nautical mile has been reduced to 30 liters per nautical mile effective from 6 July 1993).

  - For each survey, the checklist (Form No. IOPP-01) is to be duly completed as appropriate and copy forwarded to GR Head Office with survey reports (covering details of any modifications, repairs/renewals). A Form Aor Form B is also to be prepared and attached to the IOPP certificate, if one is not available on board. On satisfactory completion of a renewal or initial survey, a short term certificate may be issued valid for five months. A short term certificate is not to be issued, if an exemption or waiver is required and GR Head Office permission is not available. If some minor items remain outstanding for completion of the survey (because the required  services or replacement equipment are not available locally), a CONDITIONAL certificate should be issued with validity adequate to allow the vessel to reach the next suitable port, but not exceeding two (2) months. The conditions (outstanding items) must be entered on the Statement Report (P24) and a copy of any conditional certificate issued is to be sent by fax to GR Head Office. Conditional  Certificates may only be issued  provided that GR Class Head  Office has received written authorization from the flag and instructions have been issued by Head Office to the local surveyor. For tankers, due account should be taken of the requirements of Regulation 31 of MARPOL with regard to the Oil Discharge Monitoring and control system which states that “defective unit shall be made operable as soon as possible. The port state  authority may  allow the tanker with a defective unit to undertake one ballast voyage before proceeding to a repair port”. GR Head Office will issue the full term certificate after review of the service reports.

  - Where any alterations or additions to the ship’s equipment, fittings or construction are to be made, GR Head Office is to be advised and all necessary plans/documents requested are to be forwarded to GR Head Office. Where alterations are made to existing systems (COW, CBT or ODMCS), the relevant manuals are to be re-submitted for approval. The Form A/B is to be amended as required.

  - Any proposals/requests for waivers /exemptions are to be submitted to GR Head Office and certificates are to be issued only if instructed by GR Head Office in such cases. Normally exemptions/waivers are issued for ships operating in special areas or in coastal areas. Any request for exemption must be supported by a request from the owner stating the reason for requesting the exemption/waiver and the trading areas of the vessel and a copy of any previous exemption/waiver is to be submitted.

  - Annex-I of MARPOL is applicable to all ships. However IOPP certificate is only required for tankers with gross tonnage 150 and above and other ships of 400 GT and above. IOPP certificate is also required for cargo ships of less than 400 GT if they carry substantial amount of oil (200 tonnes or more - trawlers, offshore supply vessels etc). Annex-I mainly deals with two types of discharges: discharges from machinery spaces (applicable to all ships) and discharges from cargo or ballast spaces (applicable to oil tankers and ships belonging to third category mentioned above).

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

# SECTION 2

Annual Survey

  - The scope of the mandatory annual survey should include an examination of the various items of equipment together with tests as considered necessary to ensure adequate condition and standard of maintenance until next survey.

  - Ensure that the IOPP certificate onboard is valid and complete along with their supplements (Form A/B) and copies of type approval certificates for equipment such as oil filtering equipment, oil content meter, ODME, oil-water interface detectors etc. All approved manuals/documents such as SOPEP, ODME/COW/CBT must be verified onboard. The Manuals must be in a language understood by the ship’s personnel engaged in such operations, while the SOPEP should be in a language understood by the ships Master and Officers. The Master/Chief Engineer must confirm that no unapproved modifications have been made to the pollution prevention arrangements and equipment since the last survey.

  - Oil Record Book should be checked to ensure that regular entries are made regarding bunkering, discharging at sea through 15ppm equipment, disposal by incinerating, etc. The quantities recorded as disposed should be reasonable (for ships using heavy fuel oil approximately 1% sludge is generated – rough amount can be worked out based on number of days of voyage and daily fuel consumption). There should be regular entries in the oil record book (normally at the end of each voyage or at weekly intervals) regarding the quantity of sludge retained on board in each of the tanks specified in the IOPP Form A/B. Only those tanks listed in the Form A/B should be used as sludge/bilge holding tanks. A monitoring of the entries should be done to check whether the oil record book entries are genuine or fabricated figures. Where discharge to shore reception facilities is indicated, the receipt should be attached to the oil record book. The oil record book is to be in the format recommended in the MARPOL book and should be regularly signed by both Chief Engineer & Master.

  - Cargo Record Book (for tankers only): There should be regular entries for cargo loading, ballast operation at port or at sea, discharge/disposal of ballast through ODME or to shore facility and COW etc. The quantities should be correct. Verify the correctness of entries by checking records of the discharge of ballast through ODME with ODME printouts. The cargo record book is to be in the format recommended in the MARPOL book and should be regularly signed by chief officer (or person in-charge of cargo control room) and Master.

  - The 15ppm oil filtering equipment should be type approved by a national administration or a recognized authority. Check that the type approval certificate  is available onboard  and relates to the equipment installed. Check the function by operation or simulation (most of the activators are photo electric, so blocking the light by inserting some object would simulate the alarm. Visual inspection of the filtered bilge water should be carried out (from sample point on the unit) to ensure that oil traces are not found and the

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

unit is functioning properly. Check the 15ppm equipment and associated piping for signs of corrosion / leaks, check that pressure gauges are operational and check operation of 15ppm alarm and automatic stopping device (if fitted). The manual valve should be checked to ensure that it is not seized and moves freely. Check that the piping has not been modified and there are no connections to the  overboard discharge line. Audiovisual alarm or automatic stopping is required only for vessels over 10,000 Gross Tonnage. Automatic stopping device is mandatory if the vessel is operating in special areas.

  - Check the bilge pump in running condition (pressure gauge, smooth operation etc).

  - Check whether any piping arrangement is fitted which bypasses the 15ppm filter and leads direct over board from bilges or from sludge tanks.  Check the condition of valve and pipes for wastage/leak etc.

  - Check sludge tank for corrosion. Check that there is no  discharge connection other than through the standard discharge connection. By taking sounding check that the remaining capacity of the sludge tank is sufficient for the voyage.

  - Engine Room/Pump room Bilges: Check for excessive oil/oily water mixture.

  - Verify that standard discharge connection is fitted and it is of correct size as per regulation. Check the condition of piping & discharge flange. Check that special tally plate and/or distinctive paint identify it.

  - Check that notices/placards are posted at over board discharge valve, standard discharge connection, oily bilge pump, bunkering stations etc. warning the crew of consequences of unauthorized discharges and oil spillage.

  - Verify correct functioning of incinerator, boiler etc (using sludge).

  - Check the Sounding of Holding Tanks and see that they have sufficient capacity (only if holding tanks are required for waiver of regulation 1614)

  - Oil discharge monitoring equipment is to be checked for proper operation. Check the external condition of system & equipment. Functioning of the equipment including automatic/manual stopping of discharge and interlock for starting the pump to be verified. This may be done by simulating input values for flow rate

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

(based on pump characteristics), ships speed, oil content etc. The rate of instantaneous discharge should not exceed 30 liters per nautical mile. Check the associated audio-visual alarms and indicators on the control panel. Check that recording/indicating devices are working properly, check the printouts for last few weeks (records showing date, time, position etc should be retained for 3 years), verify with oil record book entries for overboard discharge, check supply of consumables (paper, ink etc).

  - Oil water interface detector are to calibrated and maintained periodically. Check function at random by introducing into ballast or slop tanks or a test bucket containing mixture of water/oil.

  - Check segregated ballast tanks (SBT) for contamination with oil (visual examination as practicable). Check SBT lines for any cross connection with cargo line, if a portable spool piece is fitted check that a non-return valve is fitted and notice is posted for restricted use. Check dedicated clean ballast tank for contamination.

  - An examination of the Crude Oil Washing System (COW) should be carried out and should include external examination of the system including pump, piping, valves & deck-mounted washing machines for leakage, corrosion/wastage, wear & tear. Where water washing is also fitted, check that provision for isolating steam heaters is operational during crude oil washing. Check that relief valve is fitted for COW pump. Check the operation of COW machines, if practicable during survey.

  - Check the effectiveness of the stripping line by  hand dipping. Check the stripping line including small diameter pipe for shore connection for wastage/leakage. Check the over board discharge lines including part flow line on deck (in case of discharges situated below water line in existing tankers).

  - Check the means of communication between observation point (where discharge can be observed) & control station.

  - Check the function of emergency stopping device for cargo pumps from control station & pump room.

  - Check that arrangements to prevent spillage of oil, especially at bunkering stations, tanker manifolds, save all trays around F.O tank air pipes etc. are adequate.

# SECTION 3

Intermediate Survey

  - An Intermediate survey should, in addition to the requirements of an annual survey, be a thorough examination and sufficiently extensive to ensure that the equipment and associated pumping and piping systems comply with the regulations and are in good working order sufficient until the next renewal survey.

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

  - The oily bilge pump has to be dismantled and examined. Any part found excessively worn like shaft, gland, impeller or bearing to be renewed/repaired. Oil filtering equipment, to be dismantled and examined, with particular attention being paid to the underside of the equipment. Casing should be checked for wastage. Filter elements to be thoroughly cleaned. Damaged filter must be replaced. It is to be ensured that sufficient spare elements/parts are available. The filtering unit should be flushed with fresh water after re-assembly and before use. Over board discharge valves to be opened up for examination. 15-PPM oil content meter must be calibrated. Complete piping is to be thoroughly examined and all valves in the system should be checked for ease of operation. Pipelines are to be traced to ensure that no unauthorized modification has been carried out.

  - Sludge tanks to be cleaned and examined for wastage.

  - Check the calibration of oil discharge monitoring device, thorough examination of control system & oil content meter for defects.

  - COW piping to be visually examined and in case of doubt, the lines are to be pressure tested and/or gauged.

  - For tankers over 10 years old, two selected cargo tanks to be examined for verifying the continued effectiveness of the COW and stripping systems. If the tanks cannot be gas freed, the internal examination of the tanks may be conducted separately with the classification and SOLAS CCC survey requirements. If vessel is not classed by GR Class Classified for Standard (GR), survey records should be verified.

# SECTION 4

Renewal Surveys

  - The renewal survey should be a thorough and complete examination with tests as appropriate. On completion of a renewal survey, the equipment, systems, fittings and arrangements should, given adequate routine maintenance, remain in good condition and working order till the next renewal survey. In addition to the requirements of an intermediate survey, a renewal survey should include the following:

    - For tankers fitted with a COW system, all cargo tanks must be internally examined to confirm continued effectiveness of the COW system. In lieu of the internal examination, oil on top test could be

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

carried out within 6 months prior to or after the due date for the renewal survey. If oil on top test is to be carried out, GR Head Office should be contacted for instructions.

    - COW lines are to be pressure tested (at least to working pressure).

    - Check that there are no leaks from ballast pipes passing through cargo tanks or cargo pipes passing through ballast tanks.

    - Examine slop tanks & piping for wastage.

# SECTION 5

Reporting

  - The relevant survey checklist Form No. IOPP-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. For initial surveys, Form I-IOPP must be used and submitted to GR CLASS Head Office.

  - At first surveys by GR, under transfer of certification, copies of the previous society’s certificates, record of equipment Form A/B and copy of approval for various manuals/plans and list of any outstanding deficiencies, recommendations etc should also be forwarded. Copies of type approval certificates for the pollution prevention equipment (oil filtering equipment, 15 ppm alarm, oil/water interface  detectors, ODME, incinerators etc) should be verified and confirmed by attending surveyor

  - On completion of renewal surveys or First Survey by GR Class Classified for Standard (GR), a SHORT TERM certificate should be issued, valid for 5 months. When survey is incomplete or deficiencies are noted, the Short term Certificate should be marked CONDITIONAL and the outstanding items/deficiencies must be entered at the reverse of the certificate and on the Statement Report P4 (and P25 if necessary. The certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two (2) months (except where only approval of manuals remain outstanding, in which case the validity should be restricted to three (3) months). A copy of the CONDITIONAL certificate is to be e-mail/faxed sent to GR Head Office. Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

  - On completion of other surveys (annual and intermediate), the existing full term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR  seal/stamp.  A  copy  of  the  endorsed  certificate  must  be  submitted  to  GR  Head  Office.  At

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-IOPP | MARPOL ANNEX I (IOPP) SURVEYS |

annual/intermediate surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the Statement Report P24 (and P25, if applicable . The Full Term IOPP certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the Form No. P24.

  - A Statement report P24 (and P25 if applicable should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owner’s proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

# Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or  www.amp.gob.pa. In case of conflict, the surveyor should follow the instruction issued by the flag state that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.