| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-ISPP | MARPOL ANNEX IV (SEWAGE) SURVEYS |

# SECTION I

General

  - Marpol 73/78 Annex IV contains the regulations for the prevention of pollution by sewage from ships. The Annex has not yet been adopted by sufficient number of countries with the required combined tonnage andis therefore not internationally mandatory. However, some countries have adopted the Annex and apply it unilaterally.

  - Marpol 73/78 Annex IV is applicable to all ships of 400 gross tonnage and above and ships of less than 400 gross tonnage which are certified to carry more than 15 persons, with respect to the disposal of:

Drainage and other wastes from any form of toilets, urinals, and WC scuppers,

Drainage from medical premises (dispensary, sick bay, etc.) via wash basins, wash tubs and scupperslocated in such premises;

Drainage from spaces containing living animals and

Other waste-waters when mixed with the drainages defined above.

  - The regulatory means of sewage discharge is dependant on whether or not a treatment plant is provided and is an operational procedure. The survey and certification Regulations are in respect of the arrangements and equipment provided only.

  - Where equipment is being installed for the first time or where equipment is already installed but not surveyed till date, plans should be submitted to GR Head Office for approval. The sewage treatment plant should be approved in accordance with IMO resolution MEPC 2(VI) or MEPC.157 (55) and, MEPC.227 (64), the sewage comminuters should be type approved by a competent administration. If the equipment are approved by other organization, GR attending surveyor should confirm they are type approved in accordance with the IMO resolutions.

  - When survey is carried out by GR Class-Classified For Standard (GR CLASS) for the first time, the previous certificate (or statement of compliance) onboard should be sighted to establish the survey due. Any due or overdue survey should be carried out and if no survey is due, a general examination should be carried out. Acopy of the existing certificate should be submitted to GR Head Office along with survey reports.

  - A renewal survey is due every five years. On completion of the initial/renewal survey, an Interim Certificate/STATEMENT of COMPLIANCE should be issued valid for five

(5) months. GR Head Office will issue the full-term Certificate/Statement of Compliance after review ofthe survey reports.

  - The survey should be such as to ensure that the equipment provided, fittings and arrangements are in good condition and given adequate routine maintenance be expected to remain in good working order until the next survey. The owner’s representative should confirm that no unapproved modifications have been carried out since the last survey.

  - Surveyors should on no account carry out inspection of holding tanks or equipment that has been openedup unless it has been thoroughly cleaned and disinfected. Both methane and hydrogen sulphide

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-ISPP | MARPOL ANNEX IV (SEWAGE) SURVEYS |

gas can be generated from raw sewage. Operating instructions and chemicals such as disinfectant necessary for operation of the plant should be available on board.

  - Sewage treatment plant should be examined for leakage of liquids and gases such as hydrogen sulphide. A bad smell is an indication that the plant is not working properly. Disinfectant dosing points should be accessible and access for the taking of samples should be adequate. The type, model and name of the manufacturer should be clearly shown on the sewage treatment plant by means of a durable label or nameplate.

  - Black water piping feeding to sewage treatment plant or holding tank and shore connections should be examined to verify their continued satisfactory condition. The arrangements should include a pipeline terminating on the open deck and fitted with a shore connection for the purpose of discharging the sewage to an external reception facility.

  - Ventilation pipes should be independent of other vent systems.

  - According to regulation 11.1.1 of the revised Annex IV of MARPOL 73/78 requires that untreated sewage, which may be discharged at more than 12 nautical miles from the nearest land, should not be discharged instantaneously but at a moderate rate of discharge, when the ship is “en route” and proceeding at a speed not less than 4 knots, while the rate should be approved by the Administration. For Panamanian vessels, the rate of discharge of sewage must be approved according MMC 168, dated July 2008.

# SECTION 2

Reporting

  - The relevant survey checklist Form No. ISPP-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. For new buildings, the Form I-ISPP should be used.

  - At first surveys by GR, under transfer of class and certification procedure, copies of the previous society’s certificates and list of any outstanding deficiencies, recommendations etc should also be forwarded. Copies of type approval certificates for the sewage treatment plant should also be verified and confirmed by attending surveyor.

  - On completion of renewal surveys or First Survey by GR CLASS CLASSIFIED FOR STANDARD (GR), an Interim certificate should be issued, valid for five (5) months. When survey is incomplete or deficiencies are noted, Interim Certificate should be marked CONDITIONAL and the outstanding items/deficiencies must be entered at the reverse of the certificate and on the Survey Statement, if necessary. The certificate is to be issued for such validity as will permit the vessel to reach the next suitable port and in no case should exceed two (2) months (except where only approval of manuals remains outstanding, in which case the validity should be restricted to three (3) months). Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-ISPP | MARPOL ANNEX IV (SEWAGE) SURVEYS |

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the Survey Statement

  - A Survey Statement whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owners’ proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or  www.amp.gob.pa. In case of conflict, the surveyor should follow the instruction issued by the flag state that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-ISPP | MARPOL ANNEX IV (SEWAGE) SURVEYS |

# 3. REFERENCES

| MARITIME ADMINISTRATION | NATIONAL REGULATIONS (OR SIMILAR INSTRUMENT) |  |  | WEBSITE |
| --- | --- | --- | --- | --- |
| Panama | 168 | Approval of the Rate of Discharge for Untreated Sewage | May 2022 | https://panamashipregistry.com/circular- category/circulares/ |
| Barbados | - | N.A. | - | https://barbadosmaritime.org/documents/ |
| Sierra Leone | - | N.A. | - | https://slmarad.com/maritime-circulars/ |
| Vanuatu | - | N.A. | - | http://www.vanuatumaritimeships.com/Sa fety_Document.ASPX |
| St. Kitts and Nevis | MC 25 09 | Rate of discharge of untreated sewage mepc resolution 157(55) | Aug 2009 | https://www.skanregistry.com/en/maritim e-circulars |
| Belize | MSN-0037 | Entry into force of marpol 73/78 annex IV prevention of pollution by sewage from ships | Aug 2008 | https://www.immarbe.com/circularslist/ |
| Palau | - | N.A. | - | https://www.palaureg.com/information- centre/online-library/legislation/ |

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)

E-mail: info@grclass.com Web: www.grclass.com

Approved by: GM	                                 	                                                                                                                            Page 4 of 4