GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

# REPORT OF SURVEY FOR THE INTERNATIONAL CERTIFICATE OF

FITNESS FOR THE CARRIAGE OF DANGEROUS CHEMICALS IN BULK (IBC)1 AND THE CERTIFICATE OF FITNESS FOR THE CARRIAGE OF DANGEROUS

CHEMICALS IN BULK (BCH)2

| NAME: | IMO No.: | CALL SIGN: | FLAG: | CLASS No. |
| --- | --- | --- | --- | --- |

For Initial survey mark “X” to indicate “Yes” and “N” to indicate an outstanding action in appropriate block for each item. If an item is

not applicable mark “NA” in appropriate block.

Initial Survey (IS) :

is a complete inspection before a ship is put into service of all the items relating to this particular

certificate, to ensure that the relevant requirements are complied with and that these items are satisfactory for the service for which the ship is intended.

1. INITIAL SURVEY

- Determining the products that it is intended that the ship will be permitted to carry and noting the corresponding minimum special requirements (IBC Code 83/90/00, ch.17) and any other special requirements (IBC Code 83/90/00, ch.15);

- Examining the plans for the ship type, location of the cargo tanks, cargo containment, materials of construction, cargo temperature control, cargo tank vent systems, environmental control, electrical installations, fire protection and fire extinction, instrumentation and the provision, specification and stowage of the equipment for personnel protection (IBC Code 83/90/00, chs.2, 4, 6, 7, 8, 9, 10, 11, 13 and 14);

- Examining the plans for the freeboard and intact stability, discharges below the bulkhead deck and survival capability (IBC Code 83/90/00, ch.2);

- Examining the plans for the ship arrangements IBC Code 83/90/00, ch.3);

- Examining the plans for the cargo transfer IBC Code 83/90/00, ch.5);

- Examining the plans for the mechanical ventilation in the cargo area (IBC Code 83/90/00, ch.12);

- Confirming that tanks containing cargo or residues of cargo are suitably segregated from accommodation, service and machinery spaces and from drinking water and stores for human consumption, that cargo piping does not pass through any accommodation, service or machinery space other than cargo pump rooms or pump rooms

and that cargoes are not to be carried in either the fore or the aft peak tank (IBC Code 83/90/00, ch.3);

- Examining the air intakes and openings into the accommodation, service and machinery spaces in relation to the cargo piping and vent systems and their entrances, air inlets and openings in relation to the cargo area (IBC Code 83/90/00, ch.3);

- Examining the arrangements of the cargo pump rooms (IBC Code 83/90/00, ch.3);

- Examining the accesses to spaces in the cargo area (IBC Code 83/90/00, ch.3);

- Examining, when applicable, the bow or stern loading and unloading arrangements with particular reference to the air inlets and entrances to the accommodation, machinery and service spaces, the electrical equipment, fire- fighting arrangements and means of communication and testing the remote shut down for the cargo pumps (IBC Code 83/90/00, ch.3);

- Confirming that the cargo tank types are arranged and installed in accordance with the approved plans, internally examining the cargo tanks, water ballast tanks and other spaces in the cargo area and pressure testing the boundaries (IBC Code 83/90/00, ch.4);

- examining the cargo transfer arrangements and confirming that any hoses are suitable for their intended

purpose and, where appropriate, type-approved or marked with date of testing (IBC Code 83/90/00, ch.5);

1 IBC code applies to ships the keel of which was laid on or after 1986-07-01.

2 BCH code applies to ships the keel of which was laid before 1986-07-01.

- Examining and testing any cargo heating and cooling systems (IBC Code 83/90/00, ch.7);

- Confirming that high-level alarms, or overflow control systems or spill valves or other equivalent means provided to control possible liquid rising in the venting system, are operating satisfactorily (IBC Code 83/90/00, ch.8);

- Confirming that suitable provision is made for drainage of vent lines and that no shut-off valves or other means of stoppage, including spectacle or blank flanges, are fitted either to the individual vents or to the header, if the vents are combined or either above or below pressure/vacuum relief valves with closed vent systems (IBC Code 83/90/00, ch.8);

- Examining the arrangements for environmental control, including the means of storing or generating and drying an inert gas (IBC Code 83/90/00, ch.9);

- Examining the electrical installations and confirming that, when appropriate, special materials have been used and that the electrical equipment installed in hazardous locations, as permitted, is certified by a recognized authority for the cargoes to be carried (IBC Code 83/90/00, ch.10);

- Examining the fixed fire fighting system for the cargo pump room and confirming that the installation tests have  been satisfactorily completed and that its means of operation are clearly marked (IBC Code 83/90/00, ch.11);

- Checking the deck foam system for the cargo area, including the supplies of foam concentrate, and testing that the minimum number of jets of water at the required pressure in the fire main is obtained, see (EI) 1.1.3.1 in Annex 1, when the system is in operation (IBC Code 83/90/00, ch.11);

- Examining, and confirming the satisfactory operation of, the arrangements for the mechanical ventilation of spaces in the cargo area normally entered during cargo handling operations (IBC Code 83/90/00, ch.12) and checking in particular that:

it may be controlled from outside the space;

warning notices concerning its use have been posted;

it is of the extraction type, with extraction from below the floor plates, unless the space houses   electrical motors driving cargo pumps when it should be of the positive pressure type;

the ducting does not pass through accommodation, machinery and service spaces and that the exhaust ducts are clear of the ventilation inlets and openings to such spaces;

electric motors driving ventilation fans are positioned outside the ventilation ducts and the ventilation fans and the ducts, in way of the fans only, are of non-sparking construction in hazardous locations;

- Confirming that double bottoms, cofferdams, duct keels, pipe tunnels, hold spaces and other spaces where cargo may accumulate are capable of being efficiently ventilated to ensure a safe environment when entry into the space is necessary and that, when appropriate, permanent ducting is provided and any ventilation fans comply with (DI) 1.1.2.22.5 (IBC Code 83/90/00, ch.12);

- Examining the intrinsically safe systems and circuits used for measurement, monitoring, control and communication purposes in all hazardous locations (IBC Code 83/90/00, ch.13);

- Checking the provision of equipment for personnel protection (IBC Code 83/90/00, ch.14) and in particular that:

suitable protective clothing is available for the crew engaged in loading and discharging operations and that suitable storage is provided;

The required safety equipment and associated breathing apparatus and air supplies and, when appropriate, emergency-escape respiratory and eye protection, are provided and are properly stowed;

medical first-aid equipment, including stretchers and oxygen resuscitation equipment are provided;

decontamination arrangements and eyewashes are operational;

the required gas detection instruments are on board and that arrangements have been made for the supply of the appropriate vapour detection tubes;

- Confirming that a loading and stability information booklet, containing details of typical service and ballast conditions, provisions for evaluating other conditions of loading, a summary of the ship's survival capabilities and sufficient information to ensure that the ship is loaded and operated in a safe and seaworthy manner, is available on board (IBC Code 83/90/00, ch.2);

- Confirming that a copy of the International Code for the Construction and Equipment of Ships Carrying Dangerous Chemicals in Bulk, or the equivalent national regulations, has been provided (IBC Code 83/90/00, ch.16);

- Confirming that information relating to the chemical and physical properties of the products to be carried has been provided and that provision has been made for the measures to be taken in an accident (IBC Code 83/90/00, ch.16);

- Confirming that a manual covering procedures for cargo transfer, tank cleaning, gas freeing, ballasting, etc., has been provided (IBC Code 83/90/00, ch.16);

- Confirming that compatibility information as to material of construction, protective linings and coating is provided on board. (IBC Code 83/04 Ch 6).

- After a satisfactory survey a short term  the International Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk was issued and the undersigned Recomends the issuance of its full term certificate

- If survey shows that the condition of the ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

Place and Date of Survey	GR CLASS Surveyor Name – ID No.