| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-NLS | MARPOL ANNEX II (NLS) SURVEYS |

# MARPOL ANNEX II (NLS) SURVEYS

SECTION I General

1.1 Surveyors conducting Marpol 73/78 Annex II surveys with respect to the control of noxious liquid substances in bulk are to be familiar with the applicable provisions of Annex II of the International Convention for the Prevention of Pollution from ships 1973, the Protocol of 1978 and further amended by the Protocol of 1997 relating thereto and, future amendments.

    - Marpol 73/78 Annex II surveys may be carried out in conjunction with the chemical code survey for chemical tankers or on those ships holding a Certificate for the Carriage of Noxious Liquid Substances (NLS Certificate). An initial survey should be held before the ship is put in service and, should include a complete inspection of all the items relating to NLS certificate in order to ensure that the relevant requirements are complied with and that the relevant requirements are complied with and that these items are satisfactory for the service for which the ship is intended.

    - Renewal surveys are required every five years before the certificate in force has expired. During the duration of the certificate are required four annual surveys and one of them must be intermediate survey, as per the Harmonized System of Survey and Certification (HSSC). Annual surveys are to be held within three months before and three months after the anniversary date. Intermediate surveys are to be held within three months before and three months after the second the second or third anniversary date.

  - The surveys should be held within the permitted range dates only.

  - The existing certificate onboard should be sighted to establish the category of survey due.

  - The various arrangements and items of MARPOL 73/78 Annex II equipment to be surveyed will be identified in the “Procedures and Arrangements” (P&A) manual on board. At each survey, the survey checklist NLS- 1 should be completed, and a copy submitted to GR Head Office along with survey reports.

  - Where new installations or major modifications are proposed, plans and P&A manuals should be submitted to GR Head Office for approval before the work is commenced.

During the first survey by GR Class Classified For Standard (GR)onboard, under transfer of class and certification, it should be verified whether the ship already has an existing certificate. If the vessel is already in possession of a certificate issued by a class society, the national administration or, Recognized Organization (RO) duly authorized by the same Administration, the certificate should be sighted to establish whether any survey is due or overdue. The relevant due/overdue survey is to be carried out. If no survey is due, a general examination, equivalent, to an annual survey is to be carried out. The P&A manual may be accepted if it is approved by a class society or a signatory flag state or a RO duly authorized by the same flag state or Administration. In such cases, the manual must be endorsed “This manual been accepted by GR Class  Classified  For  Standard  (GR)on behalf of “flag state” on the basis of

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-NLS | MARPOL ANNEX II (NLS) SURVEYS |

previous approval by “name of society”. A copy of the endorsed page, copy of any approval letter by the previous society and a copy of the full-term certificate should be forwarded to GR Head Office along with survey reports. If a previous certificate is not available or not issued by a major society, then a complete renewal survey should be carried out. The owner should be advised to submit the P&A manual for approval and place a copy on board. The copy on board is to be endorsed “provisionally accepted pending approval by GR Class Classified For Standard (GR)Head Office”. Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

  - On completion of the renewal surveys or the initial survey by GR Class Classified for Standard (GR), Interim certificate is to be issued with validity of five months (except for ships provided with chemical code certificate). GR Head Office will issue the full-term certificate after review of the survey report. On completion of annual or intermediate surveys, the certificate on board should be endorsed.

# SECTION 2 Annual Surveys

  - The purpose of the annual survey is to carry out an examination of the various items of equipment together with tests as considered necessary, all to an extent which, in the Surveyor’s judgment, indicates adequate condition and standard of maintenance until the next due survey.

  - The Surveyors should verify that the certificate and P&A manual on board are in order. The P&A manual should be written in a language understood by the ship’ personnel engaged in the operations described therein, and it is the Owner’s responsibility to provide a copy written in an appropriate language. Similarly system operating instructions and on board maintenance manuals etc., should also be in a language understood by the crew.

  - The owner’s representative must confirm that no unapproved modifications have been carried out to the pollution prevention arrangements and equipment since the last survey.

  - The stringency of the survey should be to the Surveyor’s satisfaction and in general will depend upon the

condition of the equipment and the level of maintenance.

# SECTION 3 Intermediate Surveys

3.1 An intermediate survey should, in addition to the requirements of an annual survey, be a thorough examination and be sufficiently extensive to ensure that the equipment and the associated piping and pumping systems, including any efficient stripping and line blowing arrangements, and tank washing systems as applicable, comply with the requirements of the Regulations, are in accordance with the P&A manual and are in good working order sufficient until the next renewal survey.

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-NLS | MARPOL ANNEX II (NLS) SURVEYS |

# SECTION 4 Renewal Surveys

4.1 The renewal survey should be a thorough and complete examination with efficient stripping tests and other tests where appropriate. On completion the equipment, systems, fittings and arrangements should, given adequate routine maintenance, be expected to remain in good condition and working order until the next renewal survey. At renewal surveys of chemical tankers having products of pollution category should carry out practical efficient stripping tests using water to verify the continued efficiency of the stripping arrangements in at least two cargo tanks. The tanks should be selected at random and the Surveyor may require systems in other tanks to be tested. The tests are to be carried pout in accordance with the procedures described in the P&A manual and should also include any cargo pipeline purging arrangements and the measurement of residues remaining in the lines.

# SECTION 5	Reporting

5.1 The relevant survey checklist Form No. NLS-01 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports. For initial survey Form I-NLS should be used.

5.2 At first surveys by GR, copies of the previous society’s certificates, copy of approval for P&A manual and

list of any outstanding deficiencies, recommendations etc should also be forwarded.

  - On completion of renewal surveys, initial survey or First Survey by GR Class Classified for Standard (GR), an Interim certificate should be issued, valid for five (5) months.

  - On completion of other surveys (annual and intermediate), the existing full-term certificate on board is to be endorsed in the relevant column indicating the date, place and surveyors name and endorsed using the GR seal/stamp. A copy of the endorsed certificate must be submitted to GR Head Office. At annual/intermediate surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the Survey Statement. The Full Term NLS certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the Survey Statement.

  - A Survey Statement should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owners proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or  www.amp.gob.pa. In case of conflict, the surveyor should follow the instruction issued by the flag state

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-NLS | MARPOL ANNEX II (NLS) SURVEYS |

that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

| MARPOL CONVENTION |  |
| --- | --- |
| GR-PM-NLS | MARPOL ANNEX II (NLS) SURVEYS |

# REFERENCES

| MARITIME ADMINISTRATION | NATIONAL REGULATIONS (OR SIMILAR INSTRUMENT) |  |  | WEBSITE |
| --- | --- | --- | --- | --- |
| Panama | 88 | Shipboard Marine Pollution Emergency Plans for Oil and/or Noxious Liquid Substances. | Feb 2022 | https://panamashipregistry.com/circular- category/circulares/ |
|  | 200 | Revision of the IBC/BCH Code and MARPOL 73/78 Annex II | Sep 2020 |  |
| Barbados | - | N.A. | - | https://barbadosmaritime.org/documents/ |
| Sierra Leone | - | - | - | https://slmarad.com/maritime-circulars/ |
| Vanuatu | - | N.A. | - | http://www.vanuatumaritimeships.com/Sa fety_Document.ASPX |
| St. Kitts and Nevis | - |  | - | https://www.skanregistry.com/en/maritim e-circulars |
| Belize | - |  | - | https://www.immarbe.com/circularslist/ |
| Palau | MN 159.1 | Approval For Shipboard Marine Pollution Emergency Plans for Oil And Noxious Liquid Substances | Oct 2018 | https://www.palaureg.com/information- centre/online-library/legislation/ |

  - MARPOL Chapter III, Reg 8

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)

E-mail: info@grclass.com Web: www.grclass.com

Approved by: GM	                                         	                                                                                                                    Page 5 of 5