# SECTION 1

## General

Ship Types: The International Load Line convention 1966 came into force in 21st of July 1968. Accordingly ships whose keel were laid before this date are to be treated as existing ships and all ships whose keel were laid after this date are to be treated as new ships.

Applicability: A Load Line certificate is not required for the following ships:

New ships of load line length less than 24 m Existing ships of less than 150 gross tonnage Pleasure yachts not used for commercial purpose Fishing vessels.

Marking of Load line: As far as practicable free board marks should be made by welding. Where due to operational or safety reasons (in way of fuel oil tanks etc.) the marking may be done by painting, however inthis case the surveyor shall place a recommendation for the welding to be carried out at next dry-dock. Special attention to be paid to load line marks by painting since possibilities of altering the marks are greater here. The Master should be encouraged to maintain a wooden freeboard batten for easy measurement at load line surveys. The freeboard grid letters should be marked as follows:

| Marks | Freeboard grid letters | Timber Freeboard grid letters |
| --- | --- | --- |
| Tropical Fresh water | TF | LTF |
| Fresh water | F | LF |
| Tropical | T | LT |
| Summer | S | LS |
| Winter | W | LW |
| Winter North Atlantic | WNA | LWNA |

Surveyors conducting load line surveys are to be familiar with the applicable provisions of the International Convention on Load lines, 1966 (ICLL 1966), as modified by the Protocol of 1988. The existing certificate on board should be examined to establish the type of survey due.

The various load line items to be surveyed are itemized on the Record of Conditions of Assignment of Freeboards (Form No. LL-02), a copy of which should be available on board the vessel.

Whenever a vessel is surveyed for the first time, during initial survey or renewal survey under change of flag, the freeboard calculations must be submitted, if not, the surveyor shall take all the necessary data to calculate the new freeboard assignments. It is understood that the surveyor will verify that freeboards assigned are marked on the vessel’s side.

The conditions of assignment shall then be sent to GR Panama for revision and procedure.

Any requests for the issue of multiple load line certificates (more than one load line certificate – for tankers) should be referred to GR Head Office.

For load line purposes, Position 1 means at any location on the free board deck (upper most continuous weather deck) and raised quarterdeck and on super structure within 25% length from forward end and Position 2 means on exposed superstructure deck aft of 25% length from forward end.

The requirements for coaming height and thickness are indicated below for easy reference:

| Item |  | Thickness (mm.) |  |  | Height |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Position 1 | Position 2 |
| Cargo coamings | hatch | As | per plans | approved | 600 mm. | 450 mm. |
| Doors |  |  |  |  | 600 mm. | 380 mm. |
| Access coamings | hatch |  |  |  | 600 mm. | 380 mm. |
| Ventilator |  | Dia.<85mm - 6.0 85 -165 mm- Interpolate Dia.>165mm. - 8.5 |  |  | 900 mm. | 760 mm. |
| Air vent pipes |  |  |  |  | Freeboard Deck | Superstructure Deck |
|  |  |  |  |  | 760 mm. | 450 mm. |

## Other items

Bulwarks	- 1.0 m above the deck

Guard Rails	- 1.0 m. above the deck, At least 3 rows of rails

Side Scuttles	- 500 mm. or 2.5% Breadth above the summer load line

# SECTION 2

## First Survey by GR Class Classified For Standard (GR CLASS)  under transfer of Class andCertification

  - Previous certificates issued by an IACS class society or RO duly authorized by the same Flag State Administration.

    - As soon as the vessel arrives in port or at the earliest opportunity, the surveyor should send a copy of the existing load line certificate to GR Head Office. In such cases normally GR Head Office would advise the surveyor to reassign the same freeboards and maintain the existing survey cycle. Any due or overdue survey to be carried out as per the existing cycle and any outstanding survey recommendations imposed by the previous society to be dealt with. Surveyor should then proceed to check the load line marks for verifying freeboards against the certificate. He can then issue an INTERIM Certificate with the same freeboards after satisfactorily completing the following:

      - The Survey Report Form No. LL-01 must be filled out and submitted to GR Head Office.

      - All load line items should be checked for compliance with load line requirements and verified against existing Conditions of Assignment form available on board. If Conditions of Assignment Form (Form No. LL- 02) is not available onboard, a new form LL-02 is to be prepared and copy placed on board. Any

deviations found from the existing Conditions of Assignment form is to be checked for compliance with the LL requirements and the existing Conditions of Assignment form is to be updated as applicable. A copy of the Conditions of Assignment (Form No. LL-02) (previous or new) is to be submitted to GR Head Office along with survey reports.

      - It is to be ensured that no significant change in structure or arrangement has been made that will alter the particulars of the certificate (examples of such changes are, removal or fitting of weather tight doors on main deck & superstructures, removal or fitting of side scuttles below freeboard deck or enclosed superstructures, removal or modification of hatch covers, conversion of ship from one type to another say from oil barge to deck barge, removal of deck / bulkhead etc. which might reduce the structural strength of the ship, removal or fitting of side / stern or bow doors & ramps etc.). In case of such changes being observed the GR Head Office should be informed immediately with a drawing or a sketch with all details showing the modification. GR Head Office may then, require a change in the free board marking.

      - It is to be verified that the stability information (stability booklet, for all ships, damage stability booklets for type–

      - A ship with length greater than 150 M and type–B reduced free board ships greater than 100 M in length) is available onboard, in a language understood by the ships officers and approved by the administration or the previous society. If, for any reason (i.e. change of flag), the booklet needs to be re- approved, it may not be required to submit all the documents required as per the Stability Booklet approval procedures requirements for approval and, the booklet may be re-approved provided that the attending surveyor has verified that, since the date of approval of the booklet, there are no alterations to the ship arrangements that may produce a variation on the stability conditions of the ship. The booklet to be endorsed as appropriate.

      - If no booklet is available on board, or the booklet on board is not approved, the Owners should be advised to submit the booklet for approval at the earliest, according to the stability booklet procedure, in order to be reviewed and approved. The certificate validity is to be restricted to maximum three months. For ships, which have timber load lines assigned, there should be an approved timber deck cargo loading and lashing plan onboard.

      - For deck barges (unmanned/non propelled) in restricted service within sheltered waters (harbour and/or smooth water service), with fixed loading condition, considered Pontoon type according GR Class rules, and as per Intact Stability Code, the stability booklet can be dispensed. Except for new building.

      - Any service restriction imposed on the earlier certificate or by the previous society should be reflected inthe new certificate without any change.

      - Load line marks should be verified for accuracy and the identification marks of the issuing authority to be changed to “OS”.

    - If Owner requests for a change of freeboard or if modifications have been carried out which will affect the freeboards assigned, new freeboard calculations must be submitted to the GR Head Office for approval along with copies of relevant drawings such as general arrangement plan. In some cases, the

stability booklet may also need re-approval in view of changes carried out. New certificate is to be issued assigning the freeboards forwarded by GR Head Office. The Surveyor should ensure that the new freeboards are properly marked on the ship’s side and the previous marks have been properly removed.

    - Load line certificate is generally not to be issued to un-classed ships. The case has to be submitted to GR Head Office for special consideration.

## Previous Certificates not issued by an IACS class society or RO duly authorized by the same Flag State or from other organization not recognized by the GR Class.

    - Any request for issuance of Load Line should be forwarded to GR Head Office. In these cases GR HeadOffice will require to verify the freeboard calculations before accepting the previous assigned freeboards and in such cases the Surveyor must submit Load Line plan showing the conditions of assignment & general arrangement, freeboard computations, Mid ship section drawing and loading manual and stability Booklet incorporating conditions corresponding to the requested freeboard. Based on these documents GR HeadOffice will check the freeboard computations and assign freeboards. The assigned freeboards should be marked on the side of the ship and verified by the surveyor. After this the surveyor shall perform the following:

      - The Survey Report Form No. LL-01 must be filled out and submitted to GR Head Office.

      - All load line items should be checked for compliance with load line requirements. Conditions of Assignment,Form No. LL-02, is to be prepared and copy placed on board. A copy of the new Conditions of Assignment is to be submitted to GR Head Office along with survey reports.

      - It is be verified that the stability information (stability booklet, for all ships, damage stability booklets for type–A ships with length greater than 150 M and type–B reduced free board ships greater than 100 M in length) is available onboard, in a language understood by the ships officers. A copy is to be submitted for approval to GR Head Office. The copy on board should be endorsed as follows: “provisionally approved by GR Class Classified For Standard (GR CLASS) on behalf of (flag) pending approval by GR Class Classified For Standard (GR CLASS) GR Head Office”. For ships, which have timber load lines assigned, there should be an approved timber deck cargo loading and lashing plan onboard.

      - Any known service restriction should be reflected in the new certificate.

      - Load line marks should be verified for accuracy and the identification marks of the issuing authority should be marked as “OS”.

## For new ships, initial survey should be reported on form I-LL

    - On satisfactory completion of above, an INTERIM load line certificate may be issued, valid for 3 months, with approval of stability booklet as an outstanding item.

# SECTION 3

## Annual Load Line Survey

  - General

    - Four annual surveys (Periodical Inspection in ICLL 1966) are required during the duration of the certificate’s validity. Annual surveys are to be held within 3 months before and 3 months after the anniversary date (anniversary date is the expiry date of the certificate without the year).

    - The survey checklist Form No. LL-03 is to be filled up as appropriate and a copy submitted with the reports. The load line certificate is to be endorsed for annual surveys on completion of the survey. A copy of the certificate, clearly showing the endorsement is to be submitted along with the reports. The survey date endorsed on the certificate should be the date of completion of the survey.

## Scope of Survey

    - The validity of the load line certificate and other statutory certificates on board should be verified. It should be verified that the approved stability data is available on board.

    - The load line marks should be verified with the certificate.

    - Weather Tight Doors: Check overall conditions to ensure no excessive corrosion has taken place, especially the gasket retaining channels (door lips) and no distortion has taken place. Check hinges and doorframe for proper closing. Check conditions of rubber gasket - too hard or too soft, worn out, etc. shall be rejected and rubber packing shall not protrude outside the channel. Dogs/Clips/Toggles should be checked for seizure and open-close markings. In case of doubts, passage of light test (Surveyor shall stand inside the compartment and check that no day light passes through the door & edges with the door fully closed) or chalk test on doorframe & rubber gasket may be carried out.

    - Scuttles & Windows- Check for broken glass, perished or missing dead lights and proper closure. Chalk test or hose test may be carried out, if required by Surveyor.

    - Access Hatches – Check condition of coaming, especially lower portion adjacent to deck (hammer testing may be carried out). Check rubber gaskets for wear and loose fitting. Check cover for wastage, for seizure & operate the toggles from inside and outside and check effectiveness of pins for retaining in open position. Passage of light test or chalk test may be carried out, if required by Surveyor.

    - Manholes – Check covers for wastage, missing or worn-out packing, bolts and nuts.

    - Air and sounding pipes – Check pipes for wastage, especially the lower portion near stub pieces. Check for missing protective cover for air pipes in holds. Check for missing caps/ keep chains for sounding pipes.Check condition of air pipe hood for wastage and missing wire gauze diaphragm for oil tank vents. Check condition of float and sealing washer for automatic air vents by dismantling and examining at least 10 percent of the air pipes. Floatability test for floats may be carried out, if required by Surveyor (particularly if repairs/renewals have been carried out to floats). Ensure that vents for oil fuel tanks are fitted with flameproof wire mesh of correct size.

    - Ventilators – Check for corrosion of coaming/casing, proper closure of flaps, missing butterfly nuts & packing and open-shut markings. Operating handle or spindle should be checked for free movement. Check support stays & brackets for wastage/detachment.

    - Bulwarks, Guardrails and lifelines – Check for missing, damaged or broken railing and stanchions. Wasted/detached stays and cap rails should be repaired. Check for missing lifeline stanchions & wasted shoes and worn lifelines.

    - Freeing ports should be examined for proper operation of flaps/shutters and for obstructions. Examine grillsfor wastage/damage.

    - Scuppers & Discharges – Check for clogged deck scuppers, missing spindles/hand wheels, pipes for wastage and overboard valves for wastage/leaks. Operation of local and remote controls must be verified.

    - Skylights, Casings & Funnel flaps to be checked for broken glass, wasted covers, worn or loose gaskets and missing butterfly nuts or toggles.

    - Side/Bow doors – Check for deterioration of hinges, seized dogs and verify proper operation of closing mechanism (hydraulic). Logbook entries, of opening and closing, should be verified. Doors should be hose tested to ensure water tightness.

    - Cargo Hatch covers and coamings – Checking that no unapproved changes have been made to the hatchcovers, hatch coamings and their securing and sealing devices since the last survey.

Where mechanically operated steel covers are fitted, checking the satisfactory condition of:

      - hatch covers.

      - tightness devices of longitudinal, transverse and intermediate cross junctions(gaskets, gaskets lips, compression bars, drainage channels);

      - clamping devices, retaining bars, cleating;

      - chain or rope pulleys;

      - guides;

      - guide rails and track wheels;

      - stoppers, etc.;

      - wires, chains, gypsies, tensioning devices;

      - hydraulic system essential to closing and securing;

      - safety locks and retaining devices.

Where portable covers, wooden or steel pontoons are fitted, checking the satisfactory condition of:

      - wooden covers and portable beams, carriers or sockets for the portable beams, and their securing devices;

      - steel pontoons;

      - tarpaulins;

      - cleats, battens and wedges;

      - hatch securing bars and their securing devices;

      - loading pads/bars and the side plate edge

      - guide plates and chocks;

      - compression bars, drainage channels and drain pipes (if any)

Random checking of the satisfactory operation of mechanically operated hatch covers:

      - stowage and securing in open condition;

      - proper fit, locking and efficiency of sealing in closed condition;

      - operational testing of hydraulic and power components, wires, chains, and link drives.

Checking the satisfactory condition of hatch coamings plating and their stiffeners. Examine cover and coamings for wastage of plating, stiffeners and stays. The hatch covers should be operated and examined for proper operation of opening and closing mechanism and if chain operated examine chain for slackness, worn links and wasted guides. Check the condition of hydraulic system including checking for any leaks, in case of hydraulically operated covers. Examine tracks and guides for wear, distortion and wastage. Check drain channels and pipes for corrosion, blockage and condition of drain valves. Examine quick acting cleats for distortions, slackness, wasted brackets, etc. Check gasket retaining channel for wastage and worn out or loose gaskets. Examine compression bar for wastage and distortion. All hatch covers should be hose tested to ensure weather-tightness. GR Head Office should be consulted where the use of tarpaulins is proposed as a temporary or permanent measure to secure weather-tightness of cargo hatches.

    - Spurling pipe of Chain locker – Check for missing or wasted cover.

# SECTION 4

## (Renewal) Surveys

Renewal surveys are required every five years and before the certificate in force has expired. As far as practicable, surveys should be carried out in tandem with classification special / SAFCON CCC renewal surveys.

  - Cargo Hatch covers and coamings - This survey shall, as a minimum, consist of:

    - a thorough inspection of the items listed in section 3.2.14, and in addition:

    - checking of the satisfactory operation of all mechanically operated hatch covers:

stowage and securing in open condition; proper fit, locking and efficiency of sealing in closed condition;operational testing of hydraulic and power components, wires, chains, and link drives;

    - checking the effectiveness of sealing arrangements of all hatches covers by hose testing or equivalent asnecessary;

    - checking the residual thickness of coamings, steel pontoon or hatch cover plating and stiffening membersas deemed necessary by the Surveyor.

  - A thorough examination of the ships structure, openings & closing appliances is to be made to ensure that the items will, given routine maintenance, remain in satisfactory condition.

  - On satisfactory completion of surveys, an INTERIM Load Line certificate is to be issued valid for 5 months.

# SECTION 5

## Reporting

  - The relevant survey checklists Form No. LL-01, Form No. LL-02 and Form No. LL-03 should be used when carrying out surveys and a copy submitted to GR Head Office along with survey reports.

  - At first surveys by GR, copies of the previous society’s certificates and Conditions of Assignment and copy of approval for stability booklet/grain loading manual/timber loading & lashing plan etc. and list of any outstanding deficiencies, recommendations etc. should also be forwarded.

  - INTERIM Load Line Certificate should be issued, valid for five months on satisfactory completion of Renewal/Periodical Surveys or First Survey by GR Class Classified For Standard (GR CLASS). When survey is incomplete or deficiencies are noted, the INTERIM Certificate should be marked CONDITIONAL and the outstanding survey items/deficiencies must be noted on the reverse of the CONDITIONAL Certificate and on the Surveyor Site Statement Report (previous P24). The certificate validity must be restricted to maximum two months (except where only approval of stability booklet/ grain loading manual/timber loading & lashing plan is outstanding, in which case the validity should be restricted to three months) Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

  - On completion of annual surveys, the existing full-term certificate on board is to be endorsed in the relevantcolumn indicating the date, place and surveyors name and endorsed using the GR seal. A copy of the endorsed certificate must be submitted to GR Head Office along with the Form No. LL-

03. At annual surveys, if surveys cannot be completed or deficiencies are noted, the outstanding items/deficiencies must be entered on the surveyor site Statement report (previous P24). The Full-Term LL certificate should not be endorsed in such cases.

  - When previous recommendations are examined and completed satisfactorily, the items must be deleted on the surveyor site Statement report (previous P24)

  - A narrative survey Statement report (previous P24) should be submitted whenever a survey is incomplete (indicating the reason why the survey could not be completed), deficiencies are noted (details of deficiency noted and the owner’s proposal to deal with the deficiency), repairs are carried out (details of the temporary/permanent repairs carried out) and when existing outstanding recommendations are deleted or postponed.

  - Nothing in this procedure should contradict any administrative or technical requirement expressly communicated by the flag in written on a formal and custom matter. For example, Panama Merchant Marine Circulars or Administrative Resolutions properly posted on www.segumar.com or www.amp.gob.pa or under Barbados Flag https://barbadosmaritime.org/bulletins/. In case of conflict, the surveyor should follow the instruction issued by the flag state that GR Class is working on behalf of unless there is an express authorization in written from the flag to act otherwise.

REFERENCES

| ADMINISTRATION | NATIONAL REGULATION (OR SIMILAR INSTRUMENT) | DATE ISSUED | WEBSITE |
| --- | --- | --- | --- |
| Panama | MMC 152: Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021 | 06/2022 | https://panamashipregistry.com/circular- category/circulares/ |
| Barbados | NA | NA | https://barbadosmaritime.org/bulletins/ |
| Belize | NA | NA | https://www.immarbe.com/circularslist/ |
| Sierra Leone | NA | NA | https://slmarad.com/maritime-circulars |
| Palau | NA | NA | https://www.palaureg.com/information- centre/online-library/marine-documents/ |
| Cameroon | NA | NA | https://mintransports.net/fr/documentation/i nstruments-and-laws/ |
| Honduras | NA | NA | https://merchantmarine.gob.hn/aspectos- legales/circulares-y-notificaciones/ |
| Bolivia | NA | NA | https://www.ribb.gob.bo/web/site/disposicion es |
| Nigeria | NA | NA | https://nimasa.gov.ng/press-center/marine- notice-adverts/ |
| Togo | NA | NA | https://www.togoregistrar.com/circulars |
| Colombia | NA | NA | https://www.dimar.mil.co/normatividad |
| Guinea Bissau | NA | NA | NA |