# GENERAL

## Application

    - The requirements apply to all Mobile Offshore Drilling Units after their construction.

    - The requirements apply to surveys of the hull, structure, equipment, and machinery subject to classification.

      - Each unit should be subject to the surveys specified below:

An initial survey before the unit is put into service or before the Certificate required under this section of the Code is issued for the first time, which should include a complete survey of its structure, equipment, fittings, arrangements, and material in so far as the unit is covered by the Code. This survey should be such as to ensure that the structure, equipment, fittings, arrangements, and material fully comply with the applicable provisions of the Code.

Periodical surveys at intervals specified by the Administration, but not exceeding five years, which should be such as to ensure that the structure, equipment, fittings, arrangements, and material fully comply with the applicable provisions of the Code.

Intermediate surveys at intervals specified by the Administration, but not exceeding thirty months, which should be such as to ensure that the structure, fittings, arrangements, safety equipment and other equipment fully comply with the applicable provisions of the Code and are in good working order. Such intermediate surveys should be endorsed on the Certificate issued under the provisions of this section.

A survey either general or partial according to the circumstances should be made every time a defect is discovered, or an accident occurs which affects the safety of the unit or whenever any significant repairs or alterations are made. The survey should be such as to ensure that the repairs or alterations have been effectively made, are in all respects satisfactory and fully comply with the applicable provisions of the Code.

      - These surveys should be carried out by officers of the Administration. The Administration may, however, entrust the surveys either to surveyors nominated for the purpose or to organizations recognized by it. In every case the Administration concerned should fully guarantee the completeness and efficiency of the surveys.

      - After any survey under this section has been completed no significant change should be made in the structure, equipment, fittings, arrangements, or material covered by the survey, without the sanction of the Administration, except the direct replacement of such equipment and fittings for the purpose of repair or maintenance.

      - A Certificate may be issued, after survey in accordance with this section, either by officers of the Administration or by any person or organization duly authorized by it. In every case the Administration assumes full responsibility for the Certificate.

      - The Certificate should be drawn up in the official language of the issuing country in the form corresponding to the model given in the Appendix to the Code. If the language used is neither English nor French, the text should include a translation into one of these languages.

      - Any exemptions granted under 1.4 of MODU CODE should be clearly noted on the Certificate.

      - A Certificate should be issued for a period specified by the Administration and should not exceed five years from the date of issue.

      - No extension of the five-year period of validity of the Certificate should be permitted.

      - A Certificate should cease to be valid if significant alterations have been made in the construction, equipment, fittings, arrangements, or material specified by the Code without the sanction of the Administration, except the direct replacement of such equipment or fittings for the purpose of repair or maintenance, or if surveys as specified by the Administration under the provisions of 1.6.1 are not carried out.

      - A Certificate issued to a unit should cease to be valid upon transfer of such a unit to the flag of another country.

      - The privileges of the Code may not be claimed in favor of any unit unless it holds a valid Certificate.

## Definition

    - A Ballast Tank is a tank which is used primarily for salt water ballast.

    - Spaces are separated compartments.

    - Representative Spaces are those which are expected to reflect the condition of other Spaces of similar type and service and with similar corrosion protection systems; when selecting Representative Spaces account should be taken of the service and repair history on board and identifiable critical and/or Suspect Areas.

    - Suspect Areas are locations considered by the Surveyor to be prone to rapid wastage.

# SPECIAL SURVEY

## Schedule

    - Special Surveys (Renewal Surveys) of hull, structure, equipment, and machinery are to be carried out at 5 year intervals to renew the Classification Certificate(s).

    - The first Special Survey is to be completed within 5 years from the date of the initial classification survey and thereafter within 5 years from the credited date of the previous Special Survey. However an extension of class of 3 months maximum beyond the 5th year can be granted in exceptional circumstances. In this case the next period of class will start from the expiry date of the Special Survey before the extension was granted.

    - For survey completed within 3 months before the expiry date of the Special Survey, the next period of class will start from the expiry date of the Special Survey. For Survey completed more than three months before the expiry date of the Special Survey, the period of class will start from the survey completion date.

    - The Special Survey may be commenced at the 4th Annual Survey and be progressed with a view to completion by the 5th anniversary date.

    - When considered necessary by the GR the interval between Special Surveys may be reduced.

    - Special Survey requirements of units of unusual design, in lay-up or in unusual circumstances will be determined on individual basis.

    - At the request of the Owner, and upon GR’s approval of the proposed arrangements, a system of Continuous Survey may be undertaken whereby the Special Survey requirements are carried out in regular rotation in accordance with the Rules of GR to complete all the requirements of the particular Special Survey within a five-year period. Any defects that may affect classification found during the survey are to be reported to the GR and dealt with to the satisfaction of the Surveyor.

## Scope

    - The Special Surveys shall include, in addition to Annual Survey and Drydocking Survey (or equivalent) requirements, the following examinations, tests, and checks of sufficient extent to ensure that the hull, structure, equipment, and machinery are in satisfactory condition and that the Mobile Offshore Drilling Unit is fit for its intended purpose for the new period of class of 5 years to be assigned subject to proper maintenance and operation and the Periodical Surveys carried out at the due dates.

## Special Survey No. 1 - The following hull structure and equipment items are to be examined.

    - All Drilling Units

- The hull or platform structure including tanks, watertight bulkheads and deck, cofferdams, void spaces, sponsons, chain lockers, duct keels, helicopter deck and its supporting structure, machinery spaces, peak spaces, steering gear spaces, and all other internal spaces are to be examined externally and internally for damage, fractures, or excessive wastage. Thickness gauging of plating and framing may be required where wastage is evident or suspected.

- All tanks, compartments and free-flooding spaces throughout the drilling unit are to be examined externally and internally for excess wastage or damage.

- Internal examinations of spud cans and mats are to be specially considered.

- Watertight integrity of tanks, bulkheads, hull, decks and other compartments is to be verified by visual inspection.

- Suspect areas may be required to be tested for tightness, non-destructive tested or thickness gauged.

- Tanks and other normally closed compartments are to be ventilated, gas freed and cleaned as necessary to expose damages and allow meaningful examination and thickness gauged in case of excessive wastage.

- Internal examination and testing of void spaces, compartments filled with foam or corrosion inhibitors, and tanks used only for lube oil, light fuel oil, diesel oil, or other non-corrosive products may be waived provided that upon a general examination the Surveyor considers their condition to be satisfactory. External thickness gauging may be required to confirm corrosion control.

- Structures such as derrick substructure and supporting structure, jack-houses, deck houses, superstructures, helicopter landing areas, raw water (sea water intake) towers and their respective attachments to the deck or hull.

- Windlass and attachments of anchor racks and anchor cable fairleads.

- Foundations and supporting headers, brackets, and stiffeners for drilling related apparatus, where attached to hull, deck, superstructure or deck house.

- Thickness gaugings are to be carried out where wastage is evident or suspect.

    - Surface-type Units

- Structural appendages and ducts for positioning units.

    - Self-Elevating Units

- All legs, including chords, diagonal and horizontal braces, gussets, racks, joints, together with leg guides are to beexamined. Tubular or similar type legs are to be examined externally and internally, together with internal stiffeners and pinholes as applicable.

- Structure in, around and under jack-house and leg walls. Non-destructive testing of suspect areas may be required.

- Leg jacking or other elevating systems externally.

- Leg connections to bottom mats or spud cans, including non-destructive testing of leg connections to mats or spud cans.

- Jetting piping systems or other external piping, particularly where penetrating mats or spud cans.

- Spud cans or mats. Where the spud cans or mat are partly or entirely obscured below the mud line where the Special Survey is otherwise being completed, consideration will be given to postponement of the examinationsuntil the next Rig move.

    - Column-Stabilized Units

- Connections of columns and diagonals to upper hull, structure or platform and lower hull, structure or pontoons. Joints of supporting structure including diagonals, braces and horizontals, together with gussets and brackets. Internal continuation or back-up structure for the above.

Non-destructive examination may be required of suspect areas.

## Special Survey No. 2 and Subsequent Special Surveys - Hull, Structure and Equipment

These Surveys are to be at least as comprehensive as Special Survey No. 1, with special attention being given to the condition and thickness of material in high corrosion areas. Representative gaugings will be required. Thesegaugings will be specified in advance by GR. Special attention should be paid to splash zones on structure, legs or related structure, and in ballast tanks, pre-load tanks, free-flooding spaces, spud cans and mats.

## Special Surveys - Machinery

Machinery installations of all types of drilling Units are to undergo Special Surveys at intervals similar to those for Special Survey on the hull, in order that both may be recorded at approximately the same time.

    - Non-Self-Propelled Units

In addition to the requirements for Annual Surveys, at each Special Survey, special attention is to be given to the following items as applicable:

- All openings to the sea, including sanitary and other overboard discharges, together with cocks and valves connected therewith are to be examined internally and externally while the Unit is in drydock, or at the time of underwater examination in lieu of drydocking, and the fastenings to the shell plating are to be renewed when considered necessary by the Surveyor.

- Pumps and pumping arrangements, including valves, cocks, pipes and strainers are to be examined. Non-metallic flexible expansion pieces in the main saltwater circulating system are to be examined internally and externally.

The Surveyor is to be satisfied with the operation of the bilge and ballast systems. Other systems are to be tested as considered necessary.

- The foundations of machinery are to be examined.

- Heat exchangers and other unfired pressure vessels within the scope of classification are to be examined, opened out or thickness gauged and pressure tested as considered necessary, and associated relief valves proved operable. Evaporators that operate with a vacuum on the shell need not be opened, but may be accepted on basis of satisfactory external examination and operational test or review of operating records.

    - Self-Propelled Units

In addition to the requirements for non-propelled units, the main and auxiliary propulsion machinery, including associated pressure vessels should be surveyed. In addition, examination of the steering machinery is to be carried out, including an operational test and checking or relief-valve settings. The machinery may be required to be opened for further examination as considered necessary by the Surveyor.

2.5.3. Units with Propulsion - Assist or Dynamic Position

Propulsion-assist and dynamic positioning equipment should be surveyed on the basis of Special Periodical Survey- Machinery in accordance with the requirements of GR.

## Special Survey - Electrical Equipment

In addition to the requirements for Annual Surveys, at each Special Survey, special attention is to be given to the following items as applicable:

- Fittings and connections on main switchboards and distribution panels are to be examined, and care is to be taken to see that no circuits are over fused.

- Cables are to be examined as far as practicable without undue disturbance of fixtures.

- All generators are to be run under load, either separately or in parallel. Switches and circuit breakers are to be tested.

- All equipment and circuits are to be inspected for possible development of physical changes or deterioration. The insulation resistance of the circuits is to be measured between conductors and between conductors and ground and these values compared with those previously measured.

- Electrical auxiliaries installed for vital purposes, generators and motors are to be examined and their prime movers opened for inspection. The insulation resistance of each generator and motor is to be measured.

- The windings of main propulsion generators and motors are to be thoroughly examined and found or made dry and clean. Particular attention is to be paid to the ends of all windings of stators and rotors.

## Special Survey - Shipboard Automatic and Remote-Control Systems

In addition to the requirements of Annual Surveys the following parts are to be examined:

- Control Actuators: All mechanical, hydraulic, and pneumatic control actuators and their power systems are to be examined and tested as considered necessary.

- Electrical equipments: The insulation resistance of the windings of electrical control motors or actuators is to be measured, with all circuits of different voltages above ground being tested separately to the Surveyor's satisfaction.

- Unattended Plants: Control systems for unattended machinery spaces are to be subjected to dock trials at reduced power on the propulsion engine to ensure the proper performance of all automatic functions, alarms, andsafety

systems.

## Special Survey - Special Features (All Types)

Mobile Offshore Drilling Units may have many items of machinery and electrical equipment not found on conventional vessels. Certain of these items are required for classification even if the unit is without propulsion machinery. Items to be especially examined and reported upon at all Special Surveys are as follows:

    - Hazardous Areas - Enclosed hazardous areas such as those containing open active mud tanks, shale shakers, degassers and desanders are to be examined and doors and closures in boundary bulkheads verified as effective. Electric lighting, electrical fixtures, and instrumentation are to be examined, proven satisfactory and verified as explosion-proof or intrinsically safe. Ventilating systems including ductwork, fans, intake and exhaust locations for enclosed restricted areas are to be examined, tested and proven satisfactory. Ventilating air alarm systems to be proven satisfactory. Electrical motors are to be examined including closed-loop ventilating systems for large D-C motors. Automatic power disconnect to motors in case of loss of ventilating air is to be proved satisfactory.

    - Remote Shutdown Arrangements - Remote shutdown for fuel-oil transfer service pumps and ventilating equipment, together with oil tank outlet valves where required to be capable of being remotely closed are to be proved satisfactory. Emergency switch(s) for all electrical equipment including main and emergency generators, except alarm and communication systems and lighting in vital areas such as escape routes and landing platforms, are to be proved satisfactory.

    - Fire Fighting Apparatus where included in the Societies’ Rules - A general examination of the fire detection and extinguishing apparatus is to be made in order that the Surveyor may be satisfied with its efficient state. The following items are to be especially examined:

- Fire hoses, nozzles, and spanners at each fire station.

- Servicing of all portable soda-acid and foam extinguishers.

- Weighing and re-charging as necessary of all dry chemical and CO 2 extinguishers.

- Fire pumps and piping including operation and capacity.

- Alarm systems

    - Self Elevating Systems - On self elevating type Mobile Offshore Drilling Units, the elevating systems are to be examined and reported on. Pinions and gears of the climbing pinion gear train of rack and pinion systems are to be examined, as far as practicable, to the Surveyor's satisfaction by an effective crack detection method.

    - Piping Systems - Piping systems used solely for drilling operations and complying either with GR’s requirements or a recognized standard are to be examined, as far as practical, operationally or hydrostatically tested to working pressure, to the satisfaction of the Surveyor.

    - Miscellaneous - Bilge alarm systems, if fitted, to be tested.

# ANNUAL SURVEY

## Schedule

Annual Surveys are to be held within 3 months before or after each anniversary date from the date of the initial classification survey or of the date credited for the last Special Survey.

## Scope

The survey consists of an examination for the purpose of ensuring, as far as practicable, that the hull, structure, equipment, and machinery are maintained in a satisfactory condition.

## Annual Survey - Hull, Structure and Equipment

    - At each Annual Survey the exposed parts of the hull, deck, deck house, structures attached to the deck, derrick substructure, including supporting structure, accessible internal spaces, and the applicable parts listed below are to be generally examined and placed in satisfactory condition as found necessary.

    - The Surveyors are to be satisfied at each Annual Survey that no material alterations have been made to the unit, its structural arrangements, subdivision, superstructure, fittings, and closing appliances upon which the stability calculations or the load line assignment is based.

    - All Drilling Units

      - Accessible hatchways, manholes and other openings.

      - Machinery casings and covers, companionways, and deck houses protecting openings.

      - Portlights together with deadcovers, cargo ports and similar openings in hull sides, ends, or in enclosed superstructures.

      - Ventilators, tank vent pipes together with flame screens, and overboard discharges from enclosed spaces.

      - Watertight bulkheads and end bulkheads of enclosed superstructures.

      - Closing appliances for all the above, including hatchcovers, doors check valves, together with their respective securing devices, dogs, sill, coamings and supports.

      - Freeing ports together with bars, shutters and hinges.

      - Windlass and attachment of anchor racks and anchor cables.

      - Protection of the crew, guard rails, lifelines, gangways, and deck houses accommodating crew.

    - Surface-Type Units

The hull and deck structure around the drilling well (moon-pool) and in vicinity of any other structural changes in section, slots, steps, or openings in the deck or hull and the back-up structure in way of structural members or sponsons connecting to the hull.

    - Self-Elevating Units

Jack-house structures and attachments to upper hull or platform. Jacking or other elevating systems and

leg guides, externally. Legs as accessible above the waterline, Plating and supporting structure in way of leg wells.

    - Column-Stabilized Units

Columns, diagonal and horizontal braces together with any other parts of the upper hull supporting structure as accessible above the waterline.

Note: At the 1st Annual Survey after construction, Column Stabilized and Self Elevating Units may be subject to examination of major structural components including non-destructive testing, as deemed necessary by GR. If the GR deems such survey to be necessary, the extent should be agreed to by the GR and the Owner or operator prior to commencement of the Survey.

## Annual Survey - Machinery

    - Self-Propelled Units

A general examination of main and auxiliary engines, boilers, steering machinery, pumps, pipings, electrical installation including those in hazardous areas, and fire extinguishing apparatus.

    - Non-Self Propelled Units

A general examination of items required for classification such as auxiliary machinery, pumps, piping, electrical installation in hazardous areas and fire extinguishing apparatuses to be carried out.

## Annual Survey - Electrical Equipment

A general examination of electrical machinery, the emergency sources of electrical power, the switchgear, and other electrical equipment, including operation of same. The operation of the emergency sources of power, including their automatic operation, is to be confirmed as far as practicable.

## Annual Survey - Shipboard Automatic and Remote-Control Systems

A general examination of the automatic and remote-control system is to be made to the Surveyor's satisfaction. The machinery-space fire-detection and bilge water-level alarms are to be tested to confirm satisfactory operation.

## Annual Survey - Special Features

A general examination of hazardous areas, remote shutdown arrangements, firefighting apparatus were includedin the Societies’ Rules, self-elevating systems, piping systems, and bilge systems is to be made.

# SURVEY OF THE OUTSIDE OF UNIT’S BOTTOM AND RELATED ITEMS

## Schedule

    - The outside of the unit’s bottom and related items are to be examined two times in any five (5) year period, with an interval not exceeding three (3) years between examinations. For units operating in salt water for less than six (6) months each year, the survey interval may be increased by GR in coordination with flag state.

    - Consideration may be given at the discretion of GR, to any special circumstances justifying an extension of the interval.

    - Proposals for alternative means of examining the unit’s bottom and related items while afloat may be considered, provided they are in general agreement with Appendix A. in case of Panama merchant marine circular No.289 shall be followed.

## Parts to be Examined

    - Surface-type Units (ship or barge type units)

- External surfaces of the hull, keel, stem, stern frame, rudder, nozzles, and sea strainers are to be selectively cleaned to the satisfaction of the attending Surveyor and examined together with appendages, the propeller, exposed parts of stern bearing assembly, rudder pintle and gudgeon securing arrangements, sea chest and strainers, and their fastenings.

- Propeller shaft bearing, rudder bearing, and steering nozzle clearances are to be ascertained and reported upon.

    - Self-Elevating Units

- External surfaces of the upper hull or platform, spud cans, mat, underwater areas of legs, together with their connections as applicable, are to be selectively cleaned to the satisfaction of the attending Surveyor and examined.

- At each Drydocking Survey or equivalent, after Special Survey No. 2, the Surveyor is to be satisfied with the condition of the internal structure of the mat or spud cans. Leg connections to mat and spud cans are to be examined at each Drydock Survey or equivalent. Non-destructive testing may be required of areas considered tobe critical by the GR or found to be suspect by the Surveyor.

## Column-Stabilized Units

- External surfaces of the upper hull or platform, footings, pontoons or lower hulls, underwater areas of columns, bracing and their connections, sea chests, and propulsion units as applicable, are to be selectively cleaned and examined to the satisfaction of the attending Surveyor.

Non-destructive testing may be required of areas considered to be critical by the GR or found to be suspect by the Surveyor.

## Ballast Spaces

In conjunction with Drydocking Surveys (or equivalent) after Special Survey No. 1 and between subsequent Special Surveys, the following ballast spaces are to be internally examined, thickness gauged, placed in satisfactorycondition as found necessary, and reported upon. If such examination reveals no visible structural defects, the examination may be limited to a verification that the corrosion control arrangements remain effective.

    - All Units

Particular attention is to be given to corrosion control systems in ballast spaces, free-flooding areas and other locations subjected to sea water from both sides.

    - Surface type units

One peak tank and at least two other representative ballast tanks between the peak bulkheads used primarily for water ballast.

    - Self elevating units

Representative ballast tanks or free-flooding compartments in mat or spud cans, if accessible, and at least two representative hull pre-load tanks.

    - Column stabilized units

Representative ballast tanks in footings, lower hulls, or free-flooding compartments as accessible, and at least two ballast tanks in columns or upper hull, if applicable.

# TAILSHAFT SURVEYS

Refer to the schedule in TD-SP-T for tailshaft surveys.

# BOILERS SURVEYS

## Schedule

Boiler Surveys are to be carried out according to the schedule found in TD-SP-B.

## Scope

    - At each Boiler Survey the boilers, superheaters, and economizers are to be examined internally (water- steam side) and externally (fire side).

    - Boiler mountings and safety valves are to be examined at each Boiler Survey and opened as considered necessary by the Surveyor.

    - The proper operation of the safety valves is to be confirmed at each survey.

    - When considered necessary by the Surveyor, the boilers and superheaters are to be subjected to hydrostatic pressure test.

# SURVEY PREPLANNING AND RECORD KEEPING

  - A specific Survey Program for Special Surveys and Special Continuous Surveys must be worked out

in advance of the Special Survey by the Owner in cooperation with the Classification GR. The Survey Program shall be in written format.

  - Plans and procedures for Drydocking Surveys (or Underwater Inspection in Lieu of Drydocking Survey per Appendix A) are to be submitted for review in advance of the survey and made available on board. These should

include drawings or forms for identifying the areas to be surveyed, the extent of hull cleaning, non-destructive testing locations (including NDT methods), nomenclature, and for the recording of any damage or deterioration found. Submitted data, after review by GR, will be subject to revision if found to be necessary in light of experience.

# OCCASIONAL SURVEYS

## Damage Survey

    - It is the responsibility of the Owner/operator of the unit to report to the GR without delay any damage, defect or breakdown, which could invalidate the conditions for which a classification has been assigned so that it may be examined at the earliest opportunity by GR’s Surveyor(s). All repairs found necessary by the Surveyor are to be carried out to his satisfaction.

## Repairs

    - Where repairs to hull, legs, columns or other structures, machinery or equipment, which affect or may affect classification, are planned in advance to be carried out, a complete repair procedure including the extend to proposed repair and the need for Surveyors attendance is to be submitted to and agreed upon by the GR reasonably in advance. Failure to notify GR, in advance of the repairs, may result in suspension of the unit’s classification until such time as the repair is redone or evidence submitted to satisfy the Surveyor that the repair was properly carried out. This applies also to repairs during voyage or on site.

    - The above is not intended to include maintenance and overhaul to hull, other structures, machineryand equipment in accordance with recommended manufacturers procedures and established marine practice and which does not require GR approval; however, any repair as a result of such maintenance and overhauls which affects or may affect classification is to be noted in the ships log and submitted to the Surveyor.

## Reactivation Surveys

8.3.1In the case of units which have been out of service for an extended period, the requirements for reactivation surveys will be specially considered in each case with due regard given to the status of surveys at the time of the commencement of the lay-up period, the length of the period, and conditions under which the unit had been maintained during that period.

## Alterations

8.4.1 No alterations which may affect classification are to be made to the hull or machinery of a classed unit unless plans of proposed alterations are submitted and approved by the GR before the work of alterations is commenced. Such work is to be carried out in accordance with approved plans and tested on completion as required by the Rules

and to the satisfaction of the Surveyor.

## Welding and Replacement of Materials

    - Welding of steels, including high strength structural steel, is to be to the satisfaction of GR.

    - Welding or other fabrication performed on steels of special characteristics or repairs or renewals of such steel or in areas adjacent to such steel is to be accomplished with procedures approved by the GR considering the special materials involved. Substitution of steels differing from those originally installed is not tobe made without approval by GR.

    - The GR may reference IACS Recommendations No. 11 - "Materials Selection Guideline for Mobile Offshore Drilling Units" when considering suitable replacement materials.

## Lifting devices

  - Cranes

8.1.1 Each crane, including its supporting structure, which is used for the transfer of material, equipment or personnel between the unit and attending vessels should be of a design and construction to the satisfaction of the Administration and adequate for the service intended in accordance with the requirements of a recognized classification society or with national or international standards or codes.

    - Cranes should be so located and protected as to reduce to a minimum any danger to personnel, due regard being paid to moving parts or other hazards. Their design should have regard to the materials used in construction, the working conditions to which they will be subjected and the environmental conditions. Adequate provisions should be made to facilitate cleaning, inspection and maintenance.

    - Consideration should be given to the failure mode for each crane in the event of extreme overload so that the crane operator is exposed to minimum danger.

    - An officer of the Administration or a duly authorized person or organization should survey the installation of each crane, with particular regard to its supporting structure.

    - After each crane has been erected on board, and before it is placed in service, operational and load tests should be conducted. These tests should be witnessed and verified by an officer of the Administration or a duly authorized person or organization. A record of these tests and other information concerning initial certification should be readily available.

    - Each crane should be examined at intervals not exceeding 12 months. It should be further tested and recertified, at intervals not exceeding 5 years, or after substantial alteration or repairs. These tests

should be witnessed and verified by an officer of the Administration or a duly authorized person or organization. A record of these examinations, tests and certifications should be readily available.

    - Cranes used for loading and discharging of offshore supply vessels should be furnished with rating tables or curves which take into account the dynamics associated with the unit's and vessel's motions.

    - Except when loads are determined and marked prior to lifting, each crane should be fitted, to the satisfaction of the Administration, with a safety device to give the crane operator a continuous indication of hook load and rated load for each radius. The indicator should give a clear and continuous warning when approaching the rated capacity of the crane.

    - The Administration should consider the installation of limit switches to provide for the safeoperation of the crane.

    - A crane manual should be provided for each crane and should be readily available. This manual should contain full information concerning:

      - design standard, operation, erection, dismantling and transportation;

      - all limitations during normal and emergency operations with respect to safe working load, safe working moment, maximum wind, maximum heel and trim, design temperatures and braking	systems;

      - all safety devices;

      - diagrams for electrical, hydraulic and pneumatic systems and equipment;

      - materials used in construction, welding procedures and extent of non-destructive testing; and

      - guidance on maintenance and periodic inspection.

  - Personnel lifts

    - Personnel lifts should be of a design acceptable to the Administration and adequate for the service intended.

    - The construction and installation should be surveyed by an officer of the Administration or a duly authorized person or organization. The inspections should be carried out on installation and at intervals not exceeding 12 months and certificates or reports should be readily available.

    - Each lift car in a column of a column-stabilized unit should provide for an emergency exit with an escape ladder in the hoistway.

  - Drilling derricks

The design of each drilling derrick and its supporting structure should be to the satisfaction of the Administration. The rated capacity for each reeving should be included in the operating manual.

- Atmosphere testing instrument for enclosed spaces

  - Each unit should carry an appropriate portable atmosphere testing instrument or instruments*. As a minimum, these should be capable of measuring concentrations of oxygen, flammable gases or vapours, hydrogen sulphide and carbon monoxide prior to entry into enclosed spaces**. Instruments carried under other requirements may satisfy this regulation. Suitable means should be provided for the calibration of all such

instruments.

* Refer to the Guidelines to facilitate the selection of portable atmosphere testing instruments for enclosed spaces as required by SOLAS regulation XI-1/7 (MSC.1/Circ.1477).

** Refer to the Revised recommendations for entering enclosed spaces aboard ships (resolution A.1050(27)).

  - Such instruments should be in addition to those provided with the unit's firemen's outfits.

# PROCEDURAL REFERENCES:

See “GR Rules and Regulations”

See “Mobile offshore Drilling Unit Code”

See “Flag state circulars or technical notes and GR Circulars”

# RECORDS:

Revised specifications for the construction and equipment of mobile offshore drilling units form No. MODU-01.

When survey is incomplete or deficiencies are noted, CONDITIONAL Certificate may be issued, and the outstanding

items/deficiencies must be entered into “Survey Statement” into ORION MIS.

Conditional Certificates may only be issued provided that GR Class Head Office has obtained authorization from the flag and instructions have been issued by Head Office to the local surveyor. Upon rectification of the deficiency or deficiencies, Head Office should notify this fact to the flag state that authorized the conditional certificate.

# REFERENCES (FLAG ADMINISTRATION REQUIREMENTS)

GR CLASS Will apply all national regulations (or similar instrument) as applicable in all services provided as required. Instructions regarding above are available in “Flag Authority Administration” >“Country”>”Circulars” in ORION MIS.

  - www.imo.org

  - www.ilo.org

  - https://panamashipregistry.com/circular-category/circulares/

  - https://barbadosmaritime.org/documents/bulletins/

  - Maritime Circulars - Sierra Leone Maritime Administration (slmarad.com)

  - Vanuatu Maritime Services Safety Forms (vanuatumaritimeships.com)

  - Maritime Circulars (skanregistry.com)

  - https://nimasa.gov.ng/?s=notice

Panama

152	Survey Guidelines under the Harmonized System of Survey and Certification (HSSC), 2021, June, 2022

289	Bottom Surveys of Mobile Offshore Units (Drilling and non-Drilling) certificated under MODU Code adoption, April, 2014

328	Validity of some certificates for MODU/MOU, March, 2016

289	Bottom Surveys of Mobile Offshore Units (Drilling and non-Drilling) certificated under MODU Code adoption, April, 2014

150	MODU Annual Safety Inspection, July, 2007

87	MODU Code 1979 & 1989, April, 1994

82	MODU Code 1989, January, 1993