# PURPOSE

  - Further amendments to MARPOL Annex VI, adopted at MEPC 70, establish a mandatory IMO Data Collection System (DCS) for the collection and reporting of fuel oil consumption data for each type of fuel used by a ship, by means of Resolution MEPC.278(70). As such, now the SEEMP must be developed in two parts, as described below:

    - Part I – ship management plan that provides a possible approach for monitoring ship and fleet energy-efficiency performance over time, applicable to ships of 400 gross tonnage and above engaged on international voyages.

    - Part II – methodologies used to collect fuel oil consumption data applicable to ships engaged on international voyages. SEEMP Part II should be prepared as an attachment to the main SEEMP or can be issued separately from SEEMP I.

  - Under the system, ships over 5000 GT will be required to collect fuel consumption data under certain methodologies for each type of fuel they use. The data will be reported to the flag Administration or Recognized Organization after the end of each calendar year and the flag Administration (or recognized authorization), after proper verification, taking into account the guidelines of MEPC.292(71) and having determined that the data has been reported in accordance with the requirements, will issue a Statement of Compliance to the ship. Flag Administrations will be required to subsequently transfer this data to the IMO Ship Fuel Consumption Database.

# RESPONSIBILITY

  - The Technical Department will review the Data Collection Plan (DCP) under the following timeline scheme:

    - 31st Dec. 2018: Deadline for completion and SEEMP Part II assessed by GR , who will issue the Confirmation of Compliance (CoC), according to MARPOL Annex VI, Reg. 5.4.5

    - 1st Jan 2019: Start of first Reporting Period or first calendar year.

    - 31st Dec. 2019: End of first Reporting Period or first calendar year.

    - 31st January 2020 (and yearly): Companies to create annual FOC (Fuel Oil Consumption) report and submit to GR  not later than March 31st, 2020 (and yearly).

    - 31 May 2020 (and yearly): GR or flag state to issue On-board Statement of Compliance on the FOC report.

# SCOPE OF APPLICATION

  - IMO DCS will apply to ships of 5,000 gross tonnage and above international voyages, for which the Ship Energy Efficiency Management Plan (SEEMP) is required to be retained on board. It is not applied to ships not propelled by mechanical means, and platforms including FPSOs, FSUs and drilling rigs, regardless of their propulsion.

# PROCEDURAL DESCRIPTION

  - From the allowable data collection methods, the IMO Data Collection System (DCS) requires the collection of data on the total annual fuel oil consumption. Apart from berthing and travelling at sea, this data consists of fuel consumption due to activities including but not limited to anchoring, drifting, bunkering/de-bunkering, supply stops, maintenance, cargo loading/offloading, and ballast and commercial purposes.

  - To collect the required data a new SEEMP Part II must be developed to outline the methodology used to collect fuel oil consumption data. The SEEMP Part II shall include information of fuel oil consumption by the main engines, auxiliary engines, gas turbines, boilers and inert gas generator, for each type of fuel oil consumed, regardless of whether a ship is underway or not as well as on the methods to measure distance travelled, hours underway and other information.

  - SEEMP Part II must be developed following the IMO MEPC.282(70), “2016 Guidelines for the Development of a Ship Energy Efficiency Management Plan (SEEMP)’’

  - REVIEW OF SEEMP

The SEEMP II and Data Collection Plan (DCP) is to be reviewed by GR  for the approval process. All approvals must be performed by December 31, 2018.

    - For items to be entered in SEEMP PART II, please refer to the Appendix 2 of the SEEMP guidelines, MEPC.282(70).

Accordingly, the basic structure of SEEMP II is represented as following nine sections:

- Ship particulars

- Record of revision of Fuel Oil Consumption Data Collection Plan

- Ship engines and other fuel oil consumers and fuel oil types used

- Emission factor

- Method to measure fuel oil consumption

- Method to measure distance travelled

- Method to measure hours underway

- Processes that will be used to report the data to the Administration

- Data quality

  - After review of the SEEMP II by technical staff, GR  will issue the Confirmation of Compliance (CoC), according to Regulation 5.4.5 of MARPOL Annex VI. This document is to be retained on board and will remain valid for the life of the ship, except when the IEEC becomes invalid or where a major conversion affects the data collection methodology or reporting processes, needing a revision of the SEEMP Part II.

  - VERIFICATION OF ANNUAL REPORT

    - Marpol Annex VI, regulation 22A.7 specifies that the data shall be verified according to procedures established by the Administration, or recognized organization considering the Guidelines to developed by the Organization, MEPC.292(71). This information will be analyzed by GR  technical department under a data verification procedure to ensure the reliability of the collected data while minimizing the costs and associated burdens to the ship and the Administration.

    - A data verification procedure showed below should ensure the reliability of the collected data while minimizing the costs and associated burdens to the ship and the Administration.

    - GR  will verify the data for compliance with the requirements, issue the Statement of Compliance (SoC), submit the data either to the Flag state or to the Organization (depending on the requirements of the flag state who authorized GR ) and perform other actions authorized by the Administration with respect to the IMO Ship Fuel Oil Consumption Database.

# PROCEDURE

      - Verification procedure is guided mainly by means of MARPOL Resolution MEPC.292(71) and will include among others, the following not exhaustive items:

        - a copy of the ship's Data Collection Plan

        - summaries of bunker delivery notes (BDNs)

        - summaries of disaggregated data of fuel oil consumption, distance travelled and hours underway.

        - information to demonstrate that the ship followed the Data Collection Plan set out in its SEEMP including information on data gaps and how they were filled as well as how the event that caused the data gap was resolved; and

        - copies of documents containing information on the amount of fuel oil consumption, distance travelled and hours underway for the ship's voyages during the reporting period (e.g. the ship's official logbook, oil record book, BDNs, arrival/noon/departure reports, etc.)

The Administration or GR  acting on their behalf may request a ship to submit documentation needed to perform a comprehensive review of a ship's annual fuel oil consumption, distance travelled, and hours underway.

The Data Collection Plan should include data quality control measures which should be incorporated into the existing shipboard safety management system (ISM/SMC). Additional measures to be considered could include:

- the procedure for identification of data gaps and correction thereof; and

- the procedure to address data gaps if monitoring data is missing, for example, flow meter malfunctions.

    - Upon the verification of the submitted data, GR  or flag state will issue to the ship a Statement of Compliance (SoC), according to Regulation 4.6.3 and pursuant to regulation 6.6

of Annex VI, shall be valid for the calendar year in which it is issued and for the first five months of the following calendar year.

    - Any discrepancy be identified by the Administration or GR  in the reported data, it should be communicated to the Company on a timely basis for correction.

    - The Statement of Compliance pursuant to regulation 6.7 of Annex VI shall be valid for the calendar year in which it is issued, for the following calendar year, and for the first five months of the subsequent calendar year. All Statements of Compliance shall be kept on board for at least the period of their validity

# REFERENCES

  - IMO Resolution MEPC.292(71), 2017 Guidelines for Administration verification of ship fuel oil consumption data

  - IMO Resolution MEPC.293(71), 2017 Guidelines for the Development and Management of the IMO ship fuel oil consumption database

  - IMO Resolution MEPC.278(70), Amendments to MARPOL Annex VI (Data Collection System for Fuel Oil Consumption), adopted 28 October 2016

  - IMO Resolution MEPC.282(70), 2016 Guidelines for the Development of a Ship Energy Efficiency Management Plan (SEEMP), adopted 28 October 2016

  - IMO Circular MEPC.1/Circ.795/Rev.2, Unified Interpretations to MARPOL Annex VI, issued 01 December 2014

# DEFINITIONS

The following definitions for terminology will apply in this Procedure:

- "Calendar year" means the period from 1 January until 31 December inclusive.

- "Company" means the owner of the ship or any other organization or person such as the manager, or the bareboat charterer, who has assumed the responsibility for operation of the ship from the owner of the ship and who on assuming such responsibility has agreed to take over all the duties and responsibilities imposed by the International Management Code for the Safe Operation of Ships and for Pollution Prevention, as amended.

- "Distance travelled" means distance travelled over ground while ship is underway under its own propulsion.

- "Hours underway" means duration time while ship is underway under its own propulsion.