# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

# SURVEY REPORT ON SOLAS GMDSS

RADIO TECHNICIAN’S SURVEY1

.

(ORIGINAL TO BE RETAINED ON BOARD UNTIL NEXT SURVEY)

| Name of Ship: | Port of Registry: | GT: | Call Letters: | Year Keel Laid: |
| --- | --- | --- | --- | --- |
| Patent Number and Expiration: | IMO No.: | Telex ID No: |  |  |
| Sea area in which vessel is certified to operate2: A1( ) A2( ) A3( ) A4( ) |  | MMSI No.: |  | INMARSAT ID No.: |

| 1. The following test instruments used: | Y | N | NA |
| --- | --- | --- | --- |
| a. Frequency counter |  |  |  |
| b. Watt meter with plug in elements covering MF, HF and VHF |  |  |  |
| c. Ampere / Volt / Ohm meter |  |  |  |
| d. Insulation resistance tester. |  |  |  |
| e. Acid tester (specific gravity) |  |  |  |
| f: instrument for decoding the ID-signal of satellite EPIRB’s |  |  |  |
| g. Spectrum analyzer.* |  |  |  |
| h. Oscilloscope * |  |  |  |
| i. Deviation meter. * |  |  |  |
| j. Demute tester for testing the radiotelephone distress frequency watch receiver (2182 kHz) * |  |  |  |

# The following items were checked and tested as necessary and found satisfactory:

| 2. Sources of energy: | Y | N | NA |
| --- | --- | --- | --- |
| a. Checked main source of energy available in accordance with requirements. |  |  |  |
| b. Emergency source of energy (specify below) b 1. Capacity (Hrs & Kw) b 2. Location |  |  |  |
| c. Reserve source of energy (specify below) c 1. Capacity c 2. Location |  |  |  |

| 3. Radio Installations: | Y | N | NA |
| --- | --- | --- | --- |
| a. The radio controls for operating the radio installation are adequately illuminated. |  |  |  |
| b. The capacity of battery(s) has been checked at intervals not exceeding 12 months |  |  |  |
| c. Electrical lighting is permanently arranged and connected to a source of power independent of the main / emergency3 source of power. |  |  |  |
| d. Radio installation clearly marked with call sign, ship station identify and other applicable codes. |  |  |  |
| e. Radio equipment is located at4: |  |  |  |
| f. Remote control from conning position provided. |  |  |  |

1 The following part of the survey should always be performed by a fully qualified Radio Technician who has adequate knowledge of the Radio Regulations, the Convention, as amended, and the IMO performance standards for radio equipment.

2 Verify and assign the navigation area according to the Radio Equipment installed on board.

*This test equipment may also be used but is not mandatory.

3 Delete as appropriate.

4 Please, indicate where is located.

| 4. Radio Equipment Requirement: | Y | N | NA |
| --- | --- | --- | --- |
| a. Equipment installed fulfills the functional requirements for the vessel’s sea areas of operation. |  |  |  |

| 5. Method of availability of functional requirements:5 | Y | N | NA |
| --- | --- | --- | --- |
| a. Duplication of equipment. |  |  |  |
| b. Shore-based maintenance (Contract should be verified on board). |  |  |  |
| c. At-sea maintenance. |  |  |  |

| 6. Antennas: | Y | N | NA |
| --- | --- | --- | --- |
| a. Was a visual inspection of all antennas including, INMARSAT, GPS and AIS VHF antennas, and feeders for satisfactory sitting *including consideration of any possible interference and defects made?. |  |  |  |
| b. Checked that arrangements are provided enabling MF/HF transmitting antennas to be grounded. |  |  |  |
| c. Checked that the MF/HF transmitting antennas are protected against being touched accidentally. |  |  |  |
| d. For NIS ships, transmitting results to be tested with MEGGER MEGOHM (>50 MOHM dry, > 5 humid). |  |  |  |
| e. For NIS ships, antenna coupling for MF/HF located outside of deckhouse. |  |  |  |

| 7. Reserve source of energy: | Y | N | NA |
| --- | --- | --- | --- |
| a. Checking there is sufficient capacity to operate the basic or duplicated equipment For 1 hour or 6 hours as appropriate (Regulation IV/3).6 |  |  |  |
| b. If reserve source of energy is a battery, specify type: |  |  |  |
| b. 1. Checked its sitting and installation. Specify location: |  |  |  |
| b. 2. Checked for defects, including all cables |  |  |  |
| b. 3. Checked its conditions by specific gravity measurement or voltage Measurement. Specify Voltage / Specify Gravity: |  |  |  |
| b. 4. With battery off change, and the maximum required radio installation load connected to the reserve source of energy, checked the battery voltage and discharge current. Specify Maximum discharge current: |  |  |  |
| b. 5. Checked that the charger(s) are capable of recharging the reserve battery within 10 hours. |  |  |  |
| b. 6. Checked that battery charger is of an automatic type. |  |  |  |

| 8. VHF Transreceivers: | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Checked for operation on channels 6, 13, and 16. |  |  |  |
| --- | --- | --- | --- |
| b. Checked that equipment is within frequency tolerance. |  |  |  |
| c. Checked RF power output and VSWR on 6, 13 and 16. |  |  |  |
| d. Checked correct operation of all controls including priority of control. |  |  |  |
| e. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy. |  |  |  |
| f. Checked operation of the VHF control unit(s) of portable VHF equipment provided for navigational safety from bridge wings. |  |  |  |
| g. Checked for correct operation by on–air contact with a coast station or other ship. |  |  |  |
| h. Checked compliance with IMO performance standards. |  |  |  |

5 Ships engaged on voyages in sea area A3 and A4 must use a combination of two methods (check all that apply).

6 Specify 1 or 6 hours.

| 9. VHF DSC Controllers and Channel 70 DSC watch receiver: | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Performed and off-air check confirming the correct Maritime Mobile Service Identify programmed in the equipment. |  |  |  |
| --- | --- | --- | --- |
| b. Checked for correct transmission by means of a routine or test call to a coast station other ship, on board duplicate equipment, or special test equipment. |  |  |  |
| c. Checked for correct reception by means of a routine or test call to a coast station other ship, on board duplicate equipment, or special test equipment. |  |  |  |
| d. Checked the audibility of the VHF/DSC alarm. |  |  |  |
| e. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy. |  |  |  |
| f. Checked that the ship’s position in the distress alert is automatically provided with this information from an internal or external navigation receiver (e.g. GPS). |  |  |  |
| g. Checked for compliance with IMO performance standards. |  |  |  |
| h. Checked DSC alerting available from conning position. |  |  |  |
| i. Checked that DSC distress procedure and DSC number are clearly displayed near the unit. |  |  |  |

| 10. MF / HF Radiotelephone equipment: | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy. |  |  |  |
| --- | --- | --- | --- |
| b. Checked antenna tuning in all appropriate bands. |  |  |  |
| c. Checked that equipment is within frequency tolerance on all appropriate bands (10 kHz). |  |  |  |
| d. Checked for correct operation by contact with a coast station and / or measure RF power output and VSWR. |  |  |  |
| e. Checked receiver performance by monitoring known stations on all appropriate bands. |  |  |  |
| f. Checked that the control unit on the bridge has first priority for the purpose of Initialing distress alerts, if control units are provided outside the navigational bridge |  |  |  |
| g. Checked for compliance with IMO performance standards |  |  |  |

| 11. MF / HF Radio telex equipment: | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy. |  |  |  |
| --- | --- | --- | --- |
| b. Confirmed that the correct selective calling number is programmed in the equipment. |  |  |  |
| c. Checked correct operation by inspection of recent hard copy or by a test with a coast radio station. |  |  |  |
| d. Checked for compliance with IMO performance standards |  |  |  |

| 12. MF / HF DSC controller(s): | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy. |  |  |  |
| --- | --- | --- | --- |
| b. Confirmed that the correct Maritime Mobile Service Identify is programmed in the equipment. |  |  |  |
| c. Checked the off air self test program. |  |  |  |
| d. Checked operation by means of a test call on MF and / or HF to a coast radio station if the rules of the berth permit the use of MF/HF transmissions. |  |  |  |
| e. Checked the audibility of the MF/HF DSC alarm. |  |  |  |
| f. Checked that the ship’s position in the distress alert is automatically provided with this information from an internal or external navigation receiver (e.g. GPS). |  |  |  |
| g. Checked for compliance with IMO performance standards. |  |  |  |
| h. Checked DSC alerting available from conning position. |  |  |  |

| 13. MF / HF DSC watch receivers: | Y | N | NA |
| --- | --- | --- | --- |

|  | Basic | Duplication |
| --- | --- | --- |
| Make / Model: |  |  |

| a. Confirm that only DSC channels indicated in the Regulation IV/9, 10, 11, and 12 are being monitored. |  |  |  |
| --- | --- | --- | --- |
| b. Checked that a continuous watch is being maintained while keying MF/HF radio transmitters. |  |  |  |
| c. Checked for correct operation by means of a test call from a coast station or other ship. |  |  |  |

| 14. INMARSAT Ship Earth Station(s): | Y | N | NA |
| --- | --- | --- | --- |

|  | No.1 | No.2 | No.3 |
| --- | --- | --- | --- |
| Make / Model: |  |  |  |

Specify Type:	B	C	F77	Specify:	Basic	Duplication

| a. Checked that the equipment operates from the main, emergency (if provided) and reserve sources of energy, and that |  |  |  |
| --- | --- | --- | --- |
| b. Where an uninterrupted supply of information from the ship’s navigational or other equipment is required, ensuring such information remains available in the event of failure of the ship’s main or emergency source of electrical power. |  |  |  |
| c. Checked the distress function by means of an approved test procedure, where possible. |  |  |  |
| d. Checked for correct operation by inspection of recent hard copy of test call by telex or telephone. |  |  |  |
| e. Checked distress function only if permitted to carry out test by the coast earth station. |  |  |  |
| f. Checked for compliance with IMO performance standards as follow: |  |  |  |
| f.1. Resolution A.1001(25) |  |  |  |
| f.2. Resolution MSC.434(98) |  |  |  |
| f.3. Resolution MSC.130(75) |  |  |  |
| f.4. Resolution A.808(19) |  |  |  |
| f.5. Resolution A.698(17) |  |  |  |

| 15. NAVTEX equipment: | Y | N | NA |
| --- | --- | --- | --- |

|  | No.1 |
| --- | --- |
| Make / Model: |  |

| a. Checked for correct operation by monitoring incoming messages or inspecting recent hard copy. |  |  |  |
| --- | --- | --- | --- |
| b. Performed test run of the self-test program, if provided. |  |  |  |
| c. Checked for compliance with IMO performance standards. |  |  |  |

| 16. Enhanced Group Call: | Y | N | NA |
| --- | --- | --- | --- |

Make / Model:

| a. Checked for correct operation by monitoring incoming messages or inspecting recent hard copy. |  |  |  |
| --- | --- | --- | --- |
| b. Performed test run of the self-test program, if provided. |  |  |  |
| c. Checked for compliance with IMO performance standards. |  |  |  |

| 17. Float free satellite EPIRB: | Y | N | NA |
| --- | --- | --- | --- |

Make / Model:

| a. Checked position and mounting for float free operation. Verified that EPIRB is installed in an easily accessible position and is ready to be manually released and capable of being carried by one person into a survival craft. Location: |  |  |  |
| --- | --- | --- | --- |
| b. Verified that the lanyard is firmly attached in a good condition, neatly stowed, and not tied to the vessel or the mounting bracket. |  |  |  |
| c. Carried out visual inspections for defects . |  |  |  |
| d. Carried out the self-test routine. |  |  |  |
| e. Checked that the EPIRB ID and other information (included call sign of the ship) is clearly marked on the outside of the equipment. |  |  |  |
| f. Decoding the EPIRB identify number an other information confirming it is correct and the same as that marked on the EPIRB Identity number: |  |  |  |
| g. Checked registration through documentation or through the point of contact associated with a country code. |  |  |  |
| h. Checked battery expiry date: |  |  |  |
| Checked hydrostatic release unit (HRU) and its expiry date: |  |  |  |
| j. Checked the emission in the 406 MHz band using the self–test mode or an appropriate device to avoid transmission of a distress call satellites. |  |  |  |
| k. If possible, checked the emission on the 121.5MHz frequency using the self–test mode or an appropriate device to avoid activating the satellite system. |  |  |  |
| l. Checked that the EPIRB has been maintained by an approved shore–based maintenance provider at interval required by the administration (but not exceed 5 years)7. Date: , SBM Provider: |  |  |  |
| m. Checked that no transmission has been started after the test and remounting of the EPIRB in its bracket. |  |  |  |
| n. The presence o bacon operating instructions was verified. |  |  |  |
| o. Checked for compliance of IMO performance standards. |  |  |  |

18.   Secondary means of alerting:

Type of secondary means of alerting:

| 19. Two way VHF Radiotelephone apparatus for survival craft : | Y | N | NA |
| --- | --- | --- | --- |

7 The results of shore–based maintenance should be provided in a form of shore–based maintenance report a copy which should be on board the vessel, and a label affixed to the exterior of the bacon detailing the name of the SBM provider and the date when the next shore–based maintenance is due. The SMB provider may affix a tamper proof seal or similar device on completion of the SBM. The maintenance interval provided it does not exceed 5 years, may be aligned with the replaced date of the battery.

|  | Make / Model | Battery expiration date |
| --- | --- | --- |
| No.1 |  |  |
| No.2 |  |  |
| No.3 |  |  |

| a. Checked for correct operation on channel 16 and one other by testing with another fixed or portable VHF installation. |  |  |  |
| --- | --- | --- | --- |
| b. Checked the battery charging arrangements where rechargeable batteries are used. |  |  |  |
| c. Checked that available channels are in compliance with requirements of flag administration. |  |  |  |
| d. Checked the battery expiry dates if primary cells are used to verify if valid. |  |  |  |
| e. Checked any fixed installation provided in a survival craft, where appropriate. |  |  |  |
| f. Checked they are clearly marked with ship’s call sign (permanently marked). |  |  |  |
| g. Checked for compliance of IMO performance standards. |  |  |  |

| 20. Two way portable Radiotelephone apparatus for fire fighter’s communication: | Y | N | NA |
| --- | --- | --- | --- |

|  | Make / Model | Battery expiration date |
| --- | --- | --- |
| No.1 |  |  |
| No.2 |  |  |
| No.3 |  |  |

| a. Correct operation on channel |  |  |  |
| --- | --- | --- | --- |
| b. Checked the battery charging arrangements where rechargeable batteries are used. |  |  |  |
| d. Checked the battery expiry dates if primary cells are used to verify if valid. |  |  |  |
| e. Checked any fixed installation provided, where appropriate. |  |  |  |
| f. Checked they are clearly marked |  |  |  |
| g. Checked for compliance of IMO performance standards. |  |  |  |

| 21. Radar transponders : | Y | N | NA |
| --- | --- | --- | --- |

|  | Make / Model | Frequency Band |
| --- | --- | --- |
| No.1 |  |  |
| No.2 |  |  |

| a. Checked for satisfactory functional test using on board 9 GHZ radar, if possible. |  |  |  |
| --- | --- | --- | --- |
| b. Checked for satisfactory stowage. |  |  |  |
| c. Checked for operating instructions. |  |  |  |
| d. Checked for sufficient battery capacity for stand-by condition and to provide transmissions. |  |  |  |
| e. Checked for clear markings with ship’s call sign. |  |  |  |
| f. Battery expiration date 1) 2). |  |  |  |
| g. Checked for compliance with IMO performance standards |  |  |  |

| 22. Equipment and Spares: | Y | N | NA |
| --- | --- | --- | --- |
| a. Checked test equipment and spares carried to ensure carriage is adequate in accordance with the sea areas in which the ship trades and the declared options for maintaining availability of the functional requirements. |  |  |  |

| 23. Radars : | Y | N | NA |
| --- | --- | --- | --- |

|  | Make / Model |
| --- | --- |
| No.1 |  |
| No.2 |  |

| a. Checked for satisfactory functioning of equipment. |  |  |  |
| --- | --- | --- | --- |
| b. Checked capability to be operated individually and simultaneously, where two radars are required to be carried (For radars installed on or after 1984-09-01). |  |  |  |
| b. Checked radar(s) connected to emergency generator (For radars installed on or after 1984-09-01). |  |  |  |
| d. Checked capability to operate on 9GHz frequency8 |  |  |  |
| e. Compliance with IMO performance standards. |  |  |  |

| 24. ARPA : | Y | N | NA |
| --- | --- | --- | --- |

|  | Make / Model |
| --- | --- |
| No.1 |  |
| No.2 |  |

| a. Checked for satisfactory functioning of equipment. |  |  |  |
| --- | --- | --- | --- |
| b. Checked radar facilities operational (if ARPA integral part of radar). |  |  |  |
| - Checked acquisition, if test means is provided. - Manual. - Automatic. |  |  |  |
| d. Checked audible / visual operational warnings |  |  |  |
| e. According to GMDSS all equipment needs to be type approved. |  |  |  |
| f. Checked for compliance of IMO performance standards. |  |  |  |

| 25. Equipment Approval : | Y | N | NA |
| --- | --- | --- | --- |

| a. According to GMDSS all equipment needs to be type approved. |  |  |  |
| --- | --- | --- | --- |
| b. Checked for compliance of IMO performance standards. |  |  |  |

| 26. Receiver for a Global Navigation Satellite system or a Terrestrial Radio navigation System (GPS)9 : | Y | N | NA |
| --- | --- | --- | --- |

|  | Make / Model |
| --- | --- |
| No.1 |  |
| No.2 |  |

| a. Information on the ship’s position is continuously and automatically provided to all relevant GMDSS equipment. |  |  |  |
| --- | --- | --- | --- |
| b. The navigation receiver is supplied from a source of energy ensuring continuous supply of the ship’s position information in the event of failure of the ship’s main or emergency source of energy. |  |  |  |
| c. Checked for compliance of IMO performance standards. |  |  |  |

The following requirement(s) is/are considered part of the Safety Equipment Survey, however it must be inspected by an GR approved Radio Firm. If this equipment is not inspected at time of the Radio Survey, then and GR approved Radio Firm is to be on board at the time of the Safety Equipment survey when an inspection of the below items is carried out:

| 27. Automatic Identification System (AIS): | Y | N | N/A |
| --- | --- | --- | --- |

8 As per regulation V/12 (g) and (h) of the GMDSS Amendments, for ships that are required to be fitted with radar installation(s), at least one radar installation shall be capable of operating in the 9 GHz frequency band from 1 February 1995.

9 A RDF is not longer required, as per 2000 SOLAS amendments to Chapter V.

| a. Checked for compliance of IMO performance standards, and for new installations, verify antenna layout, initial configuration report, interconnection diagrams, provision of the pilot plug and power supply arrangements. |  |  |  |
| --- | --- | --- | --- |
| b. Verified that the AIS power supply unit is type-approved or tested for electromagnetic compatibility according to IMO Res. A.813(19), if the AIS does not have an integrated power supply unit. |  |  |  |
| c. Verified that the interface installed between the AIS and other Radio-navigational equipment is type- approved. |  |  |  |
| d. Verified that the AIS is connected to the emergency source of power. |  |  |  |
| e. Checked that the AIS is synchronized with UTC, and if provided, position information is correct and valid. |  |  |  |
| f. Verified that correct ship information has been entered into the AIS. |  |  |  |
| g. Performed test including radio frequency measurements and on-air test (to Vessel Traffic Service station or suitable test equipment). |  |  |  |

28.   Radio Technician Remarks:

Radio Technician’s Signature

Radio Technician’s Printed Name

Radio Firm Name

Port of survey	Date (mm/dd/yy)