# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

SURVEY REPORT FOR THE CARRIAGE OF SOLID BULK CARGOES (IMSBC) CODE (APPLICABLE TO SHIPS CARRYING SOLID BULK CARGOES, AS LISTED IN THE INTERNATIONAL MARITIME SOLID BULK CARGOES (IMSBC)

CODE ISSUED UNDER THE PROVISIONS OF THE INTERNATIONAL MARITIME SOLID BULK CARGOES (IMSBC) CODE

ADOPTED BY THE RESOLUTION MSC.268 (85)

Initial Survey

Periodical Survey

(Annual)	Renewal Survey

Additional

Survey (Occasional)

| Name of Ship | Port of Registry |  | Gross Tonnage | Year Keel Laid |
| --- | --- | --- | --- | --- |
| DISTINCTIVE NUMBER OR LETTERS |  | Patente Number And Date: |  |  |

Owner / Operators:    			 Date of Survey / commenced:    		 completed:	 		 Name/Number  of Surveyor:

Place of Survey:

This vessel has been duly surveyed in accordance with the provisions of the International Maritime Solid Bulk Cargoes (IMSBC) Code.

X

Note:	Mark in box

Fit or Yes	Unfit or No

Not Applicable

N/A

General-All Ship Types

  - Have any changes been made or new equipment installed which would affect the validity of

IMSBC-Code certificate? ......................................................................................................

  - Have any alterations been made or any new equipment installed? (marked in Item E)......

  - National Administration Requirements

    - Is there any flag administration particular requirements? ....................................

    - Have all flag administration requirements been complied with?..........................

  - Manning

    - Does the ship’s complement comply with the Minimum Safe Manning

document? ……………………………………………………………………………………………………….

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

    - Are the master, officers and ratings duly certified as required by the STCW Convention?............................................................................................................

# Documentation

  - Is the approved “Loading Manual” on board? (Refer to SOLAS, Chapter VI, Reg.7.2) ........

Date of approval:	Approved by (Name of RO):

  - Is the approved “Trim and Stability Booklet” on board? (Refer to SOLAS, Chapter II-1, Reg.22)........................................................................................................................................ Date of approval:	Approved by (Name of RO):

  - Is a copy of the latest edition of the IMSBC-Code on board, including all amendments?  ......

# List of Group “B” cargoes required to be carried

| Mark Applicable Cargoes | Cargo | UN. No. | IMO CLASS | See Survey Items (SI) in first column of Sections D- 1 to D-11 |
| --- | --- | --- | --- | --- |
|  | Aluminium Ferrosilicon Powder | 1395 | 4.3 | 1, 2, 5, 7, 12, 18, 29, 30, 31, 32, 43, 48, 61 |
|  | Aluminium Nitrate | 1438 | 5.1 | 47, 61 |
|  | Aluminium Silicon Powder Uncoated | 1398 | 4.3 | 1, 2, 5, 7, 12, 15, 17, 29, 30, 31, 32, 43, 48, 61 |
|  | Aluminium Smelting by- products or Aluminium Remelting by- Products | 3170 | 4.3 | 1, 2, 5, 7, 12, 15, 17, 18, 27, 32, 33, 43, 48, 52, 61 |
|  | Ammonium Nitrate with no more than 0,2% total combustible substances including any organic substance calculated as carbon, to the exclusion of any other added substance. | 1942 | 5.1 | 11, 12, 44, 46, 51, 57, 58, 61 |
|  | Ammonium Nitrate based Fertilizer | 2067 | 5.1 | 11, 12, 21, 23, 24, 41, 44, 46, 47, 51, 57, 58, 61 |
|  | Ammonium Nitrate based Fertilizer | 2071 | 9 | 11, 12, 21, 23, 24, 41, 44, 46, 47, 51, 57, 58, 61 |
|  | Barium Nitrate | 1446 | 5.1 | 16, 47, 51, 61 |
|  | Brown Coal Briquettes |  | MHB | 1, 3, 6, 7, 9, 10, 12, 13, 20, 23, 35, 36, 39, 45, 46, 47, 53, 54, 60 |

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

|  | Calcium Nitrate | 1454 | 5.1 | 21, 47, 51, 61 |
| --- | --- | --- | --- | --- |
|  | Castor beans or castor meal or castor pomace or castor flake | 2969 | 9 | 16, 47, 51, 59, 61 |
|  | Charcoal | - | MHB | 39, 40, 47, 51, 59 |
|  | Coal | - | MHB | 1, 3, 6, 7, 9, 10, 12, 13, 14, 20, 22, 23, 35, 36, 38, 45, 46, 53, 54, 60 |
|  | Copra (dry) | 1363 | 4.2 | 12, 16, 25, 39, 40, 59, 60, 61 |
| Mark Applicable Cargoes | Cargo | UN. No. | IMO CLASS | See Survey Items (SI) in first column of Sections D- 1 to D-11 |
|  | Direct reduced Iron (A) Briquettes, hot-moulded | - | MHB | 1, 2, 5, 7, 12, 14, 17, 20, 25, 32, 39, 40, 45, 47, 51 |
|  | Direct reduced Iron (B) Lumps, pellets, cold-moulded briquettes | - | MHB | 1, 2, 5, 7, 12, 21, 25, 28, 39, 45, 46, 47, 51, 56, 59 |
|  | Direct reduced Iron (C) (by product fines) | - | MHB | 1, 2, 5, 7, 12, 21, 25, 28, 39, 45, 46, 47, 51, 56, 59 |
|  | Ferrophosphorus (including briquettes) | - | MHB | 1, 2, 4, 18, 29 |
|  | Ferrosilicon, with 30% or more but less than 90% silicon (including briquettes) | 1408 | 4.3 | 1, 2, 4, 7, 12, 15, 17, 29, 30, 32, 39, 41, 43, 48, 50, 52, 61 |
|  | Ferrosilicon, 25% to 30% silicon, or 90% or more silicon (including briquettes) | - | MHB | 1, 2, 4, 7, 12, 15, 17, 29, 30, 32, 39, 40, 43, 48, 52 |
|  | Ferrous metal borings, shavings, turnings or cuttings, in a form liable to self-heating | 2793 | 4.2 | 21, 26, 39, 40, 59, 61 |
|  | Fishmeal (Fishscrap), stabilized Anti-oxidant treated | 2216 | 9 | 16, 25, 39, 40, 59, 61 |
|  | Fluorspar | - | MHB | 47, 51 |
|  | Iron Oxide, spent Iron Sponge, spent obtained from coal gas purification | 1376 | 4.2 | 1, 3, 5, 10, 16, 34, 39, 40, 47, 59, 61 |
|  | Lead Nitrate | 1469 | 5.1 | 16, 47, 51, 61 |
|  | Lime (unslaked) | - | MHB | 47, 51 |
|  | Linted cotton seed with no more than 9%moisture and not more than 20.5% oil | - | MHB | 39, 40, 46, 59 |

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

|  | Magnesia (unslaked) | - | MHB | 47, 51 |
| --- | --- | --- | --- | --- |
|  | Magnesium Nitrate | 1474 | 5.1 | 47, 61 |
|  | Metal Sulphide concentrates | - | MHB | 21, 39, 40, 59 |
|  | Peat Moss | - | MHB | 16, 39, 40, 47, 51 |
|  | Petroleum Coke(calcined or uncalcined) | - | MHB | 47 |
|  | Pitch Prill | - | MHB | 16, 47, 49, 60 |
|  | Potassium Nitrate | 1486 | 5.1 | 21, 47, 51, 61 |
|  | Pyrites, calcined (Calcined Pyrites) | - | MHB | 47, 51, 53 |
|  | Radioactive material, low Specific Activity (LSA-I), non-fissile or fissile – excepted | 2912 | 7 | 21, 47, 51 |
| Mark Applicable Cargoes | Cargo | UN. No. | IMO CLASS | See Survey Items (SI) in first column of Sections D- 1 to D-11 |
|  | Radioactive material, Surface Contaminated Objects (SCO-I), non-fissile or fissile - excepted | 2913 | 7 | 21, 47, 51 |
|  | Sawdust | - | MHB | 16, 39, 40, 51, 59 |
|  | Seedcake, containing vegetable oil (a) mechanically expelled seeds, containing more than 10% of oil or more than 20% of oil and moisture combined. | 1386 | 4.2 | 21, 25, 39, 40, 59, 61 |
|  | Seedcake, containing vegetable oil (b) solvent extractions and expelled seeds, containing not more than 10% of oil and, when the amount of moisture is higher than 10%, not more than 20% of oil and moisture combined. | 1386 | 4.2 | 1, 3, 8, 10, 12, 16, 20, 25, 39, 40, 46, 55, 58, 59, 61 |
|  | Seedcake with not more than 1.5% oil and not more than 11% moisture. | 2217 | 4.2 | 1, 3, 8, 10, 12, 16, 20, 25, 39, 45, 55, 58, 59, 61 |

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

|  | Silicomanganese (low carbon) (with known hazard profile or known to evolve gases) (with silicon content of 25% or more) | - | MHB | 12, 17, 19, 29, 30, 32, 39, 40 |
| --- | --- | --- | --- | --- |
|  | Sodium Nitrate | 1498 | 5.1 | 21, 47, 51, 61 |
|  | Sodium Nitrate and Potassium Nitrate mixture | 1499 | 5.1 | 21, 47, 51, 61 |
|  | Sulphur (crushed, lump and coarse grained) | 1350 | 4.1 | 1, 6, 7, 9, 12, 16, 20, 61 |
|  | Tankage | - | MHB | 25, 47, 51, 59 |
|  | Vanadium Ore | - | MHB | 47, 51 |
|  | Woodchips (with moisture content of 15% or more) | - | MHB | 37, 39, 40 |
|  | Woodchips (with moisture content of less than 15%) | - | MHB | 37, 39, 40, 59 |
|  | Wood pellets | - | MHB | 11, 38, 39, 40, 42, 46, 59 |
|  | Wood pulp pellets (with moisture content of 15% or more) | - | MHB | 39, 40 |
|  | Wood pulp pellets (with moisture content of less than 15%) | - | MHB | 39, 40, 59 |
|  | Zinc Ashes | 1435 | 4.3 | 1, 2, 5, 7, 12, 18, 32, 47, 61 |

- Survey Items in Sections D-1 to D-11

  - SOURCES OF IGNITION Survey Items

1	Are all electrical equipment and cabling in the cargo spaces (including that

cargo space ventilation fans) and adjacent hazardous spaces or zones either: 1(a)	capable of being positively isolated, throughout the period during which the

board, from position(s) outside any hazardous space or zone, by removal of links

operation of lockable isolating switches or circuit breakers, or other effective means; or ..

1(b)	of a suitable certified safe type with all cable (with the permitted exception of that for intrinsically safe circuits) having a metallic braid or run in metallic

conduit; or ……………………………………………………………………………………………………….

1(c)	simple apparatus (e.g. thermocouple or similar temperature sensor, level switch

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

with no electronic component, etc.) incorporated in an i-s circuit; …………………….

1(d)	(within the following locations only: ventilated spaces adjacent to a cargo hold, having access to the hold only via a self closing gastight door; on open deck,

zones, or semi- enclosed spaces having an opening, within 3m of a cargo hold

ventilation outlet) of type of protection ‘n’ or of a type not generating arcs or sparks in service and whose surface does not reach unacceptably high temperatures, or of a type otherwise offering protection against risk of ignition of gas? …………………………………………………………………………………………………………………..

- Is the electrical equipment mentioned in above SI 1 of Group IIC? (Refer to Note below) …………………...………………………………………………………………….......................

- Is the electrical equipment mentioned in above SI 1 of Group IIA, IIB or IIC? (Refer

Note 1 below) …………………………………………………………………………………………………...

- Is the electrical equipment mentioned in above SI 1 of temperature classification T3, T4, T5 or T6, or, in the case of 1(d), known to have a surface temperature not exceeding 450ºC? (Refer to Note 2 below) ……………………………………………………….

- Is the electrical equipment mentioned in above SI 1 of temperature classification T4, T5 or T6, or, in the case of 1(d), known to have a surface temperature not exceeding 300ºC? (Refer to Note 2 below) ……………………………………………………….

- Is the electrical equipment mentioned in above SI 1of temperature classification T4, T5 or T6, or, in the case of 1(d), known to have a surface temperature not

exceeding 135ºC? (Refer to Note 2 below) ………………………………………………………

- Is portable electrical equipment of a suitable certified safe type for exposure to given in atmospheres containing combustible gas, of Group and temperature

classification as the previous notes(as applicable to the cargoes permitted to

be carried)? ………………………………………………………………………………………………………

- Are all electrical equipment and cabling in the cargo holds (including those associated with cargo space ventilation fans) and in any space having a

direct opening into a hold either:

8(a)	capable of being positively isolated, throughout the period during which the cargo is on board, from position(s) outside any hold or space with an

opening into a hold, by removal of links or operation of lockable isolating

switches or circuit breakers or other effective means; or ……………………………..

# Survey Items

8(b)	of a type having ingress protection rating at least IP55 and surface temperatures not of 3 exceeding 200ºC(or assessed by a competent authority as suitable

for use in the presence combustible dusts), with all cable (with the permitted exception of that for intrinsically safe circuits) having metallic braid or armour,

or run in metallic conduit? (Refer to Note below) ……………………………………….

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

- Are all electrical equipment and cabling in the cargo holds (including those

with cargo space ventilation fans) and in any space having a direct opening into a either:

9(a)	capable of being positively isolated, throughout the period during which the cargo is on board, from position(s) outside any hold or space with an

opening into a hold, by removal of links or operation of lockable isolating

switches or circuit breakers or other effective means; or ………………………………….

9(b)	of a type having ingress protection rating at least IP5x and surface temperatures not of exceeding 135ºC (or assessed by a competent authority as suitable

for use in the presence combustible dusts), with all cable(with the permitted exception of that for intrinsically safe circuits) having metallic braid or armour, or run in metallic conduit? …………………………………………………………………………………

- Is portable electrical equipment of a type having ingress protection rating at least IP55 and surface temperatures not exceeding 200 deg. C, suitable for exposure

to atmospheres containing combustible dusts(or assessed by a competent

authority as suitable for use in the presence of combustible gas)? ………………….

- Is all electrical equipment in the cargo space that is not intrinsically safe, capable of being disconnected from the power source (by removal of links in the

system, other than fuses) from an external point to the cargo space,

throughout the period during which the cargo is onboard… …………

12(a)	Are all sources of ignition eliminated? ……………………………………………………………….

12(b)	Are there Prohibition Notices for welding, burning, cutting or other operations involving the use of fire, smoking, chipping and use of naked lights,

or the introduction of other sources of ignition including the use of spark

or arc-producing equipment, posted in the vicinity of cargo spaces? ……………………

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

| Gas Group | Temperature Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | T3 | T4 | T5 | T6 |
| IIA | Acetone | Ethanol | Diesel fuel | Acetaldehyde |  |  |
|  | Methane | Cyclohexane | Aircraft fuel |  |  |  |
|  | Ethane | Propanol 2 | Fuel oil |  |  |  |
|  | Benzene | N-butyl alcohol | N-Hexane |  |  |  |
|  | Methanol | N-butane | Heptane |  |  |  |
|  | Toluene |  | Kerosene |  |  |  |
|  | Propane |  |  |  |  |  |
|  | Acetic acid |  |  |  |  |  |
|  | Ammonia |  |  |  |  |  |
| IIB | Coal Gas | Ethylene | Ethylene glycol | Ethyl Methyl Ether |  |  |
|  |  | Ethylene oxid | Hydrogen Sulphide |  |  |  |
|  |  | Propanol 1 | Tetrahydrofuran |  |  |  |
|  |  | Methyl Ethyl |  |  |  |  |
|  |  | Ketone |  |  |  |  |
| IIC | Hydrogen | Acetylene |  |  |  |  |

| Group II Temperature Class |  |  |
| --- | --- | --- |
| T cod e | Maximum surface Temperature | Gas Ignition temperature |
| T1 | 450°C | t > 450 0 C |
| T2 | 300°C | 450°C ≥ t >300°C |
| T3 | 200°C | 300°C ≥ t >200°C |
| T4 | 135°C | 200°C ≥ t >135°C |
| T5 | 100°C | 135°C ≥ t >100°C |
| T6 | 85°C | 100°C ≥ t >850°C |

Note 1: Electrical equipment for places with an explosive gas atmosphere: Group IIA, Group IIB, Group IIC. See IEC Standard 60079

Note 2 – Temperature classification. Electrical apparatus class is based on the auto-ignition temperature of the gas, details of which are given on table below. See IEC Standard 60079.

Note 3: Certified safe type equipment having temperature classification T3, T4, T5 or T6 may be accepted, provided that it has ingress protection rating IP55, or better. See IEC 60529. IEC means International Electrotechnical Commission

# VENTILATION Survey Items

13	Are workspaces (stores, workshops etc.) adjacent to the cargo spaces

adequately ventilated? …………………………………………………………………………………….

14(a)	Can adequate surface ventilation be maintained? (Natural ventilation is sufficient)............................................................................................................

14(b)	Are arrangements such that air is not directed into the body of the cargo? .. 15(a)	Are the cargo spaces ventilated by at least two separate fans? (A common

ventilation system with 2 fans connected is acceptable)..................................

15(b)	Is the total ventilation at least six air changes per hour, based on the empty space?................................................................................................................

15(c)	Are the requirements of the Load Line Convention for openings not fitted with means of closure (i.e. height of ventilation inlets and outlets for mechanical

ventilation arranged for continuous operation) compiled with?.....................

15(d)	Are ventilator trunkings in sound condition and so arranged to prelude interconnection of the atmosphere in the cargo space with other

cargo spaces, accommodation or work areas? ...............................................

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

  - Can adequate surface ventilation be maintained? (Natural ventilation is sufficient).........................................................................................................

  - Is ventilation such that any escaping gases cannot reach living quarters on or under the deck?.................................................................................................

18(a)	Are the cargo spaces provided with mechanical ventilation?...........................

18(b)	Are the requirements of the Load Line Convention for openings not fitted with means of closure (i.e. height of ventilation inlets and outlets for mechanical

ventilation arranged for continuous operation) compiled with?.....................

19	Are the cargo spaces provided with mechanical ventilation?.......................... 20	Are spark arresting screens fitted to ventilators?............................................

- Is it possible to avoid cargo ventilation?  (i.e. Can ventilators be completely closed?).............................................................................................................

- If the vessel is a gravity fed, self-unloading bulk carrier, can the cargo and self- unloading spaces operate on both “negative-pressure” ventilation(more exhaust than supply in the tunnels to remove carbon monoxide) and

“positive-pressure” ventilation (more supply than exhaust in the

tunnels to remove methane gas)?..................................................................

# TEMPERATURE MEASUREMENT

23(a)	Are means provided for measuring the temperature of the cargo in the range

100°C? …………………………………………………………………………………………………………..

23(b)	Do such arrangements enable the temperature of the cargo to be measured, and during the voyage, without requiring entry to the cargo space? …………..

24	Are calibrated portable instruments, suitable for use from outside the cargo

spaces, provided for measuring cargo temperature? …………………………………...

25(a)	Are means provided to enable the temperature to be taken throughout the cargo?...............................................................................................................

25(b)	Are the measurements recorded and is the information kept on board? ……..

# Survey Items

26(a)	Are means provided to monitor the surface temperature of the cargo daily?..

26(b)	The temperature readings should if possible be taken in such a way as not to require entry into the cargo space but if entry is required for this purpose,

are there at least two self-contained breathing apparatuses provided additional to those required by Chapter II-2, Reg.17 or Reg. 10 of the applicable1974 SOLAS Convention, as amended? …………………………………………………………………

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# GAS DETECTION

27(a)	Are there at least two suitable calibrated and certified explosimeters provided capable of detecting flammable gases?...........................................................

27(b)	Are the measurements recorded and is the information kept on board?........ 28(a)	Are there suitable calibrated and certified instruments provided to monitor for

the presence of Oxygen and Hydrogen?.............................................................

28(b)	Are the instruments suitable for use in an atmosphere without oxygen and type?...................................................................................................................

28(c)	Are such measurements taken at regular intervals throughout the voyage, recorded, and the information kept on board and made available on

request?..............................................................................................................

29(a)	Are there suitable calibrated and certified safe type detectors provided for quantitative measurements of Phosphine?.......................................................

29(b)	Are the measurements recorded and is the information kept on board?........

30(a)	Are there suitable calibrated and certified safe type detectors provided for quantitative measurements of Arsine?.............................................................

30(b)	Are the measurements recorded and is the information kept on board?.........

31(a)	Are there suitable calibrated and certified safe type detectors provided for quantitative measurements of Silane?...............................................................

31(b)	Are the measurements recorded and is the information kept on board?.........

32(a)	Are there suitable calibrated and certified safe type detectors provided for quantitative measurements of Hydrogen?.........................................................

32(b)	Are the measurements recorded and is the information kept on board?.......... 33(a)	Are there suitable calibrated and certified safe type detectors provided for

quantitative measurements of Ammonia and Acetylene?..................................

33(b)	Are the measurements recorded and is the information kept on board?..........

34(a)	Are there suitable calibrated detectors, suitable for use in an atmosphere without oxygen and certified safe type; provided for quantitative

measurements of Hydrogen Cyanide?................................................................

34(b)	Are the measurements recorded and is the information kept on board?.......... 35(a)	Are there calibrated and certified instruments provided on board suitable for

use from outside the cargo spaces for measuring (i) concentration of Methane in the atmosphere; (ii) concentration of Oxygen in the atmosphere;

(iii) concentration of Carbon Monoxide in the atmosphere?............................

35(b)	Are the instruments fitted with an aspirator, flexible connection and a length of tubing to enable a representative sample to be obtained from within the

square of the hatch?............................................................................................

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# Survey Items

35(c)	Is the tubing of spark-proof metal construction with an integral stainless steel threaded collar? (The collar is necessary to provide an adequate seal at the

sampling point.)………………………………………………………………………………………………

35(d)	When recommended by the manufacturer, is a suitable filter provided to protect the instrument against the ingress of moisture?.................................

35(e)	Are the sample results logged on a form that records cargo hold, date and time for each measurement?....................................................................................

36(a)	Are two sampling points provided per hold (approximately 12mm in diameter),

positioned as near to the top of the hatch coaming as possible, one on the port side, the other on the starboard side?  (Refer to Note 4 below)……………………..

36(b)	Is each sampling point sealed with a screw cap to prevent ingress of water and air?......................................................................................................................

36(c)	Are the arrangements fitted such that the watertight integrity of the ship is not compromised?....................................................................................................

- Are personal Oxygen meters available onboard which can be worn by all crew when entering cargo and adjacent enclosed spaces?.........................................

- Are individual or combined personal Oxygen and Carbon Monoxide meters available onboard which can be worn by all crew when entering cargo and

adjacent enclosed spaces?.................................................................................

- Cargo spaces containing this product may become oxygen-depleted.  Have notices to this effect, warning that precautions should be taken prior

to entry, been prominently displayed in the cargo area?.................................

- Are there calibrated and certified safe type instruments suitable for use from outside the cargo spaces for measuring the concentration of Oxygen

in the atmosphere? (SOLAS, Chapter VI, Regulation 3)…………………………………

# CARGO STOWAGE

- Has it been confirmed that the hold is not adjacent to any tank or double bottom containing fuel oil if such oil is heated to more than 50°C?...............

- Are arrangements such that cargo of this type can be stowed out of direct

contact or away from hold lighting?……………………………………………………………..

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# D.6 CARGO SPACE INTEGRITY

43	Are the bulkheads to the engine-room gastight?.............................................

  - Has a pressure test been carried out to confirm that there is no leakage of manholes or piping systems from fuel tanks situated under, or leading

through, the cargo spaces in which this cargo is to be loaded?......................

  - Are the boundaries of the cargo spaces where this cargo is carried resistant to fire and the passage of liquid?..........................................................................

46	Are the hold seals in a good condition?............................................................

Note 4: The sampling points should be designed as per the “Diagram of gas sampling point” in the IMSBC-Code schedule for Brown Coal Briquettes and Coal cargoes. Refer to “Sampling and measurement procedure” according to MSC.85/26/Add.2, ANNEX 3(page 104)

# D.7. PROTECTIVE EQUIPMENT Survey Items

- Are protective clothing, dust-masks and goggles provided on board?..............

- Are there at least two self-contained breathing apparatuses provided additional to those required by Chapter II-2, Reg. 17 or Reg. 10 of the applicable

1974 SOLAS Convention, as amended?.............................................................

49	Are eye wash and sun screen provided?...........................................................

- Are full chemical protective suits readily available onboard?...........................

# HOLD BILGE ARRANGEMENTS

- Can bilge wells of the cargo spaces be protected from the ingress of the cargo?..

- Have means been provided in the cargo hold bilge arrangements to ensure against inadvertent pumping through the engine-room?.................................

- Can appropriate precautions be taken to avoid penetration of the cargo into bilges, wells or between ceiling boards?...........................................................

54(a)	Are means provided for measuring pH values of hold bilge samples?.............. 54(b)	Do such arrangements enable the pH values to be measured, while loading and

during the voyage, without requiring entry to the cargo space?.......................

# ATMOSPHERE CONTROL

55	Is the ship equipped with facilities for introducing Carbon Dioxide or another inert gas into the cargo spaces, if the voyage exceeds five days?

(Note: It is suggested that the quantity of Carbon Dioxide to be carried for topping-up purposes is 5% of the gross volume of the holds in which the cargo is to be carried. A cock or valve should be fitted to each hold

where Carbon Dioxide may be introduced. The piping system need not be

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

of a permanent nature. )……………………………………………………………………………

56	Can provision be made to introduce inert gas at tank top level so that the whole of the cargo space can be maintained at a low oxygen level (less than 5% hydrogen less than 1% by volume) throughout the voyage?   (This is required

where the substance has not been manufactured or treated with an oxidation and corrosion-inhibiting process which has been proved to provide effective protection against dangerous reaction with sea

water or air under shipping conditions)…………………………………………………………

# FIRE CONTAINMENT/EXTINCTION

57	Are there adequate supplies of water for fire-fighting purposes immediately available from the fire main whenever this cargo is on board?.......................

58(a)	Is the bulkhead between the cargo space and the engine-room insulated to Class ‘A-60’ standard or an equivalent arrangement approved by the Flag

State Administration or by any Class Society/RO?............................................

58(b)	If the bulkhead between the cargo space and the engine-room is not insulated to Class‘A-60’ standard, can this cargo be stowed “away from” the bulkhead?.

- Is a fixed gas fire-extinguishing system provided, in accordance with applicable 1974 SOLAS Chapter II-2, Regulation 53.1.2 or Regulation10.7.2?...................

- Is it confirmed that the cargo is not stowed adjacent to heated surfaces?.......

# DANGEROUS GOODS Survey Items

61	This cargo is classed as dangerous goods under 1974 SOLAS, Chapter VII, Regulation2, and compliance with 1974 SOLAS, Chapter II-2, Regulation

54/19 may be required for its carriage. ………………………………………………………….

Regulation 54/19 is a mandatory requirement for a ship carrying, or intending to carry, dangerous goods as defined in 1974 SOLAS, Chapter VII, Regulation 2,

except class 6.2 and 7, and is: ………………………………………………………………………..

      - (a)  A passenger ship constructed on or after 1-September-1984 (54) or/ 1-July-2002 (19); or………………………………………………………………….

      - (b)  A cargo ship of 500 gross tons or over constructed on or after (54) 1-September-1984/ 1-July-2002 (19); or………………………………..

      - (c)  A cargo ship of less than 500 gross tons constructed on or after 1-February-1992 (54)/ 1-July-2002 (19)………………………………………….

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

If the ship falls within the above parameters, is it provided with a Dangerous Goods, Document of Compliance(or IMDG Certificate), indicating that

it is suitable to carry the appropriate classes of dangerous goods in bulk?.....

# ALTERATIONS OR MODIFICATIONS:

  - Describe in detail all system and equipment or parts which have been altered or modified...

# DEFICIENCIES, REQUIREMENTS, REMARKS, RECOMMENDATIONS.

- FINAL RECOMMENDATION:

It is recommended that:

G.1  A Full Term IMSBC Code Certificate may be issued ...............................................................

  - No Full Term IMSBC Code Certificate to be issued until deficiencies are closed (marked in Item F)…………………………………………………………………………………………………………………………………………

  - I have issued the Interim / Conditional (*) IMSBC Code Certificate  No.:

at:	,	of	of

(Place of issue of Interim IMSBC Code)	(Day)	(Month)	(Year)

Valid until:		of		of  		 (Day)		(Month)		(Year)

| Issued at: |  | Date of Issue: |  |
| --- | --- | --- | --- |
|  | (Place of issue of certificate) |  | (DD-MM-YYYY) |

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

THIS IS TO CERTIFY THAT: This survey report is correct in all respects.

# GR CLASS Surveyor Name   - ID No.