GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)  SURVEY REPORT FOR FISHING VESSEL SAFETY CERTIFICATE

VESSEL 24 METERS IN LENGTH AND ABOVE

THIS FORM MUST BE KEPT ON BOARD AND BE AVAILABLE FOR INSPECTION BY A NOMINATED

SURVEYOR OR RECOGNIZED ORGANIZATION AT ALL TIMES

Initial Survey

Periodical  Survey

(Annual)	Renewal Survey

Additional

Survey (Occasional)

| Name Of Ship | Port Of Registry |  | Gross Tonnage | Year Keel Laid |
| --- | --- | --- | --- | --- |
| DISTINCTIVE NUMBER OR LETTERS |  | Patente Number And Date: |  |  |

Details Of Persons Requesting Survey And To Whom Certificates Are To Be Sent

OWNERS/MASTER

AGENTS/OPERATORS

Date of Survey   /   commenced:    		 completed:	 			        Name/Number  of Surveyor:	 		 Place of Survey:

X

--

N/A

Vessel's status according to the Fishing Vessels Safety Regs. (*)	A new ship	/	An existing ship

Note:	Mark in box

Fit or Yes

Unfit or No

Not Applicable

# GENERAL INFORMATION:

Certificate have been checked and the corrections made have been listed overleaf

Survey report for issue of Fishing Vessel Safety Certificate on board

Any recommendation issued or deficiencies noted upon completion of the survey are also listed overleaf

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# DOCUMENTATION

  - Fire control plans posted/booklets available …………………………………………………………………………………….

  - Muster List and Emergency procedures posted ………………………………………………………………………………..

  - Damage Control Plans posted/ booklets available …………………………………………………………………………….

A.4	Stability Data available ……………………………………………………………………………………………………………………..

  - PRACTICE MUSTERS AND DRILLS

    - Log book entries are being made by the Master ……………………………………………………………………

    - Date when last full crew muster for boat and fire drills took place ………………………………………

    - The boats equipment was examined at that time and found to be complete …………………….

    - The last occasion when the lifeboats were operated, indicate dates below:

| LB No. | Turned out | Lowered to water | Engines operated |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

A.6	First-Aid/Medicines provided and accessible ……………………………………………………………………………………

# SAFETY OF NAVIGATION

  - Shipborne Navigational Equipment according to Record, are in order: …………………………………………….

    - Compasses adjusted (main and steering) ……………………………………………………………………………..

  - Propeller speed and direction or pitch indication operational ………………………………………………………….

  - NAVIGATIONAL LIGHTS (COLREG 1972)

    - Lights type approved or lights delivered with certificates (copy of certificates enclosed)………

    - Lights are tested with power from mains and emergency source ………………………………………….

    - If fitted, light failure indicator/alarm is tested ………………………………………………………………………

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |

Sidelights are screened to prevent light from being seen across the centerline  ……………………

  - SIGNALLING APPARATUSES

    - Daylight signaling lamp is tested on mains and emergency source of power ………………………

    - Equipment, as listed in the record, are onboard and in order ………………………………………………

# LIFE SAVING APPLIANCES

  - ROCKETS AND SIGNALS

    - Rockets and/or cartridges are valid (usual validity 3 years) …………………………………………………..

    - Distress signals are valid (usual validity 3 years) ……………………………………………………………………

  - LIFEBOATS/RESCUE BOATS

|  | No. 1 | No. 2 |
| --- | --- | --- |
| a) Lifeboats now turned out and lowered to embarkation deck |  |  |
| b) Lifeboats now turned out and lowered to water |  |  |
| c) Hull inside and outside, and lifting hooks inspected |  |  |
| d) When lowered to water, the disengagement gear readily released the lifeboat |  |  |
| e) Portable or built-in aircases removed, found or placed in good condition, replaced and secured |  |  |
| f) Air-cylinders in covered boats fully charged (if provided) |  |  |
| g) Lifeboats marked with dimensions, name, port of registry and number of persons |  |  |
| h) Engines in motorboats operated |  |  |

  - LIFEBOATS EQUIPMENT

    - Food rations, tinned water and pyrotechnics are valid    (5 yrs)	(3 yrs)	(3 yrs) ………………

    - Fire extinguisher in motorboat is serviced ………………………………………………………………………………

C.3.3  Motor run ahead and reverse …………………………………………………………………………………………………

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

C.3.4  Fuel tank with pipes are checked …………………………………………………………………………………………….

    - Sufficient fuel for 4 hours operation provided …………………………………………………………………………

    - All equipment are in order and secured ………………………………………………………………………………….

  - DAVITS AND WINCHES

    - All sheaves, blocks, falls and all moving parts are free and well lubricated ………………………………

    - Hidden parts of wire falls have been inspected ……………………………………………………………………….

    - Dates of fall’s reversal  (Usually every 2nd/3rd  year) and renewal (4th/5th year)

C.5

LAUNCHING AND EMBARKATION ARRANGEMENTS

(DD-MM-YYYY)

    - All blocks, chains and shackles checked …………………………………………………………………………………..

    - Boat lashings and release arrangement in order …………………………………………………………………….

    - Lifelines fitted to davit spans checked …………………………………………………………………………………..

C.5.4  Embarkation Iadders checked …………………………………………………………………………………………………

C.5.5  Lighting at boat/raft stations and oversides tested on mains and emergency source of power

C.5.6  Alarm system tested ……………………………………………………………………………………………………………..

C.5.7  Means of preventing water discharge in boats verified ………………………………………………………….

  - LIFERAFTS

Servicing of inflatable liferafts (due every 12 months) (Hydrostatic release to be serviced together with raft)

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

| NO. | MAKERS NAME AND SERIAL NO. | NO. OF PERSONS | DATE OF SERVICE |  |
| --- | --- | --- | --- | --- |
|  |  |  | RAFTS | HYDROSTATIC RELEASE |

  - OTHER LIFESAVING APPLIANCES

    - Lifejackets examined and found in order ……………………………………………………………………………….

    - Lifejackets readily accessible and their position plainly indicated ………………………………………….

    - Lifebuoys in good condition, and marked with name and port of registry …………………………….

    - Lights and lines in order, and smoke signal valid (3 yrs.) ……………………………………………………….

C.7.5	Quick release from bridge in order ……………………………………………………………………………………….

  - PILOT LADDER

    - Ladder, as required, is in good order with necessary spreaders (battens) and with interval not more than 9 steps …………………………………………………………………………………………………………

    - Adequate Iighting and bulwark ladder provided …………………………………………………………………..

# FIRE APPLIANCES

D.1FIRE APPLIANCES IN SPACES CONTAINING MAIN AND AUXILIARY MACHINERY AND BOILERS

    - Main Fire Pumps

D.1.1.1	State pressure in fire main piping with two jets of water when two fire pumps working simultaneously

    - Emergency fire Pump

D.1.2.1	State pressure in fire main piping with two jets of water

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

    - Portable and Non-Portable fire-Extinguishers in Machinery Spaces

      - Date of Iast checking

(DD-MM-YYYY)

      - Spare charges provided for each fire-extinguisher ……………………………………………….

  - CO2 Fire-Extinguishing Installation in Machinery Spaces

    - Date of biennial checking of CO2 system and control of CO2 bottles by

recognized firm

(DD-MM-YYYY)

    - Alarms tested and instructions verified …………………………………………………………………………………

  - Fixed Foam Fire-Extinguishing System in Machinery Spaces Low expansion foam installations:

    - Distribution system and control gear tested with fresh water, compressed air or otherwise …………………………………………………………………………………………………………..

    - Date of biennial control of foam samples by recognized firm

High expansion foam installation:

    - Foam generators have been tested by discharging foam outside the protected

Area ………………………………………………………………………………………………………………………

    - Date of biennial control of foam samples by recognized firm

2

  - Fixed pressure Water Spraying Fire-Extinguishing System in Machinery Spaces

    - System Tested by fresh water or compressed air ……………………………………………………………..

    - Automatic operation of the pump tested …………………………………………………………………………

D.4.3	All controls tested ……………………………………………………………………………………………………………

  - Special Arrangements in Machinery Spaces Following tested: Following tested:

D.5.1	All remote stops ………………………………………………………………………………………………………………

    - Remote closing of engine room skylights …………………………………………………………………………

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

    - Closing of ventilation and fire flaps ………………………………………………………………………………….

D.5.4	Closing of tunnel door …………………………………………………………………………………………………….

D.5.5	Closing of oil fuel shut downs ………………………………………………………………………………………….

# FIRE APPLIANCE - OTHER FIRE APPLIANCES (outside machinery spaces)

  - FIREMAN’S OUTFIT

E.1.1	Fireman’s outfit complete …………………………………………………………………………………………………..

  - Self-containing breathing apparatus

E.2.1	Operation tested …………………………………………………………………………………………………………………

E.2.2	Air cylinders are fully charged ……………………………………………………………………………………………..

  - Smoke helmet or smoke mask

    - Functioning of smoke mask with air pump and hose tested ………………………………………………….

  - Portable and Non-Portable Fire-Extinguishers outside Machinery Spaces

    - Date of annual checking

    - There spare charges for each fire extinguisher ………………………………………………………………………

c)	Date of last pressure testing

# MACHINERY AND ELECTRICAL

  - GENERAL

    - lf arrangements are found which are materially different from those stated in the Record of Safety Details, an entry must be made at the end of this Report giving full particulars.

    - If the vessel is classed, indicate Class society and date of expiry of valid Machinery

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

Class Certificate

    - Tests required under paragraphs 6.5, 6.6 & 6.7, and may be omitted if the vessel is classed by recognized Society or certified by authorized organization to issue technical certificates on behalf of the Administration.

  - ELECTRICAL POWER

    - All main generators tested and operational ………………………………………………………………………….

    - Emergency source of power tested and found operational under following simultaneous

Ioads: emergency lighting (alleyways, stairways, propulsion machinery space, main generator space, wheelhouse, chartroom, embarkation stations, steering gear room, as applicable), navigation Iights, alarm systems ………………………………………………………………………

    - Emergency lighting adequate and in order …………………………………………………………………………

    - Visible condition of electric machinery and wiring found adequate to minimize the risk of

the risk of electric shock or short circuit ………………………………………………………………………………

  - ANCHOR AND MOORING EQUIPMENT

    - Windlass and stoppers (if provided) tested and found operational ……………………………………….

    - Capstans and mooring wincher (if provided) tested and found operational …………………………..

  - BILGE PUMPING EQUIPMENT

    - Bilge pumps tested and found operational ……………………………………………………………………………

    - Pipes and valves found in adequate condition ………………………………………………………………………

    - Bilge wells and strainers found in order ……………………………………………………………………………….

    - Automatic remote bilge high level alarm (if provided) tested and found in order ………………..

  - STEERING GEAR

F.5.1	Rudder angle indicator operational ……………………………………………………………………………………….

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |

Steering gear tested and found in order ……………………………………………………………………………….

    - Emergency steering arrangement tested and found in order ………………………………………………..

    - lnstructions for emergency steering arrangement available and adequate ……………………………

  - PROPULSION MACHINERY

    - Propulsion engines started three consecutive times ……………………………………………………………..

    - Propulsion engines run at idling speed for 30 minutes and found in order ……………………………

    - Means of going astern tested and found in order …………………………………………………………………

    - Speed controls and gauges found in order, remote RPM and direction or pitch indicators in

the wheelhouse synchronized with engine …………………………………………………………………………..

    - Propulsion auxiliaries (pumps, coolers, etc.) found in order …………………………………………………

    - Piping systems found in order, without leaks, with valves operational …………………………………

    - Inner stern tube seal examined and found in order ……………………………………………………………..

F.7.	BOILERS AND PRESSURE VESSELS

Applicable to boilers with a power greater than 175 kw (T/h) or pressure vessels of a capacity above 500 liters (about 130 gallons)

F.7.1	Boilers operated and found in order ……………………………………………………………………………………..

    - Safety valves of boilers and major pressure vessels tested and found in order ……………………..

    - Steam piping found in order, without leaks, valves operational, auxiliary tanks in order,

drains and strainers in order

………………………………………………………………………………………………………………..

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# HULL AND DECK EQUIPMENT

  - GENERAL

    - If the vessel is classed, indicate Class society and date of expiry of valid Hull

Class Certificate

(DD-MM-YYYY)

    - Examinations under section 7.4 may be omitted if vessel with a valid Load Line Certificate

issued on behalf of the flag state.

(DD-MM-YYYY)

    - Indicate date of last drydocking

(DD-MM-YYYY)

  - ANNUAL HULL SURVEY

This survey is to be carried out both for the issue and for the endorsement of the Certificate

    - Overside over the waterline parts examined all around the vessel and found

to be in an overall good condition

    - The following items were especially examined and found in the following conditions:

      - Degree of visible corrosion

      - Plates around chafing areas

      - Fenders, if fitted

      - No apparent dents at bulkhead locations or distortions in way of major

internal structural members

    - Weather decks examined from above and found in good condition

    - Watertight bulkheads examined from both sides and found in good condition (if accessible)

    - Visual condition of ladderways and railings

  - DRYDOCK SURVEY (See 7.1 b )

This survey is to be carried out for the issue of the Certificate, or as required by the Regulation

    - Underwater parts of hull examined and found in the following condition:

      - State degree of visible corrosion

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

      - Seachests (integrity, condition of grills. cleanliness)

      - Condition of shell around overboard discharges

    - Rudder examined and found in good condition, rudder pintles clearances corrected if found in excess of 10% of the pintle diameter or 10mm, whichever is the Iesser

    - Propeller examined and found in apparent good condition

    - Any cemented areas to be tested with hammer for proper adherence

    - The internal side of the boundaries of all holds and ballast, fuel-oil and cargo tanks examined and found in good condition. All oil tanks must be

gasfree and clean prior to the examination

    - Insulation panels and/or wooden deck coverings, if provided, to be removed

    - Double bottom

    - Upper side of decks

    - Insulated reefer cargo spaces

    - Main structural members were gauged. (These measurements shall only be carried out wherever heavy corrosion stage is apparent.  Where the wastage is found to exceed 25% of the original thickness, the structure should be renewed). The following locations were tested

      - Wastage	%

      - Wastage	%

  - LOAD LINES (See 7.1.b)

    - All doors, sidelights and windows in the first two superstructure levels above

the uppermost continuous deck examined and found in good condition

    - Main hull sidelights and shell opening examined and found in good condition

    - Ventilation inlets and outlets examined and found in good condition,

closing appliances operational

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |

G.4.4	Tank vents examined and found in good condition

G.4.5	Hatch covers examined and found in good condition

  - CARGO GEAR

    - Masts, booms, cranes and supporting structures examined and found in good condition

    - Blocks and chocks examined and found in good condition

    - Wires standing and running rigging examined and found in good condition

    - Winches operated and found in good condition

  - ANCHORS, CABLES, ETC.

    - Visible condition of anchors and chains or wires examined and found adequate

    - Visible condition of mooring lines found adequate

# H    RADIO INSTALLATIONS

COMPLIANCE WITH REQUIREMENTS (to be completed by Inspector and Surveyor) Specify whether the installation is “existing” or “new”

  - LISTENING WATCH

    - Name of radiotelephone operator

    - The radiotelephone operator’s certificate, class and date of issue:

Issued by

  - TWO-WAY VHF RADIOTELEPHONE APPARATUS FOR SURVIVAL CRAFT (III/6.2.1 & IV/14).

    - Correct operation on channel 16 and one other channel

    - Battery  charging  arrangements  (where  rechargeable  batteries  are used)

Expiry date of primary batteries:

  - RADIOTELEPHONE STATION

    - Is the station Iocated at the upper part of the ship?

    - Is  an  efficient  means  of  communication  provided  with  the bridge?

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

    - Is there a reliable clock?

    - Is there an emergency Iight?

    - Is there a card of instructions displayed in fuII view of the operating position and giving a clear summary of the radiotelephone distress procedure?

    - Is the station provided with the documents required by the Radio Regulations?

  - RADIOTELEPHONE INSTALLATION

    - Does the power of the transmitter satisfy the Regulations?

    - What is its normal range in miles?

    - Is  there  a  device  generating  the  radiotelephone  alarm  signal  by  automatic means?

    - Are arrangements made to check periodically the proper functioning of this device?

    - Does    the    radiotelephone    watch   distress    frequency    receiver   comply   with Regulations?

    - Does the receiver comply with the Regulations?

    - Does the main source of energy comply with the Regulations?

    - Is there a reserve energy source of power?

    - Is there a spare serial?

  - VHF RADIOTELEPHONE STATION

    - Does the VHF installation, if provided, comply with regulations?

  - RADIO / LOG

    - ls the radio telephone log kept in accordance with the Regulations?

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

  - CHARACTERISTICS OF THE INSTALLATION

# Transmitter

Makers name		Bands of frequencies:

Type:	No.

Aerial power:	Number of pre-tuned frequencies: (minimum required: A3; 15W Fe or A3 II; 60 W Pp)	Values of these frequencies:

2182

# Receiver

Makers name		Bands of frequencies:

Type:	No.

Continuous tuning control:	Number of pre-tuned frequencies:

Tuning control with preset frequencies:	Values of these frequencies: 2182

  - RADIOTELEPHONE WATCH RECEIVER

Makers name			Operated by:

Main source?

Type:		No.		Reserve source?

  - ENERGY SOURCES

Location and nature of main energy source:	Location and nature of reserve energy source:

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

  - TWO-WAY VHF RADIOTELEPHONE APPARATUS FOR SURVIVAL CRAFT (III/6.2.1 & IV/14).

CHARACTERISTICS OF THE INSTALLATION

Minimum two (2) sets

|  | Maker and type | Channels | Power Supply | Location |
| --- | --- | --- | --- | --- |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| 3. |  |  |  |  |

    - Comply with performance standards of the Organization?

    - If rechargeable batteries are used and a charging arrangement available?

  - VHF RADIOTELEPHONE STATIONS

(Chapter IV, Reg. 17) (Chapter V, Reg. 18) (if provided)

|  | MAKER AND TYPE | POWER SUPPLY | Channels | LOCATION OF CONTROL UNITS | WORKING SATISFACTORILY? |
| --- | --- | --- | --- | --- | --- |
| Set 1 |  |  |  |  |  |
| Set 2 |  |  |  |  |  |
| Set 3 |  |  |  |  |  |

Has the Bridge Control unit priority for the purpose of

Navigational safety?		Working normal?

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |

  - ADDITIONAL RADIO EQUIPMENT

# (If onboard, to be recorded)

Provided

    - State number off:		State battery expiry date:

    - Satellite navigator

    - Satellitecommunication

    - Telex equipment

    - Selective Call equipment

    - Weather facsimile

    - Walkie Talkie sets

Additional if applicable

  - Alterations, Replacements or Renewals in the Record of Approved Fishing Vessel Safety Details:

| ITEM NO. ACCORDING TO THE RECORD: | THE RECORD ONBOARD IS CORRECTED AS FOLLOWS: |
| --- | --- |

  - Particulars of outstanding safety items requiring attention or survey.

| ITEM NO. ACCORDING TO THE RECORD: | REMARKS: |
| --- | --- |

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |

# FINAL RECOMMENDATION

It is recommended that:

  - A Full Term Fishing Vessel Safety Certificate may be issued valid for a period of five (5) years

  - No  Full  Term  Fishing  Vessel  Safety  Certificate to  be  issued  until  deficiencies are  closed (marked in Item 8.14)………………………………………………….........................................................

  - The Existing Fishing Vessel Safety Certificate No.:	has been endorsed at:

,		of		of	 		 (Place of issue of endorsement)		(Day)		(Month)			(Year)

  - I	have issued  the  Interim/ Conditional (*)   Fishing Vessel Safety Certificate:

No.		at:		,		of		of   		 (Place of issue of Interim FVS)		(Day)		(Month)		(Year)

| Valid until: | of of |  |
| --- | --- | --- |
|  | (Day) (Month) | (Year) |

| Issued at: |  | Date of Issue: |  |
| --- | --- | --- | --- |
|  | (Place of issue of certificate) |  | (DD-MM-YYYY) |

THIS IS TO CERTIFY THAT: This survey report is correct in all respects.

# GR CLASS Surveyor Name  - ID No.