# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

REPORT OF SURVEY OF PREVENTION OF POLLUTION BY OIL FOR MACHINERY SPACE OPERATIONS

(OIL TANKERS)

# Initial Survey	Annual Survey

Renewal Survey	Intermediate Survey

Delete if not applicable

To Meet Regulations of ANNEX I of the International Convention for The Prevention of Pollution from Ships, 1973 and its 1978 Protocol

| NAME OF SHIP: | PORT OF REGISTRY: |  | GROSS TONNAGE: | YEAR KEEL LAID: |
| --- | --- | --- | --- | --- |
| DEAD WEIGHT (METRIC TONS): | PATENT NUMBER AND DATE OF EXPIRING: |  |  |  |
| DISTINCTIVE LETTERS: | PLACE OF SURVEY: |  |  |  |
| AT1: | DATE OF SURVEY: |  |  |  |
| Details of persons requesting survey and to whom certificates are to be sent: |  |  |  |  |
| Owner/Master |  | Agents/Operators: |  |  |

Yes

No

N/A

STATUS OF THE SHIP (Chap I, Reg. 1 (6))

- A NEW OIL TANKER IS ONE:

  - For which the building contract was placed after December 31st, 1975

  - In the absence of the building contract, the keel was laid or which was at a Similar

stage of construction after June 30th, 1976

  - If the date that the keel was laid is not available, the delivery of which was after December 31st, 1979

  - Which has undergone a mayor conversion:

    - For which the contract was placed after December 31st, 1975

    - In the absence of a contract, the construction work of which was begun after

June 30th, 1976

    - If the date of the construction work was begun is not available, which was completed,

1 The surveyor should indicate where the inspection was carried out, if in Dry-dock, on Slipway or afloat

After December 31st, 1979

# EXISTING SHIP (Chap. I, Reg. 1(7))

- DOCUMENTATION:

  - Are certificates for type approval of Oil Pollution Equipment on board, such as:

    - Oil Filtering Equipment

Yes	No	N/A

# Process Units

    - Oil Content Meters

    - Oil/water Interface Detectors

- REQUIREMENTS FOR MANDATORY ANNUAL SURVEY (MAS):

  - Are the following Statutory Certificates valid:

| Statutory Certificate | No. | Expiration Date |
| --- | --- | --- |
| Cargo Ship Safety Construction |  |  |
| Cargo Ship Safety Radio |  |  |
| International Oil Pollution Prevention Certificate |  |  |
| International Load Line Certificate |  |  |
| Cargo Ship Safety Equipment |  |  |
| Cargo Ship Safety Certificate |  |  |
| Passenger Ship Safety Certificate |  |  |
| Fishing Vessel Safety Certificate |  |  |

Yes

No

N/A

Is the ship catalogued/considered as:

      - Crude Oil/Product Carrier (Which is allowed to carry either crude oil or product oil  or both simultaneously)

      - Crude Oil Tanker (Which is allowed to carry crude oil but is prohibited from carrying product oil)

3.1.1.3. Product Carrier (Which is allowed to carry product oil but is prohibited from carrying crude oil)

  - IOPP Certificate has been duly surveyed within the required window

  - Is the Supplement (Form B) together with the IOPP Certificate on board and was it reviewed to ensure it has been properly completed to reflect the vessel’s equipment

and arrangements

  - Are the Oil Record Books (Cargo/Ballast Operations) of the required format provided on board and properly entries are being made

  - Is the Filtering System Manual provided on board

  - An approved Shipboard Oil Pollution Prevention Emergency Plan (SOPEP) is provided On board

  - Is the SOPEP provided with the latest published Interest list of coastal contacts

Yes

No

N/A

Are the following manuals and instructions on board:

    - Approved Operation and Equipment Manual (COW)

    - Approved Dedicated Clean Ballast Tank Manual (CBT)

    - Approved Loading and Stability Information as per Chap. I, Reg. 25 (5)) (Required on new oil tankers or new non self propelled oil tanker)

3.8.4	Are there instructions for the operation of the part flow system if fitted, included in  the cargo and ballast handling manuals and COW/CBT Manuals, where applicable _

3.8.5.	Approved Oil Discharge Monitoring and Control System (ODMC) Manual

  - Were the following systems and arrangements examined and tested including pumps, piping and fittings and found to be properly installed and operating in satisfactory conditions

    - Oil Filtering Equipment (15 PPM) (New and existing Oil Tankers of 150

GRT and above)

    - The above mentioned Filtering equipment is fitted with a 15 PPM bilge alarm as per MEPC. 60 (33) requires (if the installation is fitted on ships for which the

## keel is laid or similar stage of construction on or after April 30th, 1994 and to all new installations fitted on or after this date to ship for which the keel was laid before April 30th,1994)

    - Is the above mentioned Filtering Equipment fitted with 15 PPM alarm and Automatic Stopping Device (if vessel is 10,000 GRT and above)

3.9.4.	Is the Oil Content Meter fitted on board and functioning correctly (if vessel is

## 10,000 GRT and above)

- SEGREGATED/DEDICATED CLEAN BALLAST TANKS (CBT) AND CRUDE OIL WASHING (COW):

For purposes of applying regulations 13, 13B, 13E, and 18 (4) the following means new oil tanker as per Reg. 1 (26):

- A NEW OIL TANKER IS ONE:

- For which the building contract was placed after June 01st, 1979

- In the absence of the building contract, the keel was laid or which was at a Similar

stage of construction after January 01st, 1980

- If the date that the keel was laid is not available, the delivery of which was after June 01st, 1982

- Which has undergone a mayor conversion:

  - For which the contract was placed after June 01st, 1979

  - In the absence of a contract, the construction work of which was begun after

January 01st, 1980

No

N/A

Yes

If the date of the construction work was begun is not available, which was completed, After June 01st, 1982

# EXISTING SHIP (Chap. I, Reg. 1(27))

## New Crude Oil Tankers of 20,000 DW and above and New Product carriers of  30,000 DW and above

4.1.1 Is the capacity of the SBT enough that the ship may operate safely on ballast voyages without recourse to the use of cargo tanks for water ballast except as provided in the regulations 13 (3) or 13 (4) _

    - Is the vessel provided with CBT with sufficient capacity that comply with the regulation 13 (2), (3), (4) and (5) as appropriate

    - Is the vessel carrying ballast water in cargo tanks

    - Is the vessel fitted with a Cargo Tank Cleaning System using COW

    - Is the COW installation and associated equipment and arrangements in accordance with the requirements of design, operation and control of Crude Oil Washing system adopted by the A. 497 (XII)

      - Is every cargo tank and slop tank fitted with an Inert Gas System

      - Is the ship provided with an operation and equipment manual detailing the system and equipment, specifying operational procedures

## Exiting Crude Oil Tankers of 40,000 DW and above

    - Is the capacity of the SBT enough that the ship may operate safely on ballast voyages without recourse to the use of cargo tanks for water ballast except as provided in the regulations 13 (3) or 13 (4) _

    - Is the vessel fitted with a Cargo Tank Cleaning System using COW

## Exiting Product Carriers of 40,000 DW and above

    - Is the vessel provided with segregated ballast tanks

    - Are the arrangements and operational procedures for dedicated clean ballast tanks complying with all provisions of the specifications for the Oil Tankers with dedicated clean ballast tank adopted by the resolution A. 495 (XII)

    - Is the Oil content meter working properly

    - Is the ship provided with dedicated clean ballast tank operation manual, as per the

resolution A. 495 (XII) detailing the system and specifying procedures

# Segregation of Oil and Water Ballast and carriage of Oil in forepeak tanks

  - Is the vessel carrying ballast water in fuel tanks (applicable to new oil tankers of

## 150 GT and above)

Yes

No

N/A

Is the vessel carrying oil fuel in Forepeak tank (Oil tankers constructed on or after July 01st, 1982 are not allowed to carry fuel oil in fore peak tank)

- Are audible and visual alarms, automatic and manual means provided to stop the discharge to affluent in satisfactory conditions

  - Are the Indicators/meters recorders operating satisfactorily and is sufficient supply of spare consumables available for the recorders

  - Were the permanent Records from the Oil Discharge Monitoring and Control System (ODMC) sighted on board

  - Was the Oil Filtering Equipment (15 PPM) with alarm and manual stopping device,

as well as, corresponding piping/fittings examined, tested and considered satisfactory _

    - Date of last filters’ renewal (for the Oil Filtering Equipment):

    - Was the Automatic Stopping Device required for discharge in Special Areas (If vessel is navigating on Special Areas) examined, tested and considered

satisfactory

# ARRANGEMENTS OF SLUDGE TANKS (CHAP. I, REG. 17, AND MEPC/CIRC. 97):

  - Is the vessel provided with the arrangement of the Sludge Tanks according to the

Chap. I, Reg. 17, and MEPC/Circ. 97

  - Arrangement of Sludge Tank (s) and Discharge piping arrangement, including confirmation that piping to and from the sludge tank (s) has no direct connection overboard other than the Standard Discharge Connection

  - Is the Standard Discharge Connection of the required dimensions, including 6 bolts and nuts (20 mm in diameter) and its gasket of oil proof material, on board and in

satisfactory conditions

  - Is the ship fitted with other means to control the sludge as any recognized Homogenizers, incinerators, etc when the size of the sludge tank is approved

on the basis of such installation

# Other requirements for Cargo Spaces for Oil Tankers:

  - Was it verified, as far as practicable, that there was no leakage form those ballast lines passing through cargo tanks and those cargo lines passing through ballast tanks

  - Were the CBT pump, pipe and valve arrangements found in accordance with the approved CBT Operation Manual

  - Were the CBT examined by sighting and found to contain no oil contamination

N/A

Were the SBT Pump, pipe and valve arrangements found in accordance with requirements for SBT system and having no inter-connection with the cargo oil system _

  - If the vessel is operating with special ballast arrangements, are the approved arrangements and operational procedures complied with

Yes	No

  - Were the Oil Discharge Monitoring and Control System, including piping, and its associated equipment examined and tested in operation in accordance with the  guidelines developed by IMO and found in satisfactory working condition

  - Were the following pumping, piping and discharge arrangements examined and found satisfactory

Dirty Ballast Discharge Piping,	Oil Contaminated Water Discharge Piping

Means of Draining Cargo Pump and Lines,	The Part Flow System, (where fitted)

The Stripping Device and the Connections to the Slop Tank, Cargo Tanks or ashore

Communication System between the Contaminated Water Observation Position and Discharge Control Position (including a test of the system), or the Means to Stop the Discharge from a Position on the Upper Deck if a Communication System is not Provided

  - Was the COW system as far as could been seen, arranged as outlined, in the

operations and equipment manual, and in particular, were the following verified

Inert Gas System Surveys up-to date,	Piping, pumps, valves and deck machines

examined for signs of leakage and found satisfactory

COW branch lines, anchoring devices and flexible connections examined and found intact and secure.

Drive Unit (if they are not integrals with the tank cleaning machines) are there sufficient operational drive units on board as specified in the operational and equipment manual

In case of steam heaters for washing are fitted, they can be properly insolated

during crude oil washing operations, either by double shut off valves or clearly identifiable blanks.

Means of communication between the deck watchkeeper and the cargo control position was functioning in good order.

Over pressure relief device (or other approved arrangement) is fitted to the pumps supplying the Crude Oil Washing System and it is working in good conditions.

Flexible hose for use in COW on combination carriers are on an approved type, properly stored and in good condition/properly marked.

  - Was the effectiveness of the COW system verified, as far as practicable, and

in particular were the following verified

Tanks containing departure and/or arrival ballast were checked, as far as practicable, to confirm the effectiveness of the cleaning and stripping

Crude Oil washing machines, were checked, as far as practicable, to ensure they are operable

Yes

No

N/A

Requirements for Oil Discharge Monitoring and Control System (ODMC):

In order to assist in the implementation of resolution A. 445 (XI), regarding ODMC system MEPC has divided into five (5) categories the equipment that has to be fitted on board

as follows:

# Category I

    - Is this vessel fitted with a Control Unit (All new oil tanker of 4,000 DW and above and the ODMC System was installed on or after June 01st, 1982)

    - Is the control unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed,	Date and time (G.M.T.)

Discharge valve position (open or closed)

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

# Category II

    - Is this vessel fitted with a Computing Unit (All new oil tanker of 4,000 DW and above and the ODMC System was installed before June 01st, 1982)

    - Is the vessel fitted with a starting interlock and discharge valve control capability as required in resolution A. 496 (XII)

    - Is the computing unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed,	Date and time (G.M.T.)

Discharge valve activation

9.2.4	Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

## (The flow rate and ship’s speed may be manually inserted into the unit)

Yes

N/A

No

Category III

    - Is this vessel fitted with a Computing Unit (All new oil tanker of 150 GRT and above but less than 4,000 DW)

    - Is the control unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed,	Date and time (G.M.T.)

Discharge valve activation

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

# Category IV (a):

    - Is this vessel fitted with a Computing Unit (Existing oil tanker of 20,000 DW and above and the ODMC System was installed between October 1984 and October 1986)

    - Is the vessel fitted with starting interlock as required in resolution A. 496 (XII)

    - Is the computing unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed,	Date and time (G.M.T.)

Discharge valve activation

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc.)

Override action (i.e. manual override, flushing, calibration, etc.)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

## (Tankers up to 100,000 GRT DW where the discharge valve has local manual control or where the control is provided by reach roods the administration may grant waivers from having to fit a starting)

Yes

No

N/A

  - Category IV (b):

    - Is this vessel fitted with a Computing Unit (Existing oil tanker of 20,000 DW and above and the ODMC System was installed prior October 1984)

    - Is the computing unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed,	Date and time (G.M.T.)

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

# Category V (a):

    - Is this vessel fitted with a Calculating Unit (Existing oil tanker of 150 GRT and above but less than 20,000 DW and above and the ODMC System was

## installed between October 1984 and October 1986)

    - Is the calculating unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

    - The unit shall display:

Instantaneous rate of discharge of oil,

Total quantity of oil discharged, unless it permits to be calculated manually

N/A

Yes

No

Category V (b):

    - Is this vessel fitted with a Calculating Unit (Existing oil tanker of 150 GRT and above but less than 20,000 DW and above and the ODMC System was

## installed prior October 1984)

    - Is the calculating unit capable of receiving an automatic signal of:

Oil Content,	Flow rate of discharge,	Ship’s speed

    - Is this unit capable of making records of:

Instantaneous rate of discharge of oil,	Total quantity of oil discharged

Discharge valve position (open or closed),	Alarm condition,	Failure (i.e. no flow, fault, etc)

Override action (i.e. manual override, flushing, calibration, etc)	Date and time (G.M.T.)

Manual input (i.e. speed, flow),	Oil Content if the flow rate has been manually inserted

    - The unit shall display:

Instantaneous rate of discharge of oil,

Total quantity of oil discharged, unless it permits to be calculated manually

# MODIFICATIONS OR ALTERATIONS

  - Was it confirmed that no unapproved modifications have been made to the ship or its equipment

# ALTERATIONS, REPLACEMENTS OR RENEWALS IN THE SUPPLEMENT (FORM B):

| Item No. According to the Form B | The Form B, on board, is corrected as follows |
| --- | --- |

Yes

No

N/A

The alterations or modifications mentioned in the above table, are been in accordance to the approved manuals or operational procedures

  - Overboard Discharge Valve was externally examined and found satisfactory

# ADDITIONAL REQUIREMENTS FOR INTERMENDIATE SURVEY:

  - Was the Filtering Equipment, including associated pumps, piping and fittings

examined for wear and corrosion and found satisfactory

  - Was the Oil Content Meter (15 PPM alarm and bilge monitor) checked for obvious defects, deterioration or damage

  - For the Oil Content Meter, was the record of calibration checked against the manufacturer’s operation and instruction manual

  - Manual and Automatic stop of overboard discharge was functionally tested and found in good order

  - Filter(s) were renewed (indicate last date):

# PARTICULARS OF OUTSTANDING SAFETY ITEMS REQUIRING ATTENTION OF SURVEY:

| Item No. According to the Form B | Remarks: |
| --- | --- |

13. REGULATIONS 13 G & 13 H (Applicable to vessels Oil Tankers above 5,000 DWT not complying with Reg. 13 F)

Category of

oil tanker	Date or year

----------------------------------------------------------

Category 1	5 April 2005 for ships delivered on 5 April

1982 or earlier

2005 for ships delivered after 5 April 1982

----------------------------------------------------------

Category 2	5 April 2005 for ships delivered on 5 April and	1977 or earlier

Category 3	2005 for ships delivered after 5 April 1977

but before 1 January 1978

2006 for ships delivered in 1978 and 1979

2007 for ships delivered in 1980 and 1981

2008 for ships delivered in 1982

2009 for ships delivered in 1983

2010 for ships delivered in 1984 or later

----------------------------------------------------------

13.1 Vessel to comply with the requirements of Reg. 13F not later than specified

date above (If not IOPP Certificate not to be issued unless vessel is subject to the Condition Assessment Scheme (CAS) requirements)

13.2. Oil tankers of 600 tons deadweight and above carrying heavy grade oil as cargo regardless of the date of delivery to comply with the requirements of Reg. 13F by the delivery

date of the ship by 2008. (If not IOPP Certificate not to be issued unless vessel is subject to a similar inspection as the Condition Assessment

Scheme (CAS) requirements)

Yes

No

ALL PARTS SURVEYED WERE FOUND TO BE IN AN EFFICIENT AND GOOD CONDITION AND THE UNDERSIGNED RECOMMENDS:

## The issue of a full term certificate

- The issue of full term certificate, after the recommendation has been carried out

- No full term certificate to be issued before the survey has been completed (see list of outstanding items, and deficiencies above)

Place and Date of Survey	Surveyor to GR